| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reliable Multicast Transmission for M2M Applications |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Rong-Terng Juang ITRI | rtjuang@itri.org.tw +886-3-591-9024 |
| Re: | Call for contributions on the 802.16p Amendment Working Document |  |
| Abstract | The contribution proposes text for Reliable Multicast Transmission in current IEEE 802.16p AWD. |  |
| Purpose | To be discussed and adopted in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reliable Multicast Transmission for M2M Applications

Rong-Terng Juang

- Introduction

The IEEE 802.16p AWD supports reliable multicast transmission for M2M applications [1]. However, the detailed implementation has not been specified yet.

This contribution proposes to apply exclusive-OR coding to multicast transmission. To reduce the request of retransmission from a large number of M2M devices, the base station can proactively encode multiple data blocks into one and the receivers correspondingly recover the faulty data blocks based on the mixed data block. A simple embodiment of the proposed scheme is illustrated in Figure 1, where X1 and X2 are two distinct data blocks and x3=x1⊕x2 is another data block resulting from bit-wise XOR operation across X1 and X2. The XOR operation is the binary addition in modulo-2 arithmetic without carries. In such a way, two bits, say, b1 and b2, can be sent in one transmission by the XOR operation, i.e., b1⊕b2. Upon receiving b1 and b1⊕b2, a receiver can decode b2 by b1⊕(b1⊕b2). Similarly, it can decode b1 if having the knowledge of b2 and b1⊕b2. With a CRC code attached to each data block, the receiver can recover the faulty data block if either of X1 and X2 is erroneous and X3 is correct.

Figure 1. Illustration of the proposed reliable multicast transmission scheme.

- Reference

[1] IEEE 802.16p-10/0018r1, IEEE P802.16p AWD

- Proposed Text Change

Adopt the following remedy in IEEE 802.16p-10/0018r1.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2.28.4.2 Reliable multicast transmission for M2M applications

An M2M BS shall provide the reliable transmission of the multicast traffic for M2M applications.

[Insert the following sentences]

The M2M BS may provide reliable transmission by adopting exclusive-OR coding.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------