| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of Ranging Accesses from M2M Devices During Network Reentry |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for distribution of ranging access from M2M devices during network reentry from Idle Mode. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of Ranging Accesses from M2M Devices During Network Reentry

Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon

ETRI

# Introduction

In the last meeting, we agreed that a paging message may contain ranging configuration which can be used for initial ranging during network reentry. But, specific information on ranging configuration is not defined yet. This contribution proposes some texts which define ranging configuration included in a paging message.

We can classify a ranging access into two kinds of access. One is an M2M group access and the other is an individual device access which belongs to the same M2M group. This contribution is focused on the individual device access because we assume that a network can distribute M2M group accesses by assigning different paging offsets to each M2M groups based on the characteristics of M2M applications.

For the distribution of individual devices accesses, we propose to reuse the current backoff algorithm and to assign a different backoff window size dedicated for M2M devices instead of allocation additional ranging channels. If the number of M2M devices to be paged at the same time is large, then the ABS may assign bigger backoff window than backoff window defined in SFH SP3. If the number of M2M devices to be paged is not so large, then the ABS will not include a new backoff window in a paging message. Then, M2M device will perform initial ranging according to the ranging parameters defined in SFH SP3.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify Table 700 on page4 as follows;]

Table 700 – AAI-PAG-ADV message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message. | Shall be included if the ABS sends a notification for M2M groups using MGIDs.DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action Code | 1 | 0: Performing network reentry 1: Receiving multicast traffic |  |
| If (Action Code == 1) { |  |  |  |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame nuimber in which the ABS starts sending DL multicast data | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| else { |  |  |  |
| New Initial Backoff Window Size | 4 | A new initial backoff window size for ranging contention during network reentry. Expressed as a power of 2. Values of n range 0~15 | Optional |
| New Maximum Backoff Window Size | 4 | A new maximum backoff window size for ranging contention during network reentry. Expressed as a power of 2. Values of n range 0~15 | Optional |
| } |  |  |  |
| … | … | … | … |
| } |  |  |  |

[Remedy 2: Modify texts on page 7, line 1 as follows;]

16.2.18.7.2 Network re-entry from idle mode for M2M device

For network reentry from Idle Mode, ranging parameters may be different for M2M devices or M2M groups.

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2.

For the network reentry indicated by a paging message that contains a new initial backoff window size and/or a new maximum backoff window sizeranging configuration, the M2M device shall select a ranging opportunity using the assigned new backoff window parameters instead of backoff window parameters defined in SFH SP3. If a paging message does not include any new backoff window parameters, the M2M device shall perform initial ranging according to the ranging configuration defined in SFH SP3. Ranging configuration may include differentiated waiting offset time and backoff window size.

----------------- End of the text proposal ---------------------------------------------------------------------------------------