| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Group paging for M2M group |  |
| Date Submitted | 2011-05-17 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon ETRI | Voice: +82-42-860-5928 E-mail: {sjjung}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for group paging for M2M group. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Group paging for M2M group

Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon

ETRI

# Introduction

According to 16.2.18.7.1[1], group paging may be used for M2M devices. For this, M2M Group identifier (MGID) may be included in a paging message instead of an individual identifier (DID) to identify the group of M2M devices. In the 16m paging operation, an AMS monitors paging group and paging message within a paging listening interval. And the AMS shall monitor a pre-determined frame within a paging listening interval for paging message. The pre-determined frame, Npaging frame, for an AMS is implicitly determined using AMS’s DID and m in PGID-Info message as follows:

Npaging frame = AMS’s deregisration identifier mod m  (1)

For a 16p M2M device which currently belongs to M2M group(s), the M2M device shall simultaneously monitor paging message(s) for individual paging and group paging. There may be M2M devices which belong to same M2M group and shall monitor different locations for paging message. Figure 1 shows an available method that 16p BS transmits the paging message for group paging. In order to ensure that group paging message is received correctly, the 16p BS shall transmit the paging messages that contains the MGID repeatedly within a paging listening interval.

Figure 1- an available method for group paging

But, the method represented in Figure 1 is inconsistent with the goal of group paging mechanism, i.e. signaling optimization.

As alternate method, the 16p BS can transmit the paging message that contains the MGID only once at any location. However, this method also has a drawback. If a M2M device that belong to a M2M group determines the monitoring frame for paging using (1) and monitors the paging message in the same way as 16m, then, in some cases, a M2M device cannot receive the paging message for group paging. That is, if the location of paging message for M2M group and the pre-determined location of paging message for individual M2M device are different, the idle mode M2M device which is not individually paged cannot receive the paging message for M2M group and returns to paging unavailable interval. Figure 2 shows an example mentioned above.

Figure 2 –an arbitrary location (frame #0) for group paging

In order to ensure that group paging message is received correctly, the 16p BS shall transmit the information about the fixed location for group paging and all M2M devices which are belong to some M2M groups shall monitor the paging message at the fixed location. The location for group paging is determined by m and group paging indicator. Figure 3 and Figure 4 show the example of methods mentioned above. In Figure 3, the location for group paging is the first frame when the group paging indicator and the value of m are set to 0 and 4. When the group paging indicator and the value of m are set to 1 and 4, the location for group paging is the 4th frame in Figure 4.

Figure 3- Fixed location for group paging with location information (Group paging indicator = 0)

Figure 4- Fixed location for group paging with location information (Group paging indicator = 1)

# References

[1] IEEE 802.16p-10/0018r1, “Enhancements to Support Machine-to-Machine Application,” March. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts

- Texts deleted by this contribution: red strikeout texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Add the following text on line 37, page 4]

16.2.3.24 PGID-Info (paging group information) Message

Modify Table 701 as indicated

Table 701 – PGID-Info Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For(i=0;i<Num_PGIDs;i++){ |  |  |  |
| PGID | 16 |  |  |
| m | 2 |  |  |
| Group paging indicator | 1 | Indicate the location of group paging message. 0:the first among frames determined by the value of ‘m’ 1:the last among frames determined by the value of ‘m’ | Present when group paging is supported. |
| } |  |  |  |
| ……. |  |  |  |

16.2.18.7.1 Paging operation

[Add the following text on line 65, page 6]

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices. In order to ensure that group paging message is received correctly, the 16p BS shall transmit the location information of the paging message including M2M Group Identifiers (MGIDs). A M2M device which is belongs to one or more M2M groups shall monitor the pre-determined frame for group paging based on the location information described by Group paging indicator in PGID-Info message.

----------------- End of the text proposal ----------------------------------------------------------------------------------------