| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Allocation of dedicated ranging channel for M2M devices |  |
| Date Submitted | 2011-05-17 |  |
| Source(s) | Seokki Kim, Soojung Jung, Jaesun Cha, Chulsik Yoon ETRI Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) Whai-En Chen National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University | Voice: +82-42-860-0626 E-mail: {kimsk0729}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for allocation of dedicated ranging channel for M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Allocation of dedicated ranging channel for M2M devices

Seokki Kim, Soojung Jung, Jaesun Cha, Chulsik Yoon

ETRI

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien,

Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

# Introduction

According to IEEE 802.16p-10/0018r1[1], BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resource. If BS does not assign dedicated ranging resource, M2M devices perform ranging for network (re-)entry using the ranging resource defined in Table 833 in 16.3.5.5.1. In other words, it was agreed on the necessity of the dedicated ranging resource for network re-entry from Idle Mode. However there is no consensus approach to assign the resource yet. In this contribution, we propose how to allocate the dedicated ranging channel(opportunity) for M2M devices.

In 16m, it is possible that ABS allocate the ranging channel by using broadcast assignment A-MAP for HO and we can use the method for M2M devices in the range of the opportunity index the same as HO ranging channel allocation(i.e. opportunity index 0b01 and 0b10). Because the broadcast assignment A-MAP for HO ranging channel allocation is broadcasted to all mobiles, if we directly apply the existing method, AMS and M2M devices are not able to distinguish whether the ranging channel is for AMS or M2M devices. Therefore it is needed that broadcast assignment A-MAP is modified. We suggest using reserved Function Index(0b11) of broadcast assignment A-MAP for dedicated ranging channel allocation for M2M devices.

Following is how to use the dedicated ranging channel. (this is just example)

Figure 1 – Example of Use

Although reserved Function Index(0b11) may be used for 16p multicasting, but there is no problem to apply our proposal regardless of multicasting schemes for 16p by using 1-bit SubFunction Index which differentiate between multicasting and ranging channel allocation.

If this is agreed, we need to consider about more detailed and efficient procedure for practical use of the dedicated ranging channel for M2M devices.

# References

[1] IEEE 802.16p-10/0018r1, “Amendment Working Document,” March. 2011.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Modify Table 860 as follows:]

Table 860—Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reserved This IE carries multicast assignment information for M2M application or ranging channel allocation information for M2M devices |
| if {Function Index == 0b00} { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 13 | Reserved bits |
| }else{ |  |  |
| Reserved | 15 | Reserved bits |
| } else if {Function Index == 0b01} { |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 29/25 | Reserved bits. If Number of Ranging Opportunities is set to 0b0, Size in bits is 29. Otherwise, Size in bits is 25. |
| } else if {Function Index == 0b10} { |  |  |
| Multicast Group ID | 12 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 4 | Reserved bits |
| }else if {Function Index == 0b11} { |  |  |
| SubFunction Index | 1 | 0b0: Multicast assignment information for M2M application 0b1: Ranging channel allocation information for M2M devices |
| If {SubFunction Index==0b0}{ |  |  |
| MGID | 15 |  |
| Burst Size | 6 |  |
| Resource Index | 11 |  |
| Long TTI Indicator | 1 |  |
| }else{ |  |  |
| Assignment Type | 1 | 0: Assignment for specific M2M group 1: Assignment for entire M2M devices |
| if {Assignment Type ==0}{ |  |  |
| MGID | 15 | M2M Group Identifier |
| } |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH for M2M devices 1: two NS-RCHs for M2M devices |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 27/23/12/8 | Reserved bits. 27: Number of Ranging Opportunities and Assignment Type are set to 0 and 1, respectively. 23: Number of Ranging Opportunities and Assignment Type are set to 1 and 1, respectively. 12: Number of Ranging Opportunities and Assignment Type are set to 0 and 0, respectively. 8: Number of Ranging Opportunities and Assignment Type are set to 1 and 0, respectively. |
| } |  |  |
| } |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------