| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of power saving operation of M2M device in idle mode |  |
| Date Submitted | 2011-05-05 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak LG Electronics | giwon.park@lge.com +82-31-450-1879 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes to idle mode operation for power saving of M2M devices |  |
| Purpose | To be discussed and adopted by 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of power saving operation of M2M device in idle mode

Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Discussion

In IEEE 802.16e idle mode, BS shall broadcast the MOB-PAG-ADV message during the MS Paging Listening Interval in order to advertise the BS supported Paging Groups, even without notification of any MS. However, in IEEE 802.16m idle mode, if the ABS does not have the notification of the AMS, ABS may not broadcast the AAI-PAG-ADV message during the AMS Paging Listening Interval because the ABS broadcasts the PGID Information message including the BS supported Paging Groups before transmitting the AAI-PAG-ADV message. In that case, if the AMS does not receive the AAI-PAG-ADV message during the Paging Listening Interval, AMS can’t decide whether ABS does not transmit the AAI-PAG-ADV message or the AAI-PAG-ADV message is missed. In that case of HTC device, this is not a critical problem because the paging cycle of the HTC device is not the long so HTC device can monitor the paging message in next paging cycle. However, in case of M2M device, this can be a critical problem because the paging cycle of some M2M devices is so long (e.g., hours or day). If the M2M device with long paging cycle does not receive the AAI-PAG-ADV message during its own PLI, M2M device may wait much time to receive the AAI-PAG-ADV until the next paging cycle. Thus, we propose that the ABS transmits the PGID Information message including the Paging Message Indication to indicate whether the AAI-PAG-ADV message is transmitted or not during the Paging Listening Interval. This helps the devices to decide whether ABS transmits the AAI-PAG-ADV message or not. Especially, Paging Indication of PGID Information Message is useful for M2M devices with long paging cycle. For example, if the M2M device with long paging cycle does not receive the AAI-PAG-ADV message although Paging Message Indication is set to 1, M2M device requests retransmitting the AAI-PAG-ADV message without waiting the AAI-PAG-ADV message until the next PLI.

Although device receives the PGID Information message including the Paging Message Indication which is set to 1, if the device does not receive the AAI-PAG-ADV message during the Paging Listening Interval, device can perform the location update for requesting the AAI-PAG-ADV message, e.g., as shown in figure 1.

Figure 1. Error handling of paging message via location update

Moreover, it is beneficial to the AMS in the perspective of the power saving. That is, when the M2M device receives the PGID Information Message including the Paging Indication which is set to 1, M2M device can skip the decoding procedure of the AAI-PAG-ADV message.

Proposal 1. Paging Message Indication can be included in PGID information to indicate whether the AAI-PAG-ADV message is transmitted or not during the Paging Listening Interval.

Proposal 2. Device can perform the location update for error handling of the paging message reception.

- Reference

[1] IEEE C802.16p-11/00018r1, P802.16p Amendment Working Document (AWD)

- Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 16.2.18.7.1 as follows:]

16.2.18.7 Idle Mode for M2M application

16.2.18.7.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

Paging Message Indication of PGID-Info Message may be used for M2M devices. If the M2M device checks that Paging Message Indication is set to 1 in the PGID-Info Message, M2M device monitors the AAI-PAG-ADV message during the Paging Listening Interval. If not M2M device shall not decode the AAI-PAG-ADV message during the Paging Listening Interval and return to the Paging Unavailable Interval. If the M2M device does not receive the AAI-PAG-ADV message during the Paging Listening Interval although Paging Message Indication is set to 1, M2M device may request retransmitting the AAI-PAG-ADV message without waiting the AAI-PAG-ADV message until the next Paging Listening Interval.

[Remedy 2. Add the following changes on page 174, line 54 in 16.2.3.24 PGID-Info Message]

16.2.3.24 PGID-Info (paging group information) Message

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| } // End If (an ABS supports multiple carrier operation) |  |  |  |
| Paging Message Indication | 1 | 0: Paging message which is notifying the M2M devices will not be transmitted in the current superframe. 1: Paging message which is notifying the M2M devices will be transmitted in the current superframe. | This parameter is only useful for the M2M device. |

[Remedy 3. Add the following changes on page 88, line 65 in 16.2.3.1 AAI-RNG-REQ]

16.2.3.1 AAI-RNG-REQ

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = Idle mode location update of M2M device for requesting the AAI-PAG-ADV message 0b1110-0b1111 = reserved |  |

[Remedy 4. Add the following changes on page 98, line17 in 16.2.3.2 AAI-RNG-RSP]

16.2.3.2 AAI-RNG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x6 = Success of location update of M2M device for requesting the AAI-PAG-ADV message 0x07~0xF: Reserved | Shall be included when this message is sent in response to an AAIRNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension or Idle mode location update of M2M device for requesting the AAI-PAG-ADV message. |
| if (Location Update Response == 0x6) { |  |  |  |
| Network reentry required | 1 | 0: Network reentry is not required. 1: Network reentry is required. |  |
| If (Network reentry required == 0) { |  |  |  |
| Paging Group ID update | 32 | Old PG ID, New PG ID The Paging Group ID update specifies a new PGID that replaces an old PGID. | Shall be included when the Paging Group ID is changed. |
| Paging Offset update | 24 | Old Paging Offset, New Paging Offset The Paging Offset update specifies a new Paging Offset that replaces an old Paging Offset. | Shall be included when the Paging Offset is changed. |
| New Paging Cycle | 4 | New Paging Cycle to be assigned to the M2M device 0b0000-0b1111 | Shall be included when the new Paging Cycle is assigned. |
| New Paging Group ID | 16 | New PGID of the M2M device | Shall be included when the new Paging Group ID is assigned. |
| New Paging Offset | 12 | New paging offset of M2M device which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. | Shall be included when the new Paging Offset is assigned. |
| New Paging Controller ID | 48 | The new Paging Controller ID which the M2M device shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------