| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of M2M device with long paging cycle in idle mode |  |
| Date Submitted | 2011-05-05 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak LG Electronics | giwon.park@lge.com +82-31-450-1879 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes the idle mode operation to support M2M device with very long paging cycle. |  |
| Purpose | To be discussed and adopted by 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of M2M device with long paging cycle in idle mode

Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution proposes the paging operation of M2M device with long paging cycle in idle mode.

- Reference

[1] IEEE C802.16p-11/00018r1, P802.16p Amendment Working Document (AWD)

- Discussion

Proposal 1. M2M device with long paging cycle receives paging multiple times at paging cycle.

However, with such large value of paging cycle (e.g., above several minutes or hours), if M2M device misses the paging message, M2M device needs to get the paging message at the next paging cycle. Thus, whenever M2M devices wake up every long paging cycle, it can be beneficial that M2M devices check paging multiple times according to the additional paging offset to make sure reliable reception of paging, e.g., as shown in figure 1.

Figure 1. Multiple paging message transmission for M2M device with long paging cycle

Proposal 2. M2M device can perform the location update for error handling of the paging message reception.

Moreover, if the M2M device does not receive the paging message after multiple times, M2M device can perform the location update for requesting the paging message, e.g., as shown in figure 2.

Figure 2. Error handling of paging message via location update

- Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 16.2.18.7.1 as follows:]

16.2.18.7 Idle Mode for M2M application

16.2.18.7.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

When the M2M device was allocated the very long paging cycle (e.g., above several minutes or hours), the M2M device checks the AAI-PAG-ADV message multiple times according to the additional paging offset to make sure reliable reception of AAI-PAG-ADV message. After transmitting the AAI-PAG-ADV message with action code 0b0 (Performing network reentry), if the ABS does not receive AAI-RNG-REQ from the paged M2M device, the ABS may retransmit the AAI-PAG-ADV message. Every time the ABS retransmits the AAI-PAG-ADV message, it decreases the M2M Paging Retry Count by one. If the M2M device does not receive the AAI-PAG-ADV message until the M2M Paging Retry Count is exhausted, M2M device may perform the location update for requesting the AAI-PAG-ADV message.

[Remedy 2. Add the following changes on page 169, line 16 in 16.2.3.22 AAI-DREG-RSP]

16.2.3.22 AAI-DREG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Additional paging offset | TBD | Used to indicate additional paging offset for the M2M device. | Optional. |
| M2M Paging Retry Count | 2 | Used to indicate the number of the additional paging message transmission. M2M device can receive the paging message at the additional paging offset until this parameter is exhausted if the M2M device does not receive the paging message. | Optional. Shall be included when the additional paging offset parameter is present. |

[Remedy 3. Add the following changes on page 88, line 65 in 16.2.3.1 AAI-RNG-REQ]

16.2.3.1 AAI-RNG-REQ

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = Idle mode location update of M2M device for requesting the AAI-PAG-ADV message 0b1110-0b1111 = reserved |  |

[Remedy 4. Add the following changes on page 98, line17 in 16.2.3.2 AAI-RNG-RSP]

16.2.3.2 AAI-RNG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x6 = Success of location update of M2M device for requesting the AAI-PAG-ADV message 0x07~0xF: Reserved | Shall be included when this message is sent in response to an AAIRNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension or Idle mode location update of M2M device for requesting the AAI-PAG-ADV message. |
| if (Location Update Response == 0x6) { |  |  |  |
| Network reentry required | 1 | 0: Network reentry is not required. 1: Network reentry is required. |  |
| If (Network reentry required == 0) { |  |  |  |
| Paging Group ID update | 32 | Old PG ID, New PG ID The Paging Group ID update specifies a new PGID that replaces an old PGID. | Shall be included when the Paging Group ID is changed. |
| Paging Offset update | 24 | Old Paging Offset, New Paging Offset The Paging Offset update specifies a new Paging Offset that replaces an old Paging Offset. | Shall be included when the Paging Offset is changed. |
| New Paging Cycle | 4 | New Paging Cycle to be assigned to the M2M device 0b0000-0b1111 | Shall be included when the new Paging Cycle is assigned. |
| New Paging Group ID | 16 | New PGID of the M2M device | Shall be included when the new Paging Group ID is assigned. |
| New Paging Offset | 12 | New paging offset of M2M device which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. | Shall be included when the new Paging Offset is assigned. |
| New Paging Controller ID | 48 | The new Paging Controller ID which the M2M device shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------