| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of M2M device with long paging cycle in idle mode |  |
| Date Submitted | 2011-05-17 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak LG Electronics Hyunjeong Kang Samsung Electronics | giwon.park@lge.com +82-31-450-1879 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes the idle mode operation to support M2M device with very long paging cycle. |  |
| Purpose | To be discussed and adopted by 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of M2M device with long paging cycle in idle mode

Giwon Park, Youngsoo Yuk, Jin Lee, Jeongki Kim, Kiseon Ryu, Jin Sam Kwak

LG Electronics

Hyunjeong Kang

Samsung Electronics

- Introduction

This contribution proposes the paging operation of M2M device with long paging cycle in idle mode.

- Reference

[1] IEEE C802.16p-11/00018r1, P802.16p Amendment Working Document (AWD)

- Discussion

Proposal . M2M device with long paging cycle receives paging multiple times at paging cycle.

With such large value of paging cycle (e.g., above several minutes or hours), if M2M device misses the paging message, M2M device needs to get the paging message at the next paging cycle. Thus, whenever M2M devices wake up every long paging cycle, it can be beneficial that M2M devices check paging multiple times according to the additional paging offset to make sure reliable reception of paging, e.g., as shown in figure 1.

Figure 1. Multiple paging message transmission for M2M device with long paging cycle

- Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the text in 16.2.18.7.1 as follows:]

16.2.18.7 Idle Mode for M2M application

16.2.18.7.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

When the M2M device was allocated the very long paging cycle (e.g., above several minutes or hours) and multiple paging offsets, the M2M device may check the AAI-PAG-ADV message multiple times according to the additional paging offset to make sure reliable reception of AAI-PAG-ADV message. After transmitting the AAI-PAG-ADV message with action code 0b0 (Performing network reentry), if the ABS does not receive a response from the paged M2M device, the ABS may re-page this M2M device at the next paging offset via AAI-PAG-ADV message. Every time the ABS re-pages the M2M device at additional paging offset which is indicated in AAI-PAG-ADV message, ABS decreases the M2M Paging Retry Count by one. If the ABS does not receive a response from the paged M2M device until the M2M Paging Retry Count decreases to zero, the ABS determines that the M2M device is unavailable, and shall send a message over the backbone network to indicate that the list of M2M devices in idle mode shall be updated in all ABSs that belong to the same paging group.

[Remedy 2. Add the following changes on page 169, line 16 in 16.2.3.22 AAI-DREG-RSP]

16.2.3.22 AAI-DREG-RSP

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Additional paging offset | TBD | Used to indicate additional paging offset for the M2M device. | Optional. |
| M2M Paging Retry Count | 2 | Used to indicate the number of the additional paging message transmission. M2M device can receive the paging message at the additional paging offset until this parameter is exhausted if the M2M device does not receive the paging message. | Optional. Shall be included when the additional paging offset parameter is present. |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------