| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of clarification of the 802.16e/802.16m idle mode operation to support M2M communication |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Giwon Park, Jin Lee, Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak LG Electronics | giwon.park@lge.com +82-31-450-1879 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes to clarify the 802.16e.16m idle mode to support the M2M communication |  |
| Purpose | To be discussed and adopted by 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of clarification of the 802.16e/802.16m idle mode operation to support M2M communication

Giwon Park, Jin Lee, Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution proposes the clarification of the 802.16e/802.16m idle mode operation to support M2M communication.

- Reference

[1] IEEE C802.16p-11/00018r1, P802.16p Amendment Working Document (AWD)

[2] IEEE Std 802.16-2009, Part 16: Air interface for broadband wireless access systems

- Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Add a new subclause 6.3.23.10/6.3.23.10.1 on page 2, line 2 and add the new text in subclause (network reentry from idle mode for M2M devices) as follows:]

6. MAC common part sublayer

6.3.23.10 MS idle mode for M2M application

When an M2M device enters idle mode, the M2M device shall follow the procedures defined in 6.3.23 except the procedures described in this section. If an M2M device requires the longer inactive interval, longer paging cycle can be provided for the M2M device during the idle mode initiation through MOB-DREG-REQ/MOB-DREG-CMD messages. Idle mode timer for the M2M device may be assigned during idle mode initiation. In this case, the MOB-DREG-CMD message includes M2M device-specific Idle Mode Timer. When the M2M device receives the MOB-DREG-CMD message with M2M device-specific Idle Mode Timer, the M2M device shall perform location update prior to the expiration of the M2M device-specific Idle Mode Timer. At every location update including the paging group update, the M2M device-specific Idle Mode Timer is reset to 0 and restarted.

6.3.23.10.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

6.3.23.10.1 2Network reentry from idle mode for M2M devices

For network reentry from Idle Mode, ranging parameters may be different for M2M devices or M2M groups.

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in 6.3.10.3

For the network reentry indicated by a paging message that contains ranging configuration, the M2M device shall select a ranging opportunity according to the ranging configuration. Ranging configuration may include differentiated waiting offset time and backoff window size.

[Remedy 2. Add the following changes on page 2, line 14: add a new TLV in 11.14 as follows:]

11.14 DREG-CMD /REQ message encodings

11.14.1 MS-specific Idle mode timer

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M device-specific Idle mode timer | 53 | 3 | M2M device timed interval in frame to conduct location update. Timer recycles on successful idle mode location update. | DREG-CMD |

[Remedy 3. Add the following changes on page 2, line 5: add a new TLV in MOB_PAG-ADV message:]

6.3.2.3.51. MOB_PAG-ADV (BS broadcast paging) message

For M2M devices, the following TLV may be included in the MOB_PAG-ADV management message:

Waiting offset time

M2M group shall not begin ranging prior to the expiration of waiting time from the reception of paging message.

[Remedy 4. Add the following changes on page 3, line 61: add a new parameter in 16.2.3.22 AAI-DREG-RSP as follows:]

16.2.3.22 AAI-DREG-RSP

Table 699-AAI-DREG-RSP message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| M2M device-specific Idle Mode Timer | 24 | M2M device timed interval in frame to conduct location update. Timer recycles on successful idle mode location update. | May present when an M2M device enters idle mode. |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------