| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of MGID reassignment procedure in IEEE 802.16p system |  |
| Date Submitted | 2011-05-09 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on 802.16p Amendment Working Document (AWD) Content for Session #73 |  |
| Abstract | This contribution provides a text for MGID reassignment procedure in IEEE 802.16p system. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of MGID Reassignment Procedure in IEEE 802.16p system

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon

ETRI

# Proposal

According to the sections 16.2.1.3 in the AWD [1], the IEEE 802.16p system supports M2M Group Identifier (MGID). However, the reassignment procedure of MGID is TBD in the current AWD. Therefore, we propose the MGID reassignment procedure in this contribution.

# References

[1] IEEE 802.16p-10/0018r1, “IEEE 802.16p Machine to Machine (M2M) Amendment Working Document (AWD),” Mar. 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts.

- Texts deleted by this contribution: red strikeout texts.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

### 16.2.1.3.1 M2M Group Identifier (MGID)

An MGID is assigned by a network entity during initial network entry and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD. The MGID may be changed through the DSC procedure in connected mode and through the location update or network reentry procedures in idle mode.

### 16.2.3.2 AAI-RNG-RSP

Modify Table 679 as indicated

Table 679 – AAI-RNG-RSP Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| If (Location Update Response == 0x0 ) { |  |  |  |
| … | … | … | … |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| ... | ... | ... | ... |
| }//end of If (Location Update Response == 0x0 ) |  |  |  |
| … | … | … | … |
| If(it is under network re-entry for HO){ |  |  |  |
| … | … | … | … |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| ... | ... | ... | ... |
| }//end of If (it is under network re-entry for HO) |  |  |  |
| … |  |  |  |

### 16.2.3.47.4 AAI-DSC-REQ

Modify Table 737 as indicated

Table 737 – AAI-DSC-REQ Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| For(i=0;i<Num_MGID;i++){ |  | Num_MGID is the number of MGIDs to add[1..TBD] | Present when ABS initiates AAI-DSC-REQ Present only if MGID to be added exists |
| MGID to be added | 15 | MGID to be added | Present only if Num_MGID > 0 |
| FID | 4 | Multicast specific FID which is associated with newly added MGID | Present only if Num_MGID > 0 |
| } |  |  |  |
| For(i=0;i<Num_MGID;i++){ |  | Num_MGID is the number of MGIDs to delete[1..TBD] | Present when ABS initiates AAI-DSC-REQ Present only if MGID to be deleted exists |
| MGID to be deleted | 15 | MGID to be deleted | Present only if Num_MGID > 0 |
| } |  |  |  |
| ... | ... | ... | ... |

----------------- End of the text proposal ---------------------------------------------------------------------------------------