| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Short Message Services in 16e |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Honggang Li, Rui Huang, Shantidev Mohanty, Xiangying Yang Intel Inbar Anson Bratspiess Sequans Jin Lee LG Electronics | E-mail: Honggang.li@intel.com inbar@sequans.com jin1.lee@lge.com |
| Re: | 802.16p AWD |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Short Message Services in 16e

Honggang Li,

Rui Huang,

Shantidev Mohant,

Xiangying Yang

Intel

Inbar Anson Bratspiess

Sequans

# Introduction

The size of message sent by M2M devices ranges from several bytes to several hundred bytes, for small size message (e.g. less than 100 bytes message), short message service can work better esp. for the M2M devices in idle mode, because the normal process for the M2M devices in idle mode includes network re-entry, bandwidth request/allocation and data delivery so that the efficiency for short message delivery is very much low.

Short message services have been supported in 802.16m, and the SMS message can carried in AAI-RNG-REQ message, but 802.16e has not supported short message services. UL M2M SMS for M2M device in idle mode based on 16e is proposed in this contribution.

# M2M short message services based on 16e

- UL M2M SMS procedure

To support UL M2M SMS, two-round RNG-REQ/RSP are proposed as follows:

If there is short message at M2M device to send in idle mode, it follows the procedure as:

- The M2M device gets the ranging channel allocation from UL-MAP IE sent by the BS, then selects the ranging opportunity and CDMA code to do the random access

- If the BS receives the ranging code successfully without further correction of power and timing, it will send CDMA Allocation IE for RNG-REQ transmission with M2M SMS indication

- The M2M device sends the RNG-REQ with M2M SMS Request to indicate it has SMS to send with variable size.

- If the BS receives the RNG-REQ with M2M SMS Request successfully, it will send RNG-RSP with SMS Response, to accept or reject the M2M device’s request regarding if the BS has valid context information of this M2M device and/or if the BS has the enough resource for M2M SMS transmission. The context information may include security information. If the BS accepts the M2M device’s SMS request, the Basic CID will be assigned with a Temporary CID timer. Up to now, it is the first round of RNG-REQ/RSP used for M2M SMS Request and Response.

- If the BS accepts M2M SMS request indicated in RNG-RSP with SMS Response, the M2M device will listen to the UL-MAP IE for the channel allocation for RNG-REQ with M2M SMS. If the BS rejects M2M SMS request, there will be also an action code indicated in RNG-RSP to tell M2M device what next to do, e.g. to do network re-entry or to backoff and re-send M2M SMS request using a new ranging opportunity and CDMA code.

- If there is UL-MAP for RNG-REQ with M2M SMS, the M2M device will send SMS, then wait for the confirmation from the BS.

- The BS will send RNG-RSP with SMS confirmation to indicate if SMS packet is received successfully. It is the second round of RNG-REQ/RSP used for M2M SMS and Confirmation.

Figure 1: UL M2M SMS procedure

- M2M SMS TLV in 16e RNG-REQ

To support M2M short message services in 16e RNG-REQ, 2 new TLV parameters shall be supported, M2M SMS Request TLV which will appear in RNG-REQ with M2M SMS Request and M2M SMS TLV which will appear in RNG-REQ with M2M SMS.

- M2M SMS Request TLV

“M2M SMS Request” is used to tell the BS that the M2M device has SMS to send with variable size, the TLV encoding is shown in table 1,

Table 1, M2M SMS Request TLV

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| M2M SMS Request | 14 | 1 | Bit 0: M2M SMS indicator 0b0: M2M SMS request 0b1: not M2M SMS request Bit 1-7: If(if Bit 0 == 0b0){ the size of M2M SMS } else { Reserved } | All |

- M2M SMS TLV

“M2M SMS” is used to carry the M2M SMS, with the length equal to or less than the requested packet size. The length of SMS body can be up to 140 bytes.

Table 2, M2M SMS TLV

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| M2M SMS | 15 | Variable | M2M SMS message | All |

For M2M short message service in idle mode, the encodings of RNG-REQ can be simplified to save the overhead:

Only the TLV parameters related to M2M SMS are included in the RNG-REQ for M2M SMS, the TLV parameters besides the above three are:

- SS MAC address

- CMAC_KEY_COUNT

- M2M SMS TLV in 16e RNG-RSP

To support M2M short message services in 16e RNG-RSP, 3 new TLV parameters shall be supported, M2M SMS Response TLV which will appear in RNG-RSP with M2M SMS Response, together with Temporary Basic CID assignment timer, and M2M SMS Confirmation TLV which will appear in RNG-RSP with M2M SMS Confirmation.

- M2M SMS Response TLV

“M2M SMS response” is used to respond to M2M SMS request, to accept or reject the SMS request, the TLV encoding is shown in table 3,

Table 3, M2M SMS Response TLV

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| M2M SMS Response | 14 | 1 | Bit 0-1: accept or reject SMS request 0b00: accept 0b11: reject If(accept){ Bit 2-3: reject reason } else { Reserved } Bit 4-7: Reserved | All |

- Temporary Basic CID Timer TLV

Temporary Basic CID Timer is used to trigger the termination of Temporary CID assignment for M2M SMS.

Table 4, Temporary Basic CID Timer TLV

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| Temporary Basic CID Timer | 15 | TBD | TBD | All |

- M2M SMS Confirmation TLV

To support UL M2M SMS reliably, RNG-RSP can carry M2M SMS confirmation TLV to ACK or NACK the M2M SMS transmission.

Table 5, M2M SMS Confirmation TLV

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| M2M SMS Confirmation | 16 | 1 | Bit 0-1: SMS confirmation Bit 0 – ACK Bit 1 – NACK Bit 2-7: Reserved | All |

# Proposed texts

3.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 5 in page 2.]

6.3.2.3.5 RNG-REQ (ranging request) message

……

The following TLV shall be included in the RNG-REQ message when the M2M device is attempting to send a short message:

M2M Short Message Request (See 11.5)

This field contains the request of M2M SMS including the size of SMS.

M2M Short Message (See 11.5)

This field contains the content of M2M SMS.

……

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 5 in page 2.]

11.5 RNG-REQ management message encodings

[Modify Table582 as follows]

Table 582—RNG-REQ message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M2M SMS Request | 14 | 1 | Bit 0: M2M SMS indicator 0b0: M2M SMS request 0b1: not M2M SMS request Bit 1-7: If(if Bit 0 == 0b0){ the size of M2M SMS } else { Reserved } | All |
| M2M Short Message | 15 | Variable | M2M SMS message | All |

----------------------------------------------  Text End -----------------------------------------------

3.3 Proposed Text #3

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 13 in page 2.]

6.3.2.3.6 RNG-RSP (ranging response) message

……

The following TLV shall be included in the RNG-REQ message when the BS receives SMS sent by an M2M device:

M2M Short Message Response (See 11.6)

This field contains the response to M2M SMS request, accepting or rejecting the request.

M2M Short Message Confirmation (See 11.6)

This field contains the confirmation to M2M SMS transmission.

Temporary Basic CID Timer(See 11.6)

This field contains the timer for the termination of Temporary Basic CID assignment to M2M SMS transmission.

----------------------------------------------  Text End -----------------------------------------------

3.4 Proposed Text #4

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 5 in page 2.]

11.6 RNG-RSP management message encodings

[Modify Table585 as follows]

Table 585—RNG-RSP message encodings

| Name | Type(1byte) | Length | Value | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M2M SMS Response | 14 | 1 | Bit 0-1: accept or reject SMS request 0b00: accept 0b11: reject If(accept){ Bit 2-3: reject reason } else { Reserved } Bit 4-7: Reserved | All |
| Temporary Basic CID Timer | 15 | TBD | TBD | All |
| M2M SMS Confirmation | 16 | 1 | Bit 0-1: SMS confirmation Bit 0 – successful Bit 1 – failed Bit 2-7: Reserved | All |

----------------------------------------------  Text End -----------------------------------------------

3.5 Proposed Text #5

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 13 in page 2.]

##### M2M Short Messaging services

In idle mode, M2M device can send SMS in RNG-REQ in UL, the length of M2M SMS body is up to 140 bytes.

After the ranging code and UL parameter correction, the M2M device sends RNG-REQ with SMS Request, if RNG-REQ is received successfully, the BS will send RNG-RSP with SMS Response to indicate if accept or reject the SMS request. If the SMS request is accepted, the BS will allocate the channel resource for RNG-REQ with SMS using UL MAP IE. The M2M device sends RNG-REQ with M2M SMS, and the BS sends RNG-RSP with SMS Confirmation to confirm if the M2M SMS is received successfully, if the confirmation is “successful”, the M2M device will back to idle mode; if the confirmation is “failed”, the M2M device will re-transmit the SMS packet in the same allocated resources or new channel resource allocated in UL MAP IE, the No. of re-transmissions is FFS.

----------------------------------------------  Text End -----------------------------------------------