| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated ranging channel indication for M2M in 16e |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Honggang Li, Shantidev Mohant, Rui Huang, Xiangying Yang Intel | E-mail: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dedicated Ranging Channel Indication for M2M in 16e

Honggang Li,

Shantidev Mohant,

Rui Huang,

Xiangying Yang

Intel

# Introduction

Dedicated ranging channel for M2M devices was agreed on in March meeting, to address the large ranging load coming from the M2M devices.

This contribution proposes changes to the 16e standard to implement the dedicated ranging channel for M2M devices.

# Proposed Text

2.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 14 in page 2.]

8.4.5.4.4 UL-MAP Extended IE

8.4.5.4.4.1 UL-MAP Extended IE format

[Modify Table 381 as follows]

Table 381—Extended UIUC code assignment for UIUC = 15

| Extended UIUC (hexadecimal) | Usage |
| --- | --- |
| 0x0 | Power Control IE |
| 0x1 | Reserved |
| 0x2 | AAS UL IE |
| 0x3 | CQICH Allocation IE |
| 0x4 | UL Zone IE |
| 0x5 | UL-MAP Physical Modifier IE |
| 0x6 | Reserved |
| 0x7 | UL-MAP Fast Tracking IE |
| 0x8 | UL PUSC Burst Allocation in Other Segment IE |
| 0x9 | Fast Ranging IE |
| 0xA | UL Allocation Start IE |
| 0xB | Dedicated Ranging Channel for M2M IE |
| 0xC … 0xF | Reserved |

----------------------------------------------  Text End -----------------------------------------------

2.2 Proposed Text #2

----------------------------------------------  Text Start -----------------------------------------------

[Add the following text after line 14 in page 2.]

8.4.5.4.4 UL-MAP Extended IE

8.4.5.4.4.30 Dedicated Ranging Channel for M2M IE format

The Dedicated Ranging Channel for M2M IE is used to indicate if the allocated ranging channel is used for M2M devices, the M2M devices can be identified by an MGID or an M2M identifier valid in idle mode, there is an M2M group indicator to indicate the identification type for M2M devices.

[Create Table xxx as follows]

Table xxx—Dedicated Ranging Channel for M2M IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Dedicated Ranging Channel for M2M IE(){ | - | - |
| Extended UIUC | 4 | Dedicated Ranging Channel = 0xB |
| Length | 4 | Length is TBD |
| M2M group indicator | 1 | 0x1 = MGID, 0x0 = M2M ID |
| M2M Group Identifier | TBD | If M2M group indicator = 0x1 “MGID == 0x0000” indicates all M2M devices |
| M2M identifier | TBD | If M2M group indicator = 0x0 Identifier of M2M in idle mode |
| Dedicated Ranging Channel for M2M | 1 | 0x1 = M2M devices can use the allocated ranging channel 0x0 = M2M devices cannot use the allocated ranging channel |
| OFDMA Symbol offset | 8 | - |
| Subchannel offset | 7 | - |
| No. OFDMA Symbols | 7 | - |
| No. Subchannels | 7 | - |
| } | - | - |

----------------------------------------------  Text End -----------------------------------------------