| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized schemes of network re-entry for M2M in 16m |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Honggang Li, Shantidev Mohant, Rui Huang, Xiangying Yang Intel | E-mail: Honggang.li@intel.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized schemes of network re-entry for M2M in 16m

Honggang Li,

Shantidev Mohant,

Rui Huang,

Xiangying Yang

Intel

# Introduction

The traffic characteristics of M2M devices for different purposes are different, some are periodic traffic or in some known pattern, the others are burst in unknown pattern, for the devices with the traffic in the known pattern, the contention free mechanism can be used for network re-entry if the device in idle mode, for the device with the traffic in the unknown pattern, the contention based mechanism should be kept.

Another factor to be considered is mobility characteristics of M2M devices, for fixed M2M device, the large scale channel condition can be much more stable, and the small scale channel condition in UL can be measured through DL synchronization and measurement esp. in TDD mode, so that the ranging process for UL synchronization can be saved. But for mobile M2M device, the ranging process should be kept for reliable measurement and transmission in UL.

Based on this analysis, the optimization of network re-entry for M2M in 16m is proposed in this following section.

# Optimization of network re-entry for M2M

For different category of M2Mmobility and traffic characteristics, the mechanism of network re-entry can be optimized and categorized as shown in the following table:

Table 1, optimized schemes of network re-entry for M2M

|  | Known traffic pattern | Unknown traffic pattern |
| --- | --- | --- |
| Fixed | - Paging/grouping paging, cell/multi-cell wide - Dedicated ranging channel allocation for contention free access - Ranging process without ranging code sent | - Common /Dedicated ranging channel allocation for contention based access - Ranging process with ranging code - S-RCH or NS-RCH can be used |
| Mobile | - Paging/grouping paging, multi-cell wide - Dedicated ranging channel allocation for contention free access - NS-RCH should be used | - Common/Dedicated ranging channel allocation for contention based access - NS-RCH should be used |

Based on the mobility and traffic characteristics of the M2M device, the BS can select the proper network re-entry scheme for M2M device, the selection can be based on the table-1, and the BS shall indicate to the M2M device which network re-entry scheme in AAI-PAG-ADV or some other control messages.

The mobility and traffic characteristics of the M2M device can be known by:

- explicitly AAI-DSA-REQ(traffic characteristics) and/or AAI-REG-REQ(SS capability) and/or location update(mobility)

- Or the implicit statistics by the BS or some network entity

- Or pre-provision to the BS from M2M server or other network entity

# Proposed texts

3.1 Proposed Text #1

[Add the following text after line 17 in page 7.]

----------------------------------------------  Text Start -----------------------------------------------

16.2.18.7.2 Network reentry from idle mode for M2M devices

……

Based on the mobility and traffic characteristics of the M2M device, the ABS can select the proper network re-entry scheme for M2M device based on the table-xxx, and the ABS shall inform the M2M device the network re-entry scheme indicator in AAI-PAG-ADV message.

Table xxx, Scheme selection of network re-entry for M2M

| Network re-entry scheme indicator | Network re-entry scheme | Note |
| --- | --- | --- |
| 0 | Dedicated ranging channel allocation for M2M group or M2M device in AAI-PAG-ADV, NS-RCH is used | Mobile/fixed M2M, known traffic pattern, paging |
| 1 | Dedicated ranging channel allocation for M2M group or M2M device in AAI-PAG-ADV, S-RCH is used | Fixed M2M,unknown traffic pattern, paging |
| 2 | Dedicated channel allocation for AAI-RNG-REQ, A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV | Fixed M2M, known traffic pattern, paging |
| 3 | Dedicated ranging channel for M2M group or M2M device in UL-AMAP IE | All M2M |
| 4 | Common ranging channel | All M2M |

If the network re-entry indicator is set to “0”, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the M2M device can find the dedicated CDMA code assignment and transmission opportunity offset in “Dedicated channel allocation” of AAI-PAG-ADV message for ranging and network re-entry.

If the network re-entry indicator is set to “1”, the M2M device doesn’t need to send CDMA code for ranging but sends RNG-REQ message with the channel allocation in “Dedicated channel allocation” in AAI-PAG-ADV message.

If the network re-entry indicator is set to “2”, the ABS shall allocate the dedicated ranging channel for M2M device in UL-AMAP message.

If the network re-entry indicator is set to “3”, the ABS shall indicate to M2M device to use common ranging channel for network re-entry.

----------------------------------------------  Text End -----------------------------------------------

3.2 Proposed Text #2

[Add the following text after line 17 in page 7.]

----------------------------------------------  Text Start -----------------------------------------------

[Modify Table700 as follows]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Table 700—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI-PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID-Info message. 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | L equals the Num_PGIDs in PGID-Info message |
| For (i=0; i<M; i++) { |  |  | M equals the number of Bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| For (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the MS to be paged. … | Present if the S-SFH Network Configuration bit == 0b0 |
| … |  |  |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |
| For (j=0; j<Num_M2MGroup; j++) { |  |  | the number of M2M group in corresponding paging group. |
| M2M Group ID | TBD | M2M Group to be paged |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message. 0b0: All M2M devices within the corresponding M2M Group perform the network reentry. 0b1: All M2M devices within the corresponding M2M Group perform the ranging for location update. |  |
| For (k=0; k<Num_M2M devices; k++) { |  | Num_ M2M devices indicates the number of paged M2M devices in a corresponding M2M group. |  |
| M2M ID | TBD | Used to indicate M2M ID valid in jth M2M group for kth M2M device to be paged. | Optional Shall be presented when the M2M devices are individually paged. |
| M2M network re-entry indicator | 3 | Indicate the network re-entry scheme for M2M device 0b000: dedicated ranging channel allocation, NS-RCH 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated channel allocation for AAI-RNG-REQ 0b011: dedicated ranging channel allocation in A-MAP IE 0b100: Common ranging channel allocation 0b101-0b111: reserved |  |
| Dedicated channel allocation | TBD | If M2M network re-entry indicator = 0 or 1, to indicate dedicated ranging channel allocation; If M2M network re-entry indicator = 2, to indicate Resource Index for AAI-RNG-REQ; |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update | Optional Shall be presented when the M2M devices are individually paged. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: this is the last fragment of the AAIPAG-ADV message 0b1: this is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: reserved 0b1: there is emergency information | Optional Present if there is emergency information |

----------------------------------------------  Text End -----------------------------------------------