| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized schemes of network re-entry for M2M in 16m |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Honggang Li, Shantidev Mohanty, Rui Huang, Xiangying Yang Intel Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI Jinsoo, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics Yi-Ting Lin Institute for Information Industru (III) | E-mail: Honggang.li@intel.com aj@itri.org.tw js.choi@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized schemes of network re-entry for M2M in 16m

Honggang Li,Shantidev Mohanty,Rui Huang,Xiangying Yang

Intel

Wei-Chieh Huang, Chia-Lung Tsai, Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

Yi-Ting Lin

Institute for Information Industru (III)

# Introduction

The traffic characteristics of M2M devices for different purposes are different, some are periodic traffic or in known pattern, the others are burst in unknown pattern, for the devices with the traffic in known pattern, the contention free mechanism can be used for network re-entry if the device in idle mode, for the device with the traffic in unknown pattern, the contention based mechanism should be kept.

Another factor to be considered is mobility characteristics of M2M devices, for fixed M2M device, the large scale channel condition can be much more stable, and the small scale channel condition in UL can be measured through DL synchronization and measurement esp. in TDD mode, so that the ranging process for UL synchronization can be simplified and in lower complexity. But for mobile M2M device, the ranging process should be kept for reliable measurement and transmission in UL.

The control messages for network re-entry scheme can be targeted to all M2M devices or a group of M2M devices, and in pseudo-static or dynamic manner. To configure and indicate the network re-entry schemes effectively, the mechanism of hierarchical control messaging is needed.

# Hierarchical control messaging

The hierarchical model of control messaging for M2M network re-entry and ranging is shown in figure 1. Basically there are three kinds of control messages can be used as follows:

- Secondary SFH IE, used to indicate whether normal or dedicated ranging allowed to all M2M devices. Such kind of indication should be regarded as essential system parameters and system configuration information.

- AAI-SCD, used to configure the ranging channel including the usage of ranging codes to all M2M devices periodically. M2M devices must confirm/update the information in every re-entry stage, so M2M devices who use dedicated ranging can easily obtain the related information with assured reliability through the AAI-SCD update procedure.

- AAI-PAG-ADV, used to configure the ranging channel including the usage of ranging codes to group of M2M devices dynamically. If dedicated ranging channel is used for group-based M2M device network re-entry, it is proper to carry this information in AAI-PAG-ADV message, because it can be designed to support group-based M2M control in idle mode.

Figure 1, hierarchical model of control messaging for M2M network re-entry and ranging

# Periodic system configuration of dedicated ranging channel usage for M2M using S-SFH and AAI-SCD

The system-wide ranging configuration can be broadcasted in S-SFH SP3 and AAI-SCD messages, S-SFH SP3 is only used to indicate normal and/or dedicated ranging channel for M2M devices, and AAI-SCD is used to notify the basic ranging configuration to M2M devices.

- M2M ranging indicator in S-SFH SP3 IE

To control the access load between the dedicated ranging and normal (shared) ranging with existing AMSs, BS can indicate the usage of ranging for M2M devices according to system environment. 2-bit indicator can be designed to indicate normal ranging, dedicated ranging, normal and dedicated ranging, ranging disabled etc. in S-SFH SP3 IE.

- M2M ranging configuration in AAI-SCD

  - Ranging opportunity index: Indicates the index of the allocated ranging opportunity of the ranging channel used in the RA-ID. Except ‘0b00’ for NS-RCH and ‘0b11’ for S-RCH, we can set one of ‘0b01’ and ‘0b10’ for dynamic NS-RCH to be used for the M2M dedicated ranging.

  - Ranging opportunity subframe index: Indicates the subframe index of the allocated ranging opportunity. The subband of a ranging channel is same as the ranging channel allocated by SFH.

  - Periodicity of ranging: Indicates the periodicity of ranging allocation.

- M2M configuration change count and M2M SCD count: To avoid the unnecessary decoding/update the dedicated ranging information for M2M devices, additionally BS can transmit M2M configuration change count and M2M SCD count like Configuration Change Count in AAI-SCD and SCD count in S-SFH SP3. Except these information is targeting for M2M devices, the basic operation with these fields is same to existing Configuration Change Count and SCD count as defined in 16.2.3.31.

# Network re-entry scheme indication using AAI-PAG-ADV

For different category of M2M mobility and traffic characteristics, the mechanism of network re-entry can be optimized and categorized as shown in the following table:

Table 1, optimized schemes of network re-entry for M2M

|  | Known traffic pattern | Unknown traffic pattern |
| --- | --- | --- |
| Fixed | - Paging/grouping paging - Dedicated ranging channel allocation for contention free access - Ranging process without ranging code sent | - Common /Dedicated ranging channel allocation for contention based access - Ranging process with ranging code - BR channel or S-RCH can be used |
| Mobile | - Paging/grouping paging - Dedicated ranging channel allocation for contention free access - NS-RCH should be used | - Common/Dedicated ranging channel allocation for contention based access - NS-RCH should be used |

Based on the mobility and traffic characteristics of the M2M device, the BS can select the proper network re-entry scheme for M2M device, the selection can be based on table-1, and the BS shall indicate to the M2M device which network re-entry scheme in AAI-PAG-ADV.

The mobility and traffic characteristics of the M2M device can be known by:

- explicitly AAI-DSA-REQ(traffic characteristics) and/or AAI-REG-REQ(SS capability) and/or location update(mobility)

- Or the implicit statistics by the BS or some network entity

- Or pre-provision to the BS from M2M server or other network entity

# Proposed texts

5.1 Proposed Text #1

[Add the following text after line 10 in page 8.]

----------------------------------------------  Text Start -----------------------------------------------

16.3.5.5.1.2 S-SFH IE

Table 835 - S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| M2M ranging indicator | 2 | Indicate the ranging configuration for M2M devices. 0b00: normal ranging as defined in Table 833 in 16.3.5.5.1.2 0b01: dedicated ranging for M2M devices 0b10: allow both normal and dedicated ranging 0b11: no ranging (not allow network re-/entry) |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |
| M2M SCD count | 4 | The value is increased whenever the contents of the dedicated ranging information for M2M devices are changed. The value rolls over from 0 to 15. The operation of this field is same with SCD count as defined in 16.2.3.31. |
| } |  |  |
| ... | ... | ... |

----------------------------------------------  Text End -----------------------------------------------

5.2 Proposed Text #2

[Add the following text after line 61 in page 3.]

----------------------------------------------  Text Start -----------------------------------------------

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 708 - AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| If ((M2M ranging indicator == 0b01 ) or (M2M ranging indicator == 0b10 ) { |  |  |  |
| M2M Configuration Change Count | 4 | The value is increased whenever the contents of the dedicated raging information for M2M devices are changed. The value rolls over from 0 to 15. The operation of this field is same with Configuration Change Count as defined in 16.2.3.31. |  |
| M2M ranging opportunity index | 1 | Indicates the index of the allocated ranging opportunity of the ranging channel used in the RA-ID dedicated for M2M devices. 0b0: 0b01 0b1: 0b10 |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. |  |
| Periodicy of the M2M ranging | [3] | Indicates the periodicy of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: reserved] |  |
| } |  |  |  |
| ... | ... | ... |  |

----------------------------------------------  Text End -----------------------------------------------

5.3 Proposed Text #3

[Add the following text after line 10 in page 7.]

----------------------------------------------  Text Start -----------------------------------------------

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. The information of dedicated ranging resources is transmitted in the AAI-SCD message. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2. The configuration of ranging assignment for M2M devices is indicated through M2M ranging indicator in the S-SFH SP3.

----------------------------------------------  Text End -----------------------------------------------

5.4 Proposed Text #4

[Add the following text after line 17 in page 7.]

----------------------------------------------  Text Start -----------------------------------------------

16.2.18.7.2 Network reentry from idle mode for M2M devices

……

Based on the mobility and traffic characteristics of the M2M device, the ABS can select the proper network re-entry type for M2M device based on the table-xxx, and the ABS shall inform the M2M device the network re-entry type in AAI-PAG-ADV message.

Table xxx, Scheme selection of network re-entry for M2M

| Network re-entry type | Network re-entry scheme | Note |
| --- | --- | --- |
| 0 | Dedicated channel allocation for AAI-RNG-REQ, A-MAP IE offset for AAI-RNG-REQ is indicated in AAI-PAG-ADV | Fixed M2M, known traffic pattern, UL synchronization not required |
| 1 | Dedicated ranging channel allocation for M2M group, S-RCH used for ranging | Fixed M2M, UL Synchronization required |
| 2 | Dedicated ranging channel allocation for M2M group, NS-RCH used for ranging | Mobile M2M, known traffic pattern |

If the network re-entry type is set to “0”, the M2M device doesn’t need to send CDMA code for ranging but sends RNG-REQ message with the channel allocation in “Dedicated channel allocation” in AAI-PAG-ADV message.

If the network re-entry type is set to “1”, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated S-RCH allocation is used for ranging.

If the network re-entry type is set to “2”, the ABS shall allocate the dedicated ranging channel for M2M device in AAI-PAG-ADV message, the dedicated NS-RCH allocation is used for ranging.

----------------------------------------------  Text End -----------------------------------------------

5.5 Proposed Text #5

[Add the following text after line 17 in page 7.]

----------------------------------------------  Text Start -----------------------------------------------

[Modify Table700 as follows]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Table 700—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI-PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID-Info message. 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | L equals the Num_PGIDs in PGID-Info message |
| For (i=0; i<M; i++) { |  |  | M equals the number of Bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| For (j=0; j<Num_AMSs; j++) { |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the MS to be paged. … | Present if the S-SFH Network Configuration bit == 0b0 |
| … |  |  |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |
| For (j=0; j<Num_M2MGroup; j++) { |  |  | the number of M2M group in corresponding paging group. |
| M2M Group ID | TBD | M2M Group to be paged |  |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message. 0b0: All M2M devices within the corresponding M2M Group perform the network reentry. 0b1: All M2M devices within the corresponding M2M Group perform the ranging for location update. |  |
| If(Action code == 0b0){ |  |  |  |
| M2M network re-entry type | 3 | Indicate the network re-entry scheme for M2M device 0b000: dedicated channel allocation for AAI-RNG-REQ 0b001: dedicated ranging channel allocation, S-RCH 0b010: dedicated ranging channel allocation, NS-RCH 0b011-0b111: reserved |  |
| Dedicated channel allocation | TBD | If M2M network re-entry type = 0, to indicate Resource Index for AAI-RNG-REQ; If M2M network re-entry type = 1,2 to indicate dedicated ranging channel allocation; |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: this is the last fragment of the AAI-PAG-ADV message 0b1: this is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: reserved 0b1: there is emergency information | Optional Present if there is emergency information |

----------------------------------------------  Text End -----------------------------------------------