| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Dedicated Ranging for M2M Devices in Network Re-entry |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics | js.choi@lge.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document IEEE 802.16p-10/0018r1 |  |
| Abstract | The contribution provides the proposed text of dedicated ranging for M2M devices in network re-entry to be included in IEEE 802.16p amendment document. |  |
| Purpose | To be discussed and adopted in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Dedicated Ranging for M2M Devices in Network Re-entry

Jinsoo Choi, HanGyu Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

In the previous #72 Singapore meeting, it has been accepted that BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. M2M devices will perform ranging for network (re-)entry using assigned dedicated ranging resources [1]. In current 802.16m/D12, there is only dedicated ranging allocation for handover ranging so it is need to define the dedicated ranging procedure for M2M devices who try network re-entry from idle mode. In this proposal, we review the existing ranging utilization based on existing 802.16m system and provide the proposed text to support the dedicated ranging for M2M devices.

- Rationale and Proposal

- Current ranging utilization in 802.16m

Based on existing 802.16m system, the ranging preamble (RP) codes for NS-RCH are classified into initial ranging and handover ranging preamble codes that can be partitioned into maximum 32 codes, respectively (total 64 codes), through RP code partition information for NS-RCH in S-SFH SP1. Because the actual code number for NS-RCH to be designed in 16m is more than 64, so BS can pick dedicated ranging code set which are distinguished from aforementioned 64 RP codes to avoid potential collision between shared ranging and dedicated ranging codes.

- Issue on dedicated ranging for M2M devices in network re-entry

When M2M devices who are assigned by dedicated ranging code and opportunity try network re-entry from idle mode, they will decode its own CDMA Allocation A-MAP IE to get the information for RNG-REQ resource allocation. In each CDMA Allocation A-MAP IE, CRC masking with RA-ID that is composed of superframe number, frame index, preamble code index and opportunity index is done to distinguish from IEs toward other AMSs or M2M devices. In RA-ID, current preamble code index with 6bits can indicate only 64 RP code index, e.g. 0~31 for initial ranging and 32~63 for handover ranging so there is no way to make a distinction between existing 64 RP code index and dedicated ranging code index for M2M devices.

- Proposed method to resolve the problem

In RA-ID, there is opportunity index with 2bits which can indicate the usage of ranging as following

- NS-RCH with ‘0b00’

- S-RCH with ‘0b11’

- Dynamic NS-RCH with ‘0b01’ and ’0b10’

The main purpose of dynamic NS-RCH is to improve the handover reliability. If it doesn’t cause critical impact to the system to operate dynamic ranging with one opportunity index instead of two, we can utilize remaining opportunity index for M2M dedicated ranging to distinguish RA-ID in CDMA Allocation A-MAP IE. For example, the system can employ opportunity index ‘0b01’ as dynamic ranging and opportunity index ‘0b10’ as M2M dedicated ranging, when the dedicated ranging is assigned for M2M devices.

- References

[1] IEEE C802.16p-10/0018r1, “P802.16p Amendment Working Document (AWD)”

[2] IEEE C802.16p-10/0004r2, “802.16p M2M system requirements document (SRD)”

- Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------------  Text Start  --------------------------------------------------------------

In line 10 of page 7 in C802.16p-10/0018r1, insert the description as following

16.2.18.7.2 Network re-entry from idle mode for M2M devices

…

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. When BS assigns the CDMA Allocation A-MAP IEs for RNG-REQ to those M2M devices, the opportunity index in RA-ID masked for the CDMA Allocation A-MAP IEs can be set to one of opportunity index ‘0b01’ and ‘0b10’. In this case, the opportunity index for assignment of the dynamic NS-RCH shall be set to the other value.

If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2.

-------------------------------------------------------  Text End  ----------------------------------------------------------------