| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Handover in 16p System |  |
| Date Submitted | 2011-5-8 |  |
| Source(s) | Guanghui Fan, Erik Colban Huawei Technologies | E-mail: fanguanghui@huawei.com ecolban@huawei.com |
| Re: | IEEE 802.16p call for AWD Content |  |
| Abstract | Proposal for handover scheme in 16p |  |
| Purpose | To be discussed and adopted in IEEE 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Handover in 16p System

Guanghui Fan, Erik Colban

Huawei Technologies

# Introduction

In 16m and 16e, the NBR-ADV message is broadcast periodically to expedite the scanning and handover procedures. The NBR-ADV message broadcasting interval is defined for each as a global value. However, most M2M devices are expected to be fixed or seldom move. In some deployment scenarios, the M2M devices may coexist with a number of legacy MSs or AMSs that may vary over time and sometimes be zero. For this reason, this proposal suggests to allow the BS to dynamically change the broadcasting interval of the NBR-ADV message.

This proposal also suggests that the devices report their mobility capabilities during the network entry or reentry procedure and also be able to request the ABS to transmit neighbor base station information and scanning opportunities by sending an AAI-SCN-REP message.

# Text Proposal

------------------------------------------------Start of the Proposed Text Part 1------------------------------------------------

16.2.6.6 Handover support in M2M deployment scenario

16.2.6.6.1 Network topology acquisition

When 	M2M devices are deployed, the ABS may change the period at which it transmits the AAI-NBR-ADV message. For example, it may adjust the period based on the number of coexisting M2M devices and AMSs and their mobility capabilities or based on the frequency of received handover requests. The method used to determine an optimal transmission period is implementation dependent.

If the BS is not broadcasting the AAI-NBR-ADV message, an AMS may request the ABS to unicast the AAI-NBR-ADV message by sending an AAI- SCN-REP message. Upon receiving the AAI- SCN-REP message with neighbor information request, the ABS shall transmit the AAI-NBR-ADV message to the AMS. An AMS may also use Scanning_Opportunities_Required field in the AAI- SCN-REP message to request the ABS to allocate scanning intervals at the same time. In this case, the ABS shall include the scanning details or deny the scanning request in the unicast AAI-NBR-ADV message sent in response.

------------------------------------------------End of the Proposed Text Part 1-------------------------------------------------

------------------------------------------------Start of the Proposed Text Part 2------------------------------------------------

[Insert the following field to AAI-RNG-REQ message]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M Device Mobility Feature | 2 | Indication of expected mobility . 0b00: No mobility 0b01: Low mobility 0b10: Normal mobility 0b11: Reserved | Present for M2M devices |

Note:  1) No mobility means a fixed M2M device. 2) Low mobility means the device will not move frequently. 3) Normal mobility means the device can move fast.

------------------------------------------------End of the Proposed Text Part 2-------------------------------------------------

------------------------------------------------Start of the Proposed Text Part 3------------------------------------------------

16.2.3.16 AAI-SCN-REP

Table 693—AAI-SCN-REP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan Opportunities Required | 1 | Request Indication of Scan Opportunities |  |
| Neighbor_Request_Indication | 1 | Request indication for system information and the list of the neighbor Femto ABSs | Optional |
| If(Neighbor_Request_Indication == 1) { |  |  |  |
| Neighbor_Request_BS_Type | 3 4 | Indicates type of neighbor Femto ABSs for which system information is requested. Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS Bit #3: macro ABS | Optional |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs | Optional. Sent by AMS to aid in generation of optimized neighbor list by the ABS |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

------------------------------------------------End of the Proposed Text Part 3-------------------------------------------------