| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging to poll M2M uplink transmission |  |
| Date Submitted | May 8, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics | {jin1.lee, youngsoo.yuk, kiseon.ryu}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose a paging message to be used for uplink data transmission and its related operation. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging to poll uplink data transmission

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

# Background

Many of M2M application (e.g., smart metering, healthcare equipment) generate delay tolerable uplink data traffic which means those traffics can be controlled by BS to help access distribution from a large number of M2M devices. In that sense, we would like to introduce the paging message to trigger uplink data transmission from M2M devices and to manage network load by optionally paging M2M devices based on traffic load at every paging cycle.

# Proposal

- Usage of paging message

Fig1. Paging message with action code : UL report

Some M2M devices are allowed to send data only when they are requested by using paging message with uplink report indication (e.g., action code) in order to reduce access congestion. Even if the M2M device can have a chance to send data at every paging cycle, the BS may not poll the M2M device in order to manage traffic load/access congestion. Also, the M2M device may not do any action when it has no data to report upon reception of the paging message with uplink report indication.

- Load control using paging message

When the BS does not poll M2M device, the M2M device shall defer to send the data and see if it is allowed to send the data at next paging cycle. However, if the BS does not poll the M2M device continuously, the data from the M2M device could be outdated and never get to perform ranging to send data. Therefore, the following procedure is recommended.

Fig 2. Load control with paging message

- During idle mode initiation, ‘Transmission type’ and ‘Max time duration to wait for paging message’ can be negotiated. Transmission type is to notify the M2M device of uplink data transmission only after receiving a paging indication. Max time duration is for M2M device to wait for a paging message to send data.

- If the M2M device is in idle and it sends data based on the polling from paging message (Transmission type is set to 1), the M2M device shall check if the paging message includes its identification with uplink report indication (Action code 0x10).

- If the M2M device does not have at least one chance to send data during ‘Max time duration’, the M2M device may perform ranging and inform BS of it (with reception of paging message indicator in AAI-RNG-REQ).

  - Without the ‘reception of paging message indicator’ , the BS may reject the ranging access from the M2M device which is supposed to receive the paging indication before ranging access. Therefore, it is more clear to notify the BS of the fact that the current ranging access is after expiration of ‘Max time duration to wait for AAI-PAG-ADV (Action Code =0b10)’.

# Text Proposal

[Remedy1] Add the following texts at the end of section 16.2.18.7.1, in page 6 line 65.

16.2.18.7.1 Paging operation

Group paging may be used for M2M devices. For this, M2M Group Identifier (MGID) defined in 16.2.1.3.1 may be included in a paging message instead of an individual identifier to identify the group of M2M devices.

Paging message with action code 0b10 is used to poll M2M devices for uplink data transmission.  If the Transmission Type of the M2M device is set to 1 and the paging message does not include M2M device ID with action code 0b10, the M2M device shall wait for the paging message during the maximum time duration negotiated during idle mode initiation.

In this case, if the M2M device does not receive at least one paging message with action code 0b10 prior to expiration of the maximum time duration, it may request ranging and send AAI-RNG-REQ message with ‘reception of paging message indicator’ set to 1.

[Remedy2] Modify table 678, 698,699,700 in page 3 line 61.

16.2.3.1 AAI-RNG-REQ

Table 678 - AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| : |  |  |  |
| Reception of paging message indicator | 1 | 0 : Reserved 1: Expiration of maximum duration for M2M device to wait for AAI-PAG-ADV (Action Code = 0b10) |  |

16.2.3.21 AAI-DREG-REQ message

Table 698 – AAI-DREG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| : |  |  |  |
| If (De-registration_Request_Code = 0x01 { |  |  |  |
| : |  |  |  |
| Maximum duration to wait for AAI-PAG-ADV (Action Code =0b10) |  | Maximum time duration for M2M device to wait for AAI-PAG-ADV with action code 0b10 before performing ranging for UL data transmission. | Present if transmission type set to 1 |

16.2.3.22 AAI-DREG-RSP

Table 699 – AAI-DREG-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| : |  |  |  |
| Transmission Type |  | 0 : Reserved 1 : Allowed to send data only after receiving paging message with action code 0x10 | Present if needed |
| Maximum duration to wait for AAI-PAG-ADV (Action Code =0b10) |  | Maximum time duration for M2M device to wait for AAI-PAG-ADV with action code 0b10 before performing ranging for UL data transmission. | Present if transmission type set to 1 |

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Table 700 – AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| : |  |  |  |
| Action Code | 1 2 | 0b00: Perform network reentry 0b01: Receiving multicast traffic 0b10 : Notification to poll M2M device for UL data transmission 0b11 : Reserved |  |

# References

[1] IEEE P802.16m/D12

[2] IEEE C802.16p-11/017r1