| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Access Surge indication |  |
| Date Submitted | May 8, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics | {jin1.lee, youngsoo.yuk, kiseon.ryu}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose access surge indication and ranging parameters for access surge. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network access surge indication

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak

LG Electronics

# Background

Many discussions have been going on handling access congestion from power outage or catastrophic events (namely, network surge. How to determine network surge is outside scope of this contribution). Since this kind of event is hard to predict and can effect network very severely, control mechanism for surge needs to be defined.

# Proposal

- Access surge indication using AAI-RNG-ACK message

When a BS determines that network surge occurs, the BS includes ‘surge indication’ and ‘Backoff scaling factor for network surge’ in AAI-RNG-ACK message. Upon receiving the AAI-RNG-ACK with ‘network surge indicator’ and ‘Backoff scaling factor for network surge’, M2M devices shall use a ranging backoff window as a result of Backoff scaling factor for network surge * initial ranging backoff start (defined in S-SFH) so that access collision can be lessened.

Fig1. Access surge indication

# Text Proposal

[Remedy1] Insert the blue texts at the end of section 16.2.18.7.2, page7 line17:

16.2.18.7.2 Network re-entry from idle mode for M2M devices

When the M2M device receives the AAI-RNG-ACK with ‘network surge indicator’, the M2M device shall use ranging backoff window as the result of initial ranging backoff start * Bakoff scaling factor for network surge. The Backoff scaling factor is transmitted in AAI-RNG-ACK and is used when the ‘network surge indicator’ is set to 1.

[Remedy2] Insert the blue texts below in page3 line 6 :

16.2.3.3 AAI-RNG-ACK

In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. The AAI-RNG-ACK message is also transmitted to notify M2M devices of network surge with ‘network surge indicator’. In this case, the BS sends Backoff scaling factor for network surge to increase backoff window size by the scaling factor * initial ranging backoff start defined in S-SFH.

Table 680 – AAI-RNG-ACK message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| Network surge indicator | 1 | If bit is set to 1, it is to notify M2M devices of network surge situation. |  |
| Backoff scaling factor for network surge | TBD | M2M device shall use the value, Backoff scaling factor for network surge (B)* initial ranging backoff start (S) defined in S-SFH as ranging backoff window. | Present if Network surge indicator set to 1 |

# References

[1] IEEE P802.16m/D12