| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging contention resolution for M2M device |  |
| Date Submitted | May 13, 2011 |  |
| Source(s) | Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu , JinSam Kwak LG Electronics | {jin1.lee, youngsoo.yuk, kiseon.ryu}@lge.com |
| Re: | 802.16p amendment texts |  |
| Abstract | This is to propose M2M specific ranging contention resolution mechanism. |  |
| Purpose | To be discussed and adopted by 16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging contention resolution for M2M device

Jin Lee, Youngsoo Yuk, Jeongki Kim, Giwon Park, Kiseon Ryu, JinSam Kwak

LG Electronics

# Background

Since each M2M application shows regular (biased) traffic pattern and its unique characteristic of data such as delay sensitive or insensitive, it’s been considered to use priority concept for M2M devices to perform ranging for entry. For example, ranging opportunity based on priority factor [3], access restriction using access class [4]. Here, we believe priority or access class is similar to QoS of a service flow defined in 802.16m standard. From that perspective, in this contribution we propose ‘Backoff scaling factor for contention resolution’ for M2M devices to be used in ranging contention based on QoS of M2M data traffic.

# Proposal

- M2M specific contention resolution

- AAI-SCD-MSG transmits a ‘unit time value for contention resolution’

- BS can assign ‘Backoff scaling factor for contention resolution’ to M2M devices during DSx transaction.

- When ranging contention occurs, the M2M device should defer a ranging attempt until (Backoff scaling factor * unit time value for contention resolution) time duration. This is for M2M device to give differentiated waiting time at next stage of ranging attempt based on its QoS of a service flow. For example, a delay sensitive M2M application will have shorter waiting time than a delay tolerable M2M application.

- M2M devices consider ranging contention when it receives neither AAI-RNG-ACK nor UL resource allocation until T31 timer expires.

- However, backoff window size would be exponentially increased as the same with conventional contention resolution

# Text Proposal

[Remedy1: Insert the following texts at the end of section 16.2.18.7.2]

The method of contention resolution is based on QoS of a service flow of an M2M device, with the unit time value for contention resolution and the scaling factor specified in AAI-DSx message.

If the M2M device receives neither AAI-RNG-ACK nor UL resource allocation until T31 timer expires, the M2M device considers its ranging attempt is failed and restarts ranging procedure as defined in 16.2.15.3 with the exception as following:

When the M2M device restarts ranging procedure, it shall not re-attempt ranging within the time duration which is calculated by the unit time duration for contention resolution* Backoff scaling factor for contention resolution. The unit time duration is transmitted in AAI-SCD-MSG and Backoff scaling factor is assigned during DSx transaction.

[Remedy2] Modify the following text change in table 708:

Table 708 – AAI-SCD Message Field Description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| Unit time duration for M2M contention resolution |  | Used for ranging contention resolution for M2M devices. See 16.2.18.7.2 |  |

[Remedy3] Insert the following blue text change at the end of table 734, 735,737, 738:

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| : |  |  |  |
| Backoff scaling factor for M2M contention resolution |  | Used for ranging contention resolution. See 16.2.18.7.2 |  |

# References

[1] IEEE P802.16p/18r1

[2] IEEE p802.16m/D12

[3] C802.16p-11_0039

[4] C802.16p-11_0009