| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Evaluation Methodology for 802.16p |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Rui Huang, Honggang Li, Belal Hamzeh, Yang-Seok Choi, Shantidev Mohanty Intel Corporation Chun-Yen Hsu III | E-mail: rui.huang@intel.com hcy@iii.org.tw |
| Re: | IEEE 802.16p call for EMD |  |
| Abstract | This contribution proposes the evaluation methodology for the 16p system based on 16m EMD architecture. |  |
| Purpose | To be discussed and adopted as an separate document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Evaluation Methodology for 802.16p

Rui Huang, Honggang Li, Belal Hamzeh, Yang-Seok Choi, Shantidev Mohanty

Intel Corporation

Chun-Yen Hsu

Institute for Information Industry (III)

# Introduction

In the last meeting [1] [2] the need for evaluation methodology for 16p was agreed since the specific nature and requirements of machine type communications. Therefore, in this contribution we propose the evaluation methodology for the 16p system.

# Proposed Text

----------------- Start of the text proposal ----------------------------------------------------------------------------------

[Propose #1:Adopt the following text as the stand-alone 16p EMD document:]

# Introduction

The objective of this evaluation methodology is to define link-level and system-level simulation models and associated parameters that shall be used in the evaluation and comparison of technology proposals for IEEE 802.16p. Proponents of any technology proposal using this methodology shall follow the evaluation methods defined in this document and report the results using the metrics defined in this document. The current text in this documents for downlink and uplink simulation assumptions are templates that may be extended for a complete description.

# Test Scenarios

The typical testing seneario for 16p is given in Fig2-1 in which only single cell modeling is considered.

Figure15-1 The typcial test scenario for 16p

# Traffic Models

This section describes traffic models in detail. A major objective of system simulations is to provide an operator with a view of the maximum number of active users that can be supported for a given service under a specified configuration at a given coverage level.

- In-home M2M Devices Deployment

| Case | Cell Radius(m) | Number of device within a home | Number of homes within a cell |
| --- | --- | --- | --- |
| Urban | 1500 | 10 | 18116 |
| Sub-urban | 3000 | 10 | 20912 |

- Commercial Usage Devices

| Case | Cell Radius(m) | Number of devices within a cell |
| --- | --- | --- |
| Urban | 1500 | 13583 |
| Sub-urban | 3000 | 7092 |

The detail parameters for these deployment models are give in Appendix Q.

# Test Assumptions and Parameters

The purpose of this section is to outline simulation assumptions that proponents will need to provide in order to facilitate independent assessment of their proposals.

| Deployment/ parameters | In-Home and City Commercial M2M Mixed deployment |
| --- | --- |
| Carrier Frequency | 2.5GHz |
| Operating Bandwidth | 10 MHz for TDD |
| BS Site-to-site distance | 1.5km (Urban) 3.0km (Suburban) |
| MS mobility | 0-120 km/h |
| Number of transmit antennas of BS | 2 (Mandatory) 4 (Optional) |
| Number of receive antennas of MS | 1 (Mandatory) 2 (Optional) |
| M2M Tx power | TBD |
| BS Tx power per sector | 46dBm |
| Base station antenna height | 12.5m |
| Number of BS transmit antennas per sector | 2 (Mandatory) 4 (Optional) |
| Number of BS receive antennas per sector | 2 (Mandatory) 4 (Optional) |
| Number of sectors | 3 |
| Antenna front-to-back power ratio | 30 dB (Mandatory) 20 dB (Optional) |
| Antenna spacing | 4λ (Mandatory) 0.5λ (Optional) |
| Noise figure |  |
| Cable loss |  |
| Number of M2M transmit antennas | 1 |
| Number of M2M receive antennas | 1 |
| M2M Antenna type | Omni in horizontal plane |
| Antenna gain (boresight) | 20 dBi |
| Antenna 3-dB beamwidth | 20 |
| Antenna front-to-back power ratio | 23 dB |
| Antenna spacing | N/A |
| Antenna orientation | Antenna array broadside pointed to BS direction |
| Noise figure | 5 dB |
| Cable loss | 2 dB |

Table 3-1 Basic Parameters for 16p

# Channel Models

This section describes the channel models used to model propagation conditions between BS and MS/M2M device.

# Channel Models with High Mobility

# Path Loss model

# Shadowing Factor

# Shadowing Factor

# Cluster-Delay-Line Models

# Channel Type and Velocity Mix

# Doppler Spectrum for Stationary Users

# Generation of Spatial Channels

# Channel Models with Low Mobility

Current 802.16m EMD considers only mobile devices, we recommend the inclusion of channel models specific to devices with low mobility, such as the SUI-3 and SUI-4 channel models.

# Path Loss model

# Shadowing Factor

# Shadowing Factor

# Cluster-Delay-Line Models

# Channel Type and Velocity Mix

# Doppler Spectrum for Stationary Users

# Generation of Spatial Channels

# Scheduling

The generic proportional fair scheduler specified in [3] is used for allocation of the resources. Furthmore,in order to support large volume of M2M devices with low power transmission, group-based resource scheduling should be considered, e.g. the ABS can allocate the specific channel resource to a group of M2M devices to transmit and receive packets.

# Power Management

The implementation of an idle state is proposed to be used in the IEEE 802.16m and 16p broadband wireless system to conserve battery power of mobile devices when a call session is not active. A mobile device returns to active state whenever required, e.g., when there is incoming data for the said device. IDLE to ACTIVE_STATE transition latency is a key metric to evaluate and compare various proposals related to IDLE to ACTIVE_STATE transition schemes as this latency has direct impact on application performance experienced by a user.

# Performance Metrics

M2M system evaluation need the some specific performance metrics regarding to their special requirments, e.g. extra low power consumption, and large number of devices access.

# Power Consumption Metric

The power consumption metric can help the designer of the standard to evaluate if a specific design is power-efficient and able to meet the requirement of M2M as a low power device and solution.

The metrics related to averaged power consumption can be defined in the following ways,

  - Averaged power consumption per user

  - Averaged power consumption at 5% percentile of coverage

  - The CDF of averaged power consumption per user

The averaged power consumption can be defined as

Where k=1…K is the index of user, j=1… is the index of packet transmitted for the kth user, is the transmit power for UL or  the receiving power consumption for DL for the jth packet and kth user, is the time consuming for the jth packet and kth user.

For simplicity, only the UL can be considered, because it is related to the transmit power of M2M device.

# Ranging Access Performance Metrics

The following metrics should be considered the analsis of ranging access performance for M2M[8].

- Collision Probability: defined as the ratio between the number of occurrences when two or more devices send a intial access attempt using exactly the same ranging code and the overall number of opportunities (with or without access attempts) in the period

- Access Success Probability: defined as the probability to successfully complete the random access procedure within the maximum number of ranging code transmissions.

- Control overhead per successful ranging: defined as the ratio between the overhead of the ranging control messages for M2M only,e.g. the dedicate ranging resource allocation, to the total ranging access payload.

### Appendix Q.M2M Devices Deployment Model

### Q-1. In-Home M2M Devices Deployment

Table Q-1 below shows In-Home M2M device tranffic parameters[4].

Table Q-1 In-Home M2M Devices Traffic Parameters

| Appliances/ Devices | Average Message Transaction Rate/s | Average Message Size (Bytes) | Date Rate (b/s) | Number Per Home | Distribution |
| --- | --- | --- | --- | --- | --- |
| Home Security System | 1/600 | 20 | 0.2667 | 1 | Poisson, |
| Elderly Sensor Devices | 1/60 | 128 | 17.0667 | 0.1 | Possion/Uniform |
| Refrigerator | 1/3600 | 30 | 0.0667 | 1 | Possion/Uniform |
| Clothes Washer | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Clothes Dryer | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Dishwasher | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Freezer | 1/24*60*60 | 30 | 0.0028 | 1 | Possion/Uniform |
| Stoves/Ovens | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Microwaves | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Coffee Makers | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| Toaster Ovens | 1/24*60*60 | 8 | 7.407e-04 | 1 | Possion/Uniform |
| PHEV to SM | 2.4e-4 | 97.6 | 0.0234 | 2 | Possion/Uniform |
| Smart Meter | 1.1e-4 | 2017 | 1.775 | 3 | Possion/Uniform |

Table Q-2 shows the average number of homes within a cell in the different scenarios[6].

Table Q-2 Average Home numbers in a cell

| Scenario | Max Cell Radius(m) | Min Cell Radius(m) | *Average number of homes within cell (Max) | *Average number of home within cell (Min) |
| --- | --- | --- | --- | --- |
| Urban (New York City) | 1000 | 500 | 12077 | 3021 |
| Sub-urban (Washington D.C.) | 1500 | 1000 | 10456 | 4647 |

### Q-2. City Commercial M2M Devices Deployment

In table Q-3 shows city commerial M2M devices traffic parameters[5][8].

Table Q-3 City Commerical M2M Devices Traffic Parameters

| Appliances/ Devices | Average Message Transaction Rate/s | Average Message Size (Bytes) | Date Rate (b/s) | Distribution and arrival |
| --- | --- | --- | --- | --- |
| Credit Machine in grocery | 0.0083 | 24 | 0.2667 | Poisson, |
| Credit Machine in shop | 5.5556e-4 | 24 | 0.0178 | Poisson |
| Roadway Signs | 0.0333 | 1 | 0.2664 | uniform |
| Traffic Lights | 0.0167 | 1 | 1.3360 | uniform |
| Traffic Sensors | 0.0167 | 1 | 1.3360 | Poisson |
| Movie Rental Machines | 1.1574e-5 | 152 | 1.4814e-3 | Poisson |

In table Q-4 shows the city commerial facities deployment[6].

Table Q-4 City Commerial Facities deployment

| Scenario | Number of grocery stores/ meter2 | Number of shops and restaurant/ meter2 | Number of roadway signs/ meter2 | Number of traffic lights/ meter2 | Number of traffic sensors/ meter2 | Number of Movie Rental Machines/ meter2 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban (New York City) | 2.0947e-4 | 0.0022 | 3.1647e-4 | 1.503e-5 | 1.503e-5 | 6.9823e-5 |
| Sub-urban (Washington D.C.) | 2.3122e-5 | 3.4988e-4 | 9.4325e-4 | 1.1442e-4 | 1.1442e-4 | 1.1561e-5 |

----------------- End of the text proposal -------------------------------------------------------------------------------

# Reference:

[1] C80216p-11//0030

[2] C80216p-11//0006r1802.16p Closing Report for Session #72

[3] C80216m-08/004r5 IEEE 802.16m Evaluation Methodology Document (EMD)

[4]http://www.zpryme.com/SmartGridInsights/2010_Smart_Appliance_Report_Zpryme_Smart_Grid_Insights.pdf

[5] Credit card security electronic transaction protocol(SET)

[6] Google map statiscs

[7] R2-105212, MTC simulation assumptions for RACH performance evaluation, Huawei, ZTE, Institute for Information Industry (III), MediaTek

[8] http://www.nyc.gov/html/dot/html/faqs/faqs_signals.shtml