| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Group ID Addressing Scheme in IEEE802.16e System |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group ID(MGID) assignment scheme based on the 802.16e in order to minimize the modification to the standard. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group ID Addressing Scheme in IEEE802.16e System

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1], M2M group identifier (MGID) for IEEE 802.16p system is defined which used to send control messages to a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices. This feature could support a large number of M2M devices. However, several issues for MGID are still open, e.g. MGID assignment procedure and its typical usages.

In this contribution, we propose the MGID assignment scheme based on the 802.16e[2] in order to minimize the modification to the standard. The scheme based on 802.16m[3] is presented in [4] also.

# MGID Addressing Scheme in 16e

In the figure 1, the MGID address scheme including the MGID establishment and some typical usages is shown.

Figure 1. MGID assignment and typical usages

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Revised the following sections in IEEE802.16e-2009

11.13 Service flow management encodings

…

Table 606—Service flow encodings

| Type | Parameter |
| --- | --- |
| 1 | SFID |
| 2 | CID |
| 3 | Service Class Name |
| 4 | MBS |
| 5 | QoS Parameter Set Type |
| 6 | Traffic Priority |
| 7 | Maximum Sustained Traffic Rate |
| 8 | Maximum Traffic Burst |
| 9 | Minimum Reserved Traffic Rate |
| 10 | CID indicator |
| 11 | Uplink Grant Scheduling Type |
| 12 | Request/Transmission Policy |
| … |  |
| 32 | Paging Preference |
| 33 | MBS zone identifier assignment |
| 34 | Reserved |
| 35 | Global Service Class Name |
| 36 | Reserved |
| 37 | SN Feedback Enabled |

---------------------------End Text Proposal----------------------------------

# References

- IEEE 802.16p-11/0001r3

- IEEE 802.16e2009

- IEEE 802.16mD12

- C80216p-11/0104