| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Group ID Addressing Scheme in IEEE802.16m System |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | his contribution proposes the M2M group ID(MGID) assignment scheme based on the 802.16m in order to minimize the modification to the standard. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group ID Addressing Scheme in IEEE802.16m System

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1], M2M group identifier (MGID) for IEEE 802.16p system is defined which used to send control messages to a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices. This feature could support a large number of M2M devices. However, several issues for MGID are still open, e.g. MGID assignment procedure and its typical usages.

In this contribution, we propose the MGID assignment scheme based on 802.16m[2] standard in order to minimize the standard modification. The other scheme based on 802.16e[3] is presented in [4] also.

# MGID Addressing Scheme in 16m

In the figure 1, the MGID address scheme including the MGID establishment and some typical usage is shown.

- M2M devices initiated process

- BS initiated process

Figure 1. MGID assignment and typical usages

## MGID Establishment Scheme in 802.16m

In order to reduce the standard modification because of the new features, we propose to use the current service flow management message, e.g. AAI-DSA-REQ, AAI- DSA-RSP, to establish MGID with some necessary modifications which will not impact the legacy non-M2M services.

In 16m, the service flow connection could be initiated by either AMS or ABS. Therefore, MGID establishment process should consider the following situations depending on the connection initiation entity.

- M2M Devices Initiated

- Step0. When M2M device registers to the network, it will report to BS its “M2M capability” which will be applied to not only the MGID establishment but also other optimized processes for 16p.

- Step1. After M2M device’s entry into the network, it will notify its traffic characteristics, e.g. the traffic interval, traffic packet size, to the BS by “SDU-Size” and “SDU Inter-arrival Interval ” in AAI-DSA-REQ message as the grouping criteria.

- Step2. BS will aggregate the M2M devices with similar traffic pattern together as a M2M group. Subsequently BS will assign a same Multicast Group ID to these M2M devices in the same M2M group in AAI-DSA-RSP message. EMBS Service field in AAI-DSA-RSP message will be set to “111” to indicate this Multicast group ID is for M2M grouping indeed.

- BS Initiated

- Step0. When M2M device registers to the network, it will report to BS its “M2M capability” which will be applied to not only the MGID establishment but also other optimized processes for 16p.

- Step1. BS and upper layer will predefine some M2M groups.

- Step2. BS will assign a same Multicast Group ID to these M2M devices in the same M2M group in AAI-DSA-REQ . EMBS Service field in AAI-DSA-REQ message will be set to “111” to indicate this Multicast group ID is for M2M grouping indeed.

- Step3. M2M devices will feedback AAI-DSA- RSP to ABS as the confirmation.

## Typical MGID Usages in 16m

The typical group-based control and data transmission process with MGID can be:

- DL multicast data for the M2M group

- Group based control signaling for the M2M group, e.g. the group-oriented MCS, power control and resource allocation mechanism

- Group paging for the M2M group

More specific definitions for these MGID usages are FFS.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Revised the following text in Section16.2.1.3.1  in the latest 16p AWD

16.2.1.3.1 M2M Group Identifier (MGID)

A TBD-bit value that uniquely identifies an M2M group which one or more M2M devices belong to. This ID shall be used to send control messages to a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices.

An MGID is assigned by  the service flow establishment process as well as the multicast group does and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD.

Proposal #2 : Add the following text in Section16.2. in the latest 16p AWD.

16.2.3.11 AAI-REG-REQ

Table 685 —AAI-RNG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| M2M capability | 1 | 0b0: not M2M device 0b1: M2M device |  |

16.2.12.12 Service Flow/Convergence Sublayer Parameters

Table 782.—Service flow/convergence sublayer parameters

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |

Table 782a E-MBS Service for M2M Configuration

|  | Bit#0 | Bit#1 | Bit#2 | Notes |
| --- | --- | --- | --- | --- |
| Set0 | 0 | 0 | 0 | Not support EMBS and M2M grouping |
| Set1 | 1 | 0 | 0 | Support Single ABS E-MBS only |
| Set2 | 0 | 1 | 0 | Support multiple ABS E-MBS w/o macro diversity |
| Set3 | 0 | 0 | 1 | Support multiple ABS E-MBS with macro diversity |
| Set4 | 1 | 1 | 1 | Support M2M grouping. The multicast group ID in the legacy spec used as M2M group ID under this case. |

16.2.3.47.1 AAI-DSA-REQ

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |

16.2.3.47.2 AAI-DSA-RSP

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0001r3

- IEEE 802.16mD12

- IEEE 802.16e2009

- IEEE C80216p-11/0103