| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging Parameters for Voluntarily M2M Access |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Youngsoo Yuk, Jin Lee, Jeongki Kim, Giwon Park, Kiseon Ryu and JinSam Kwak LG Electronics | Email: youngsoo.yuk@lge.com, jin1.lee@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution provides the simulation results on the performance of the method for distributing ranging access over multiple ranging channels. |  |
| Purpose | For discussion in 802.16p TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging Parameters for Voluntarily M2M Access

Youngsoo Yuk, Jin Lee, Jeongki Kim, Giwon Park, Kiseon Ryu and JinSam Kwak

LG Electronics Inc.

# Introduction

Allocating ranging channel is a key issue of M2M access control. In order to control the higher UL access load, Allocating additional ranging opportunities are agreed in the last session. However the size of the ranging channel is very large comparing to other control channels, we should increase the utilization of each ranging opportunity.

In the previous sessions, the ranging optimizations discussed were focused on the special ranging trial cases (e.g. group paging, power outage etc). However, most of the transmission will be done voluntarily and the number of ranging access of MS-initiated should be optimized. In this contribution, we proposed two ranging optimization operation by introducing new ranging parameters for MS-initiated ranging access.

First we propose a ranging distribution method to increase the utilization of the ranging opportunities. In addition, ranging waiting parameter is also proposed to decrease the number of ranging trials.

# Ranging Parameters for M2M services

## Temporal Ranging Distribution Parameter

In order to utilize the ranging resource efficiently, we should consider the following points for allocating ranging resources.

- The average number of the ranging trials (Nav)

  - The capacity of ranging channel is related to the average number of trials when assuming large number of devices.

  - From the simulation results, in order to achieve the contention rate less than 1%, Nav is recommended to be less than 0.4 trials /ranging channel.

- The variance of number of trials (Vn)

  - The variance of Nav is another parameter to be considered. The smaller Vn is, the lower contention rate we can achieve.

  - Vn can be reduced as the number of MSs transmitting ranging channel increase.

  - For example, The case of lambda of 0.005 with 4500 MSs is better than case of 0.05 with 450 MSs)

  - Assuming random trials, in order to provide enough performance (less than 1%), 2/3 of all allocated ranging resources may be remained empty with random access.

Since the size of ranging resource is too large (high overhead, 4 LRUs for max 2~3 users), after forecasting ranging traffic model, appropriate number of ranging resources should be allocated.

In order to minimize the variance of the number of ranging access per ranging channel, we propose a temporal distribution method.

The ranging frame for an M2M device is determined by the following rule.

Ranging frame mod K == M2M Device ID mod K,

where K is the ranging distribution parameter and signaled on the paging message or signaled as a QoS parameter during the service negotiation.

By doing this, the variance of the ranging trials can be reduced. From the simulation results, value K can be determined by the function of the transmission rate.

## Ranging Waiting Parameter

One other way to reduce the number of ranging access is to aggregate several UL transmissions.

In case of time-tolerable M2M services, in order to suppress the frequent ranging access, we can introduce one QoS parameter, “ranging waiting timer”. This parameter is determined by considering latency margin of the service.

If a service has long latency margin, after data being received from higher layer, MS shall wait perform ranging until ranging waiting timer is expired.

Figure 1 shows the concept of the proposed method. The multiple UL access within single Ranging waiting timer shall be transmitted with one ranging access, the total number of UL ranging from single device can be reduced.

However ranging waiting timer is not expired but the L2 buffer (ARQ or Non ARQ buffer) is possible to be overflow or any urgent traffic is generated, ranging may be allowed.

Figure 1. Concept of ranging waiting timer

# Simulation Results

Table 1. Simulation Assumption

| Parameters | Values | Notes |
| --- | --- | --- |
| Number of Groups | 4 Groups |  |
| Number of MSs in a Group | 100, 200, 300 |  |
| Traffic Model | Poisson Distribution with lambda of 0.1, 0.075, 0.05, 0.025 events/MS/sec | Various configuration |
| Period of ranging channel | Every frame | Indexed by (0,1,2,3) in a superframe |
| Number of Max codes can be received in a ranging channel | 3 | More than 4 codes is considered as contention |
| Number of codes (index) | 32 | It is related to collision performance (Not critical) |

| Group ID | Lambda | Random Trial | Temporal Distribution (100 MSs per Group) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Contention Ratio (%) | K | Contention Ratio | K | Contention Ratio | K | Contention Ratio |
| 1 | 0.1 | 0.032 | 1 | 0.040 | 3 | 0.032 | 15 | 0.025 |
| 2 | 0. 075 | 0.074 | 1 | 0.032 | 4 | 0.032 | 20 | 0.026 |
| 3 | 0.05 | 0.104 | 2 | 0.032 | 6 | 0.032 | 30 | 0.026 |
| 4 | 0.025 | 0.048 | 4 | 0 | 12 | 0.032 | 60 | 0.025 |
| Total |  | 0.061 |  | 0.032 |  | 0.032 |  | 0.026 |

| Group ID | Lambda | Random Trial | Temporal Distribution (200 MSs per Group) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Contention Ratio (%) | K | Contention Ratio | K | Contention Ratio | K | Contention Ratio |
| 1 | 0.1 | 0.23 | 3 | 0.19 | 15 | 0.23 | 50 | 0.13 |
| 2 | 0. 075 | 0.25 | 4 | 0.23 | 20 | 0.19 | 67 | 0.10 |
| 3 | 0.05 | 0.22 | 6 | 0.24 | 30 | 0.15 | 100 | 0.11 |
| 4 | 0.025 | 0.32 | 12 | 0.22 | 60 | 0.20 | 200 | 0.11 |
| Total |  | 0.24 |  | 0.21 |  | 0.20 |  | 0.11 |

| Group ID | Lambda | Random Trial | Temporal Distribution (300 MSs per Group) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Contention Ratio (%) | K | Contention Ratio | K | Contention Ratio | K | Contention Ratio |
| 1 | 0.1 | 0.63 | 3 | 0.59 | 15 | 0.59 | 50 | 0.41 |
| 2 | 0. 075 | 0.68 | 4 | 0.62 | 20 | 0.56 | 67 | 0.43 |
| 3 | 0.05 | 0.63 | 6 | 0.64 | 30 | 0.63 | 100 | 0.43 |
| 4 | 0.025 | 0.72 | 12 | 0.65 | 60 | 0.62 | 200 | 0.47 |
| Total |  | 0.66 |  | 0.62 |  | 0.59 |  | 0.43 |

# Proposed Text Change

Adopt the following Remedies in IEEE 802.16p-10/0018.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1: : In line 12 of page 7 in C802.16p-10/0018r1, insert the description as following]

16.2.18.7 Idle mode for M2M application

16.2.18.7.2 Network re-entry from idle mode for M2M devices

When the ranging distribution parameter (K) is assigned on AAI_DSx message during service negotiation and an M2M device starts ranging procedure without paging notification, the M2M device shall not transmit ranging preamble in the frame except the ranging opportunity with its ranging opportunity count determined by the following equation.

Ranging opportunity count mod K == M2M Device ID mod K,   (x)

Ranging opportunity count is the index of the ranging opportunity, and the ranging opportunity count starts with “0’ for the first ranging opportunity at the frame that the Superframe Number in SFH is ‘0’, and it increase by 1 for the next allocations.

A ranging waiting timer may be assigned for a flow during service negotiation. The ranging waiting timer for the flow shall be started when the MAC SDU of the flow is arrived from the higher layer and M2M device is in idle mode. The M2M device shall wait performing ranging for transmitting the data burst for the flow until the ranging waiting timer is expired. Single ranging waiting timer is defined for a flow, and it shall not be reset even though new MAC SDU of the flow is arrived. It shall be terminated only if any ranging operation is initiated by the expiration of the ranging waiting timer of the other flow or by receiving paging notification from the BS.

[Remedy 2 : In line 22 of page 6 in C802.16p-10/0018r1, insert the description as following]

16.2.12.12 Service Flow/Convergence Sublayer Parameters

Table 782 defines the parameters associated with UL/DL scheduling for a service flow.

Table 782——Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| Ranging Distribution Parameters | 8 | The ranging frame of the AMS is restricted by the this parameter. Refer 16.2.18.7 |
| Ranging Waiting Timer | 24 | frames, AMS shall wait performing ranging access to transmit the data of this flow until this timer is expired. |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------