| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods for Dedicated Ranging Resource Allocation for M2M in 16e |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Shantidev Mohanty Honggang Li Rui Huang Xiangying Yang Joey Chou | E-mail: { shantidev.mohanty}@intel.com |
| Re: | IEEE 802.16p-10/0018r1 |  |
| Abstract | This contribution proposes methods to assign dedicated ranging resources to M2M devices. |  |
| Purpose | To be discussed and adopted into 16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods for Dedicated Ranging Resource Allocation for M2M in 16e

Shantidev Mohanty, Honggang Li, Rui Huang, Xiangying Yang, Joey Chou

Intel Corporation

# Introduction

16p AWD specifies that "BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources."

This contribution proposes changes to the 16e standard to allocate dedicated ranging resources to M2M devices performing network re-entry from idle mode.

# Proposed Text

==================Start of proposed text=====================================

[Add the following text after line 13 in page 2 in IEEE P802.16p AWD.]

The BS assigns dedicated ranging resources to M2M devices performing network re-entry from idle mode in UL-MAP message.

==================End of proposed text====================================