| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Access Control Messages for Large Numbers of M2M Devices During the Network Reentry |  |
| Date Submitted | 2011-05-16 |  |
| Source(s) | Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai ITRI | E-mail: YCFang@itri.org.tw zhengyanxiu@itri.org.tw |
| Re: | Call for Comments on the IEEE 802.16p AWD |  |
| Abstract | The contribution provides the proposed text of the access control messages for large numbers of M2M devices during the network reentry to be included in the IEEE 802.16p amendment working document. |  |
| Purpose | To be discussed and adopted in the IEEE 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Access Control Messages for Large Numbers of M2M Devices During the Network Reentry

Yu-Chuan Fang, Zheng Yan-Xiu, Chang-Lan Tsai

ITRI

# Introduction

Large numbers of M2M devices may access network at the same time. In order to avoid congestion and collision, we propose to utilize the multicast/paging operation to regulate the large numbers of M2M devices based on the M2M group. The 16p BS may transmit the control messages to regulate the group of M2M devices by using the multicast/paging operation. The control messages may indicate the specific backoff window size, ranging opportunities or other actions for the specific M2M group.

For M2M devices in the idle mode, the paging message may be used to send a group of M2M devices a transient backoff values to relieve the crowed traffic. Especially for the M2M devices with lower priority, the 16p BS should give them the larger backoff values. M2M group ID should be included in the control message. Furthermore, the 16p BS may stop some specific M2M group to perform network reentry by using the parameter, Group Bar. Timer is also required to indicate the duration of using the temporary configuration. While the timer expires or the paging message does not include any access control messages for the M2M group, the M2M devices shall perform network reentry according to the ranging configuration defined in the S-SFH IEs.

This contribution provides the proposed text within the appropriate sections for the IEEE 802.16p amendment working document.

# Proposed Text for IEEE 802.16p AWD

# ============= Start of Proposed Text 1 ============================================

16.2.18.7.2  Network re-entry from idle mode for M2M device

For network reentry from Idle Mode, ranging parameters may be different for M2M devices or M2M groups.

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in Table 833 in 16.3.5.5.1.2.

For the network reentry indicated by a paging message that contains ranging configuration, the M2M device shall select a ranging opportunity according to the ranging configuration. Ranging configuration may include differentiated waiting offset time and backoff window size.

For the network reentry indicated by a paging message that contains access control messages ranging configuration, the M2M device shall select a ranging opportunity according to the new ranging configuration. If the ranging configuration assigned by the message has different parameter value from that assigned by the S-SFH IE, the M2M devices shall utilize the new parameter values. A parameter, Timer, may be used to indicate the duration of using the new configuration. While Timer expires, the M2M devices shall return to follow the ranging configuration defined in the S-SFH IE. If the bit of Group Bar is set to 1 from a preferred ABS, the M2M group shall stop performing network reentry until Timer expires. Ranging configuration may include differentiated waiting offset time and backoff window size.

# ============= End of Proposed Text 1 ============================================

# ============= Start of Proposed Text 2 ============================================

16.2.3.23  AAI-PAG-ADV (paging advertisement) Message

Table 700—AAI-PAG-ADV message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| Num_M2MGID | TBD | Num_M2MGID is the number of M2M group IDs which should receive the control message. | Present if there is any M2M group the ABS wants to send message. |
| For (i=0; i<Num_M2MGID; i++) { |  |  |  |
| M2MGID | TBD | M2M Group ID | N.A. |
| Timer | TBD | To indicate the duration of using the temporary configuration | N.A. |
| Group Bar | 1 | 0: The M2M group may perform network reentry through the ABS 1: The M2M group shall stop performing network reentry | Optional |
| Temporary initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 | Optional |
| Temporary initial ranging backoff end | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 | Optional |
| } |  |  |  |

# ============= End of Proposed Text 2 ============================================

# Reference

[1]  802.16p-10_0018r1, IEEE 802.16p AWD

[2]  IEEE P802.16m/D12, Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks.