| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicast operation for M2M application in 802.16-2009 |  |
| Date Submitted | 2011-05-18 |  |
| Source(s) | Jeongki Kim, Jin Lee, Giwon Park, Youngsoo Yuk, Kiseon Ryu, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for contributions for DEV RG or 802.16p WG |  |
| Abstract | This contribution proposes the multicast operation in idle mode in 802.16-2009 |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multicast operation for M2M application in 802.16-2009

Jeongki Kim, Jin Lee, Giwon Park, Youngsoo Yuk, Kiseon Ryu, Jinsam Kwak

LG Electronics

# Introduction

During the last meeting the multicast operation for M2M application was defined based on the WirelessMAN-Advanced Air Interface. This contribution proposes texts about M2M multicast operation in WirelessMAN-OFDMA R1 Reference System.

# Text proposal

Note:

Blue underlined texts: Texts added.

Red strikeout texts: Texts deleted

---- Start of proposed text ----

[Remedy #1: Add following sections and texts in IEEE 802.16p AWD:]

6.3 Data/Control plane

6.3.1 Addressing and connections

[Insert the following texts at the end of the Section6.3.1]

M2M group ID (MGID) uniquely identifies an M2M group in the domain of the network entity that assigns MGID, which one or more M2M devices belong to. This ID shall be used to identify a group of devices (e.g., group paging).

An MGID is assigned to an M2M device during initial network entry and released during an explicit network exit (e.g., power down location update). The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned. During connected mode, the MGID may be added and changed by DSA and DSC procedure respectively.

[Remedy #2: Add following sections and texts in IEEE 802.16p AWD:]

[Insert the following section after the Section 6.3.27]

6.3.28 Support of Multicast Operation for machine to machine application

6.2.28.1 M2M Multicast operation in idle mode

A BS may provide a multicast service for M2M devices in idle mode with or without requiring network reentry of the M2M devices. Before a BS sends DL multicast data, the BS may transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The Multicast transmission start time TLV may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of Multicast transmission start time TLV shall be less than the start time of the next paging listening interval of the devices receiving the MOB_PAG-ADV message. The M2M device may power down until the frame indicated by the Multicast transmission start time TLV in the MOB_PAG-ADV message.

[Remedy #3: Add following sections and texts in IEEE 802.16p AWD:]

11.17 MOB_PAG-ADV management message encodings

[Insert the following section after the Section 11.17.2]

11.17.3 M2M group paging parameter

The following M2M group paging parameter TLV may be included in MOB_PAG-ADV message.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| M2M_Group_Paging parameters | x | Variable | Compound TLV to be used in M2M group paging operation | MOB_PAG-ADV |

The following TLV element shall appear in each M2M_Group_Paging_Parameters TLV.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| MGID | x.1 | 2 | Bit #0~ Bit#14: Indicates M2M Group ID; Bit #15: Padding, Will be set to 0 |
| Action code | x.2 | 1 | Bit #0~Bit#1: Indicates Action code for the M2M Group ID 0b00 - Performing network reentry 0b01- Performing location update 0b10 – Receiving multicast traffic 0b11- Reserved Bit #2~Bit#7: Padding, Will be set to 0 |

The following TLV element may appear in each M2M_Group_Paging_Parameters TLV.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Multicast transmission start time (MTST) | x.2 | 1 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data |

---- End of proposed text ----