| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of some texts in 802.16p AWD |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Jeongki Kim, Jin Lee, Giwon Park, Youngsoo Yuk, Kiseon Ryu, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes the texts to clean up some texts in 802.16p AWD |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of some texts in 802.16p AWD

Jeongki Kim, Jin Lee, Giwon Park, Youngsoo Yuk, Kiseon Ryu, Jinsam Kwak

LG Electronics

# Introduction

This contribution proposes some texts to clean up M2M multicast operation in 802.16m.

- STID is originally used to identify AMS in a cell in 802.16m. Clean up the related text.

- Add the informative texts into the Section 16.2.2.2.10 MTTEH.

- Define the length of MTST field in AAI-PAG-ADV

- Correct typos

- Add the A-MAP IE for multicast assignment for M2M application

# Text proposal

Note:

Blue underlined texts: Texts added.

Red strikeout texts: Texts deleted

---- Start of proposed text ----

[Remedy #1: Modify the Section 16.2.1.2.1 as follows:]

16.2.1.2.1 Station Identifier (STID)

Add the following texts at the end of this section

The STID is also used to identify the M2M devices in the domain of the ABS.

[Remedy #2: Modify the Section 16.2.2.10 as follows:]

16.2.2.2.10 Multicast transmission end extended header (MTEEH)

BS includes the MTEEH in the last MPDU of multicast data for an MGID when the transmission of multicast data for an MGID ends. If the MS in idle mode receives the MTE extended header during receiving the multicast data, the MS stops receiving the multicast data and may enter the paging unavailable interval.

Table 676a—MTEEH format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| METEEH () { |  |  |
| Type | 4 | Extended header type = 0b1001 (MTEEH Type) |
| } |  |  |

[Remedy #3: Modify the Section 16.2.3.23 as follows:]

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

Modify Table 700 as indicated

Table 700—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | … | … |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicate the number of MGIDs included in this paging message [0…63] | Shall be included if ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message |
| MGID | 15 | M2M Group ID |  |
| Action code | 1 | 0: Performing the network reentry 1: Receiving the multicast traffic |  |
| If (Action code == 1) { |  |  |  |
| Multicast transmission start time (MTST) | TBD 8 | Least Significant TBD 8 bits of the frame nuimber in which the ABS starts sending the DL multicast data. | Shall be present when the MTST need be included in this message |
| } |  |  |  |
| … | … | … | … |
| } |  |  |  |

[Remedy #4: Add the following sections and texts in C802.16p-10/0018r1 as follows.]

16.3.5.5.2.4 Assignment A-MAP IE

[Modify the table 845 and texts as follows:]

Table 845.—Description of the Masking Code for type indicator 001

| Decimal Value | Description |
| --- | --- |
| 4094 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment for M2M application (i.e., Function Index = 0b11) |
| 4095 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment (i.e., Function Index = 0b10) |
| Others | Reserved |

[Remedy #5: Add the following sections and texts in C802.16p-10/0018r1 as follows.]

16.3 Physical Layer

16.3.5 Downlink Control Structure

16.3.5.5 DL Control Information Elements

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

[Modify the table 860 and texts as indicated]

Table 860 Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_AMAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reserved This IE carries multicast assignment information for M2M application |
| … | … | … |
| } else if {Function Index == 0b10} { |  |  |
| … | … | … |
| } |  |  |
| Else {//Function Index == 0b11 |  |  |
| MGID | 15 |  |
| Burst Size | 6 |  |
| Resource Index | 11 |  |
| Long_TTI_Indicator | 1 |  |
| Reserved | 1 |  |
| } |  |  |

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 843. If Function index == 0b00 or 0b01, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b001. If Function index == 0b10 or 0b11, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b010.

---- End of proposed text ----