| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reporting channel status for fixed M2M device |  |
| Date Submitted | 2011-05-08 |  |
| Source(s) | Jeongki Kim, Woobong Lee, Youngsoo Yuk, Jinsoo Choi, Kiseon Ryu, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes texts about UL MIMO feedback for fixed M2M devices |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reporting channel status for M2M device

Jeongki Kim, Woobong Lee, Jinsoo Choi, Youngsoo Yuk, Kiseon Ryu, Jinsam Kwak

LG Electronics

# Introduction

To obtain the DL channel status of AMS in 802.16m/D12, ABS allocates the FFBCH to the AMS through Feedback Allocation A-MAP IE or allocates the resource to the AMS through Feedback Polling A-MAP IE. Upon receiving Feedback Allocation A-MAP IE the AMS sends the sequence including channel quality feedback and MIMO feedback through the allocated FFBCH. Upon receiving Feedback Polling A-MAP IE the AMS sends MIMO feedback Header (MFH) or AAI-SBS-MIMO-FBK message through the allocated resource.

As mentioned in [1], most of M2M devices (like smart metering, vending machine, etc.) are fixed or nomadic and the channel variations of those devices don’t happen frequently. That is, once the MCS or DL MIMO mode information (e.g., MEF, Mt, Si, etc.) for those device are decided, they are retained during long period. In this case, if the device includes the preferred MCS or MIMO feedback information (e.g., CQI, STC, PMI) in AAI-RNG-REQ during ranging procedure, it will reduce the additional process for requesting the channel status report by BS as mentioned above and BS can early configure the MCS or DL MIMO mode appropriate to the device to allocate the DL resource (e.g., AAI-RNG-RSP).

If BS wants to receive more elaborate channel quality feedback and MIMO feedback from the device after ranging procedure, the BS will send the Feedback Allocation A-MAP IE or Feedback Polling A-MAP IE to the device.

# Reference

[1] C802.16p-11_0051, Ranging process optimization in support of large number of machine to machine devices

# Text proposal

---- Start of proposed text ----

[Remedy #1: Add the Section 16.2.3.1 and texts in C802.16p-10/0018r1 as follows.]

16.2.3.1 AAI-RNG-REQ

[Modify the Table 678 the following Section and text:]

Table 678—AAI-RNG-REQ Message Field Description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| Ranging purpose indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle Mode …… 0b1101 = NS/EP call setup 0b1110 - 0b1111 = reserved |  |
| … | … |  | … |
| } else if (Ranging Purpose Indication == 0b0010) { |  |  |  |
| if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 |  |  |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 18 |  |  |
| } |  |  |  |
| MFM bitmap | 2 | Maximum of 2 distinct concurrent MFM are allowed with MFM_bitmap. LSB #0: MFM 0 LSB #1: MFM 4 | Will be present if the M2M device is fixed or nomadic |
| If (LSB#0 in MFM_bitmap == 1){ |  |  |  |
| Wideband CQI | 4 |  |  |
| Wideband STC rate | 3 | .‘STC rate - 1.’ mapped to 3-bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate=8 as 0b111) |  |
| } |  |  |  |
| If (LSB#1 in MFM_bitmap == 1){ |  |  |  |
| Wideband CQI | 4 |  |  |
| Wideband STC | 3 | .‘STC rate - 1.’ mapped to 3-bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate=8 as 0b111) |  |
| Wideband PMI | 6 | wideband preferred matrix index (PMI), size of which is number of PMI bits (.‘NB.’) used, mapped to NB LSB bits of this field, while the remaining MSB bit(s) set to zero(0) |  |
| } |  |  |  |
| Paging Controller ID |  |  |  |
| … |  |  |  |
| } else if (Ranging Purpose Indication == 0b0011 ¦0b0110¦0b0111¦ 0b1011) { |  |  |  |
| ... | ... |  | ... |

[Remedy #2: Add the Section 16.3.5.5.2.4.7 and texts in C802.16p-10/0018r1 as follows.]

16.3.5.5.2.4.7 CDMA Allocation A-MAP IE

[Modify the Table 853 the following Section and text:]

Table 853.—CDMA Allocation A-MAP IE*

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE { |  |  |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| CDMA allocation indication | 1 | 0b0: Bandwidth allocation in response to a received contention-based bandwidth request. 0b1: Bandwidth allocation in response to a received contention-based ranging request |
| If (CDMA allocation indication == 0b0){ |  |  |
| … | … | … |
| } |  |  |
| Else if (CDMA allocation indication == 0b1) { |  |  |
| Uplink/Downlink Indicator | 1 | Indicates whether the following fields are for resource assignment in the uplink or in the downlink. 0b0: Uplink 0b1: Downlink |
| Resource Index | 11 |  |
| ISizeOffset | 5 |  |
| HFA | 3 |  |
| If (Uplink/Downlink Indicator==0b0) { |  |  |
| … | … | … |
| } Else { |  |  |
| ACID | 4 |  |
| AI_SN | 1 |  |
| SPID | 2 |  |
| Reserved | 8 |  |
| MEF inclusion | 1 | Indicates if the specific MIMO encoder format is included. If this bit is set to 0, the SFBC is used. |
| If (MEF inclusion == 1) { |  |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: reserved |
| if (MEF == 0b01){ |  |  |
| Mt | 3 |  |
| Reserved | 2 |  |
| } else if(MEF == 0b10) {; |  |  |
| if (Nt == 4) |  |  |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010: 3 streams with PSI=stream2 and other modulation =QPSK 0b10011: 3 streams with PSI=stream2 and other modulation =16QAM 0b10100: 3 streams with PSI=stream2 and other modulation =64QAM 0b10101: 3 streams with PSI=stream0 and stream1 and other modulation =QPSK 0b10110: 3 streams with PSI=stream0 and stream1 and other modulation =16QAM 0b10111: 3 streams with PSI=stream0 and stream1 and other modulation =64QAM 0b11000: 4 streams with PSI=stream0 and stream1 and other modulation =QPSK 0b11001: 4 streams with PSI=stream0 and stream1 and other modulation =16QAM 0b11010: 4 streams with PSI=stream0 and stream1 and other modulation =64QAM 0b11011: 4 streams with PSI=stream2 and stream3 and other modulation =QPSK 0b11100: 4 streams with PSI=stream2 and stream3 and other modulation =16QAM 0b11101: 4 streams with PSI=stream2 and stream3 and other modulation =64QAM 0b11110: reserved 0b11111: reserved |
| } else{ |  |  |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index (PSI) in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010: 8 streams with PSI=stream0 0b10011: 8 streams with PSI=stream1 0b10100: 8 streams with PSI=stream2 0b10101: 8 streams with PSI=stream3 0b10110: 8 streams with PSI=stream4 0b10111: 8 streams with PSI=stream5 0b11000: 8 streams with PSI=stream6 0b11001: 8 streams with PSI=stream7 0b11010: 8 streams with PSI=stream0 and stream1 0b11011: 8 streams with PSI=stream2 and stream3 0b11100: 8 streams with PSI=stream4 and stream5 0b11101: 8 streams with PSI=stream6 and stream7 0b11110: reserved 0b11111: reserved |
| } |  |  |
| } else { |  |  |
| Reserved | 7 |  |
| } |  |  |
| } |  |  |
| } | … | … |
| } |  |  |

[Insert the following texts at the end of Section 16.3.5.5.2.4.7:]

For M2M devices the DL HARQ burst signaled by the CDMA Allocation A-MAP IE is transmitted using MIMO encoder format and the modulation scheme indicated the CDMA Allocation A-MAP IE.

---- End of proposed text ----