| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reliable multicast data transmission |  |
| Date Submitted | 2011-05-16 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Jin Lee, Giwon Park, Kiseon Ryu, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for comments on the 16p AWD |  |
| Abstract | This contribution proposes an method of the reliable multicast transmission using MAC layer feedback |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reliable multicast data transmission

Jeongki Kim, Youngsoo Yuk, Jinsoo Choi, Jin Lee, Giwon Park, Kiseon Ryu, Jinsam Kwak

LG Electronics

# Introduction

During last meeting there was a consensus that the 802.16p BS should provide the method of transmitting the multicast data for M2M application reliably [1].

For this purpose, two methods were proposed at the last meeting as follows.

- Method 1: Unsolicitedly multiple times retransmission [2]

  - BS should transmit a multicast packet multiple times

- Method 2: Retransmission based on the NACK-based MAC level feedback [3].

  - BS retransmits the multicast data only when the request of retransmission is received from the device.

Table 1 Comparison between two methods

|  | Advantage | Disadvantage |
| --- | --- | --- |
| Method 1 | - No feedback mechanism & management - No resource overhead by feedback | - Residual error happens. That is, some M2M devices may miss the data. - Fixed DL resource overhead due to fixed multiple times TX |
| Method 2 | - Guarantee the reliability by feedback - Efficient resource usage (only in case there is the request of retransmission from MS) | - Need UL feedback mechanism - Need additional resources for feedback |

# Details of retransmission method by NACK based MAC level feedback

The device sends the MAC level feedback only when the device finds the error of the multicast data reception as follows.

    - Burst decoding error: When an M2M device fails to decode the multicast burst, M2M device may send the MAC level feedback to BS.

    - Missing MPDUs:

When the multicast MPDU arrives out of sequence, if the device does not receive the missing multicast MPDU within the specified timer, M2M device may send the MAC level feedback to BS.

    - Missing the MTEEH :

After receiving the multicast MPDU, if an M2M device does not receive the MTEEH within the specified timer, the device shall send the MAC level feedback to BS.

Regarding MAC level feedback, device in connected mode sends the M2M multicast retransmission request (AAI-MMR-REQ) message and device in idle mode sends the AAI-RNG-REQ message (i.e., location update procedure). The following information may be included in the request messages.

# Performance analysis

Following analysis shows the comparison result between automatic retransmission and transmission with unicast and/or multicast feedback. Table 2 shows the residual error rate after n-time automatic transmission. (1% of BLER for 5% of MSs in a cell for a single transmission) The required residual error shall be less than 10-6 for TCP packet. Without feedback, BS shall transmit a burst more than 4 times.

Table 2. Residual error rate after n transmission in a cell.

|  | Number of transmission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of MSs in a cell | 1 | 2 | 3 | 4 | 5 |
| 100 | 0.05 | 0.0005 | 5.E-06 | 5.E-08 | 5.E-10 |
| 500 | 0.25 | 0.0025 | 3.E-05 | 3.E-07 | 3.E-09 |
| 1000 | 0.5 | 0.005 | 5.E-05 | 5.E-07 | 5.E-09 |
| 5000 | 1 | 0.025 | 3.E-04 | 3.E-06 | 3.E-08 |
| 10000 | 1 | 0.05 | 5.E-04 | 5.E-06 | 5.E-08 |
| 20000 | 1 | 0.1 | 1.E-03 | 1.E-05 | 1.E-07 |
| 50000 | 1 | 0.25 | 3.E-03 | 3.E-05 | 3.E-07 |
| 100000 | 1 | 0.5 | 5.E-03 | 5.E-05 | 5.E-07 |

We evaluate the required resource for transmitting 50 bytes of burst with various transmission options. Table 3 shows the evaluation assumption, and figure 1 shows the required resource according to the number of MSs in a cell. All the case, the residual error rates are the same when BS transmit the burst 4 times.

Table 3. Evaluation environment

| Multicast Data Size | 50 bytes |
| --- | --- |
| Feedback overhead | 60 bytes |
| DL MCS | 1/16, QPSK |
| UL MCS | 1/12, QPSK (HARQ) |
| DL resource | 34 LRUs |
| UL resource | 34 LRUs |
| RA_code | 4 LRUs |
| AAI_RNG-REQ | 30 bytes |
| AAI_RNG-RSP | 30 bytes |

Figure 1 Required resources (LRUs) for transmitting 50 bytes of multicast data with various initial transmission times. (Residual Error Rates are the same for all cases.)

From the simulation results, we can learn that without Feedback, too large resources are required. According to the number of MSs in a cell, in order to reduce the initial error rate we can consider combining of single automatic re-transmission and feedback.

# Conclusion

As we can see the result of performance analysis, in case that the number of MSs in a cell is less than 1000, one time transmission (No automatic repetition) with MAC level feedback mechanism is more efficient others. In case that the umber of MSs in a cell is larger than 1000 and less than about 70000, two times transmission (one time repetition) with MAC level feedback is more efficient than others.

Proposal

- BS may re-transmit the multicast data one time at the next frame.

- M2M device may request the retransmission of multicast data to BS by using NACK-based MAC level feedback.

# Reference

[1] 802.16p-10_0018r1, Enhancements to Support Machine-to-Machine Applications

[2] C802.16p-11_0003r2, The reliable delivery between 802.16p BS and M2M device

[3] C802.16p-11_0032, Reliable multicast transmission using MAC layer feedback

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Section 16.2.28.4.2 in C802.16p-10/0018r1 as follows.]

16.2.28.4.2 Reliable multicast transmission for M2M applications

An M2M BS shall provide the reliable transmission of the multicast traffic for M2M applications.

For providing the reliability of multicast data, multicast burst may be retransmitted by BS unsolicitedly one time and an M2M device may request the retransmission of multicast data to BS by using NACK-based MAC level feedback. The detailed method is as follows.

When an M2M device fails to decode the multicast burst, if the multicast burst is not new burst (Burst indicator (BI) == 1) , M2M device shall send the MAC level feedback to BS.

When multicast MPDU arrives out of sequence (e.g., when the device receives the MPDU with SN before receiving the MPDU with SN-1), if M2M device does not receive the missed MPDU within the multicast error detection (MED) timer (TMED), then the device shall send the MAC level feedback to BS. TMED starts when the multicast MPDU arrives out of sequence and TMED stops if the device receives the missed MPDUs before the TMED expires.

After receiving a multicast MPDU, if M2M device does not receives the MTEEH within the Multicast Transmission End (MTE) timer (TMTE), then the device shall send the MAC level feedback to BS.

TMTE starts when the first multicast data is received for M2M device in connected mode and starts when the paging message indicating the multicast traffic existence is received for M2M device in idle mode. TMTE may be reset whenever the multicast data is received or at the MTST in case of the MTST is included in the AAI_PAG-ADV message. TMTE stops when the device receives the MTEEH.

For MAC level feedback M2M device shall transmit M2M multicast retransmission request (AAI-MMR-REQ) message in connected mode/ AAI-RNG-REQ message (i.e., location update procedure) in idle mode which include the MGID, MAP transmission time, Sequence numbers of missed MPDUs, the SN of the latest MPDU received successfully as described in 16.2.3.xx/16.2.3.1.

Upon receiving the AAI-MMR-REQ, BS shall re-transmit the missing multicast data to the M2M device. Upon receiving the AAI-RNG-REQ including multicast MAC level feedback information, BS shall transmit the AAI-RNG-RSP message and shall re-transmit the missing multicast data to the M2M device.

[Remedy #2: Add the following sections and texts in C802.16p-10/0018r1 as follows.]

16.3 Physical Layer

16.3.5 Downlink Control Structure

16.3.5.5 DL Control Information Elements

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

[Modify the table 860 as indicated]

Table 860 Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_AMAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reserved This IE carries multicast assignment information for M2M application |
| … | … | … |
| } else if {Function Index == 0b10} { |  |  |
| … | … | … |
| } |  |  |
| Else {//Function Index == 0b11 |  |  |
| MGID | 15 |  |
| Burst Size | 6 |  |
| Resource Index | 11 |  |
| Long_TTI_Indicator | 1 |  |
| Burst indicator | 1 | Burst Indicator (BI) indicates whether the multicast burst is new or not 0b0: The burst is new multicast burst 0b1: The burst is retransmitted multicast burst |
| } |  |  |

[Insert the following texts at the end of Section 16.3.5.5.2.4.13:]

The multicast burst for M2M application may be retransmitted only once at the next frame. The SPID is set to 0 for multicast bursts for M2M application.

[Remedy #3: Add the following Section 16.2.3.xx in C802.16p-10/0018r1 as follows.]

[Insert the following Section and text:]

16.2.3.xx. AAI-MMR-REQ (multicast retransmission request) message

M2M device may send the AAI-MMR-REQ message to inform BS of the error of multicast data reception (e.g., burst decoding error, Missing MPDUs, Missing MTEEH).

Table xxx AAI-MMR-REQ (M2M multicast retransmission request) Message Field Description

| Fields | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| M2M group ID | 15 |  |  |
| Action code | 2 | 0b00: Burst decoding error 0b01: Missing MPDUs 0b10: Missing MTEEH 0b11: reserved |  |
| If (Action code == 0b00) { |  |  |  |
| MAP transmission time | 7 | Bit #0~#3: Least Significant 4 bits of the frame number in which the A-MAP IE corresponding to the erroneous burst was transmitted. Bit #4~#6: The sub-frame number in which the A-MAP IE corresponding to the erroneous burst was transmitted. |  |
| } else if (Action code == 0b01) { |  |  |  |
| FID | 4 | Flow ID for the MPDU in which the out of order happens |  |
| For (i=0; i< N_MPDUs; i++) { |  | N_MPDUs: Indicates the number of MPDUs which the device missed. 1< N_MPDUs < 15 |  |
| SN | 4 | 4 bits LSB of sequence number of the MPDU which the device missed |  |
| } |  |  |  |
| } else if (Action code == 0b10) { |  |  |  |
| FID | 4 | Flow ID for the MPDU related to the expired TMTE |  |
| SN | 10 | Sequence number of the latest MPDU which the device received successfully |  |
| } |  |  |  |

[Remedy #3: Add the Section 16.2.3.1 in C802.16p-10/0018r1 as follows.]

16.2.3.1 AAI-RNG-REQ

[Modify the Table 678 and text:]

Table 678—AAI-RNG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging purpose indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle Mode …… 0b1101 = NS/EP call setup 0b1110 - 0b1111 = reserved 0b1110 = M2M location update 0b1111= reserved |  |
| … | … | … | … |
| } else if (Ranging Purpose Indication == 0b0011 ¦0b0110¦0b0111¦ 0b1011¦0b1110) { |  |  |  |
| … | … | … | … |
| AMS Mobility Information | 2 |  |  |
| M2M LU purpose indication | 2 | 0b00 = LU for requesting the retransmission of multicast data 0b01 ~ 0b11 = reserved | Present when the Ranging Purpose Indication is set to 0b1110 |
| If (M2M LU purpose indication == 0b00) { |  |  |  |
| M2M group ID | 15 |  |  |
| Action code | 2 | 0b00: Burst decoding error 0b01: Missing MPDU 0b10: Missing MTEEH 0b11: reserved |  |
| If (Action code == 0b00) { |  |  |  |
| MAP transmission time | 7 | Bit #0~#3: Least Significant 4 bits of the frame number in which the A-MAP IE corresponding to the erroneous burst was transmitted. Bit #4~#6: The sub-frame number in which the A-MAP IE corresponding to the erroneous burst was transmitted. |  |
| } else if (Action code == 0b01) { |  |  |  |
| FID | 4 | Flow ID for the MPDU in which the out of order happens |  |
| For (i=0; i< N_MPDUs; i++) { |  | N_MPDUs: Indicates the number of MPDUs which the device missed. 1< N_MPDUs < 8 |  |
| SN | 4 | 4 bits LSB of sequence number of the MPDU which the device missed |  |
| } |  |  |  |
| } else if (Action code == 0b10) { |  |  |  |
| FID | 4 | Flow ID for the MPDU related to the expired TMTE |  |
| SN | 10 | Sequence number of the latest MPDU which the device received successfully |  |
| } |  |  |  |
| } |  |  |  |
| SMS |  |  |  |
| } else if (Ranging Purpose Indication == 0b0100) { |  |  |  |
| … | … | … | … |

---- End of proposed text ----