| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cell Bar bit for M2M devices |  |
| Date Submitted | 2011-5-8 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) Whai-En Chen National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes a cell bar bit for M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cell Bar Bit for M2M Devices

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

# Introduction

In P802.16m, a cell bar bit is used for preventing access of AMSs when the access link is overload. When the cell bar bit is set to 1, all the AMSs aren’t allowed to access the network including initial network entry, network re-entry, and handover. During the session 72 in Singapore, we had agreement that dedicated ranging access resource can be allocated for M2M devices to prevent large number of M2M devices impact on the human-based communication. However, it is not guaranteed that enough resource can be allocated for the dedicated ranging resource. If there is no sufficient resource can be allocated for the dedicated ranging resource, M2M devices perform ranging using the default ranging resource. To prevent the M2M devices affecting the ranging of AMSs, a cell bar for M2M device is proposed to stop ranging of M2M devices when there is no dedicated ranging resource for M2M devices.

The usage of cell bar bit is described as following

- If we only need to distinguish the 16m users and M2M devices, additional 1-bit M2M cell bar carried in the S-SFH SP1 IE can be used to stop all the M2M devices from accessing the network to protect the ranging of human-based communication.

- If we want to differentiate the priority between M2M devices, multiple bits M2M cell bar can be used to stop the M2M devices with different level of priority from accessing the network.

Since the priority of M2M devices/groups still needs more discussion to achieve consensus, we proposed 1-bit M2M cell bar in the S-SFH SP1 IE for M2M devices. So in the 802.16p system, there are 1-bit cell bar for AMSs and 1-bit M2M cell bar for M2M devices. The M2M devices don’t need to care the cell bar for AMS and only take the M2M cell bar bit to decide whether they can perform network entry/re-entry.

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

[2] IEEE 802.16p-10/0018r1, “P802.16p Amendment Working Document (AWD),” Mar. 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Add the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.3.5.5.1.2 S-SFH IE

M2M cell bar is used to stop the M2M devices from network access. When the M2M cell bar bit is set to 1, all the M2M devices shall not perform ranging to entry/re-entry the network.

Table 833 S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| M2M cell bar | 1 | If M2M cell bar bit = 0b1, this cell shall not allow network entry or re-entry for the M2M devices. |
| … |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------