| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dedicated Ranging Opportunity Assignment for M2M Group |  |
| Date Submitted | 2011-5-12 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) Whai-En Chen National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University | E-mail: eating@iii.org.tw |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes a cell bar bit for M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dedicated Ranging Opportunity Assignment for M2M Group

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

# Introduction

During the session 72 in Singapore, we had agreement that dedicated ranging access resource including ranging opportunity and ranging code can be allocated for M2M devices to prevent large number of M2M devices impact on the human-based communication. However, there is no method to assign the ranging opportunity yet. In this contribution we propose the assignment method of the dedicated ranging opportunity for M2M Group.

Since the dedicated ranging resource is allocated for certain M2M devices or M2M group to perform network entry/re-entry, it is better a type of dynamic ranging resource, not a type of regular ranging resource. To prevent the allocation of dedicated ranging resource impact the update of S-SFH and therefore results in unnecessary decode for the normal users, we suggest that the dynamic dedicated ranging resource can be announced in Broadcast Assignment A-MAP IE.

Through Broadcast Assignment A-MAP IE the 16p BS can allocate dedicated ranging resource for the M2M devices within a M2M group when the M2M devices need to perform network re-entry and report at a period of time. When the MGID bit is set to all 1, the dedicated ranging resource can be allocated for all M2M devices which perform initial network and have not been assigned a M2M group.

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

[2] IEEE 802.16p-10/0018r1, “P802.16p Amendment Working Document (AWD),” Mar. 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Add the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Table 860 Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast Assignment A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: reservedThis IE carries dedicated ranging channel allocation information for M2M Group/devices |
| … |  |  |
| }else if {Function Index == 0b11} { |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH for M2M group 1: two NS-RCHs for M2M group |
| MGID | 15 | M2M Group Identifier 111111111111111: For all M2M devices |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 14/10 | Reserved bits. If Number of Ranging Opportunities is set to 0b0, Size in bits is 14. Otherwise, Size in bits is 10. |
| } |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------