| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Reassignment in Connected Mode |  |
| Date Submitted | 2011-5-17 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) Whai-En Chen National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University Namgi Kim Kyonggi University | E-mail: eating@iii.org.tw ngkim@kyonggi.ac.kr |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes a cell bar bit for M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Reassignment in Connected Mode

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

Namgi Kim

Kyonggi University

# Introduction

During the session 72 in Singapore, we had agreement that 15-bit MGID is assigned to the M2M devices during initial network entry and the assigned MGID shall be retained by a M2M device in idle mode. The MGID may be re-assigned, however there is no consensus on the reassignment procedure. This contribution proposes the MGID reassignment procedure though DSx procedure in connected mode.

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

[2] IEEE 802.16p-10/0018r1, “P802.16p Amendment Working Document (AWD),” Mar. 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.1.3.1 M2M Group Identifier (MGID)

An MGID is assigned by a network entity during initial network entry and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD. During connected mode, the MGID may be added and changed by DSA and DSC procedure respectively.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

Suggested Remedy 2

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

Table 734 AAI-DSA-REQ Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| MGID | 15 | MGID to be added | Shall be present if this service flow is related with M2M multicast service. |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

Suggested Remedy 3

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.47.4 AAI-DSC-REQ

Table 737 AAI-DSC-REQ Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| MGID | 15 | MGID to be changed to | Shall be present if MGID needs to be changed |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------