| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Reassignment in Idle Mode |  |
| Date Submitted | 2011-5-17 |  |
| Source(s) | Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kenchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) Whai-En Chen National Ilan University Yih Guang Jan, Yang-Han Lee Tamkang University Namgi Kim Kyonggi University | E-mail: eating@iii.org.tw ngkim@kyonggi.ac.kr |
| Re: | Call for Comments on IEEE P802.16p AWD |  |
| Abstract | This contribution proposes a cell bar bit for M2M devices |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Reassignment in Idle Mode

Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Kanchei(Ken) Loa, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

Whai-En Chen

National Ilan University

Yih Guang Jan, Yang-Han Lee

Tamkang University

Namgi Kim

Kyonggi University

# Introduction

During the session 72 in Singapore, we had agreement that 15-bit MGID is assigned to the M2M devices during initial network entry and the assigned MGID shall be retained by a M2M device in idle mode. The MGID may be re-assigned, however there is no consensus on the reassignment procedure. This contribution proposes the MGID reassignment procedure though location update procedure in idle mode.

# Reference

[1] IEEE P802.16m/D12, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Feb. 2011.

[2] IEEE 802.16p-10/0018r1, “P802.16p Amendment Working Document (AWD),” Mar. 2011.

- Text proposal for inclusion in the 802.16p AWD

Suggested Remedy 1

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.1.3.1 M2M Group Identifier (MGID)

An MGID is assigned by a network entity during initial network entry and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD. The MGID may be updated through the location update procedure or network re-entry procedure in idle mode.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

Suggested Remedy 2

[Modify the following text into the 802.16p AWD]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.2 AAI-RNG-RSP

Table 679 AAI-RNG-RSP Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x6~0xF: Reserved |  |
| … |  |  |  |
| If (Location Update Response == 0x0 ) { |  |  |  |
| … | … | … | … |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| MGID update | 30 | Old MGID, New MGID The MGID update specifies a new MGID that replaces an old MGID. | Shall be included when the MGID is changed. |
| FID update | 8 | Old FID, New FID The FID update specifies a new FID that replaces an old FID. | Shall be included when the MGID is changed. |
| } |  |  |  |
| ... | ... | ... | ... |
| }//end of If (Location Update Response == 0x0 ) |  |  |  |
| … | … | … | … |
| If(it is under network re-entry for HO){ |  |  |  |
| … | … | … | … |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| MGID update | 30 | Old MGID, New MGID The MGID update specifies a new MGID that replaces an old MGID. | Shall be included when the MGID is changed. |
| FID update | 8 | Old FID, New FID The FID update specifies a new FID that replaces an old FID. | Shall be included when the MGID is changed. |
| } |  |  |  |
| ... | ... | ... | ... |
| }//end of If (it is under network re-entry for HO) |  |  |  |
| ... | ... | ... | ... |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------