| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Negotiation of HO Support |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes to define a new parameter used for negotiation of HO support during network entry. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Negotiation of HO Support

Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In some M2M applications such as Smart Grid, M2M devices will be fixed devices or nomadic devices and HO function doesn't need to be implemented in such M2M devices. However, all M2M devices which are compliant with 16m standard shall support HO regardless of their mobility because HO is defined as a mandatory function in 16m. It will increase device cost and implementation complexity.

In this contribution, we propose to add ‘HO support’ parameter into AAI-REG-REQ/RSP messages in order for M2M devices and ABS to negotiate HO function during initial network entry. This will make it possible for device venders to produce cheap M2M devices in which HO function is not implemented.

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Add the following new subclause on page 11, line 52 as follows;]

16.2.3.8 AAI-REG-REQ

Modify Table 691 as indicated

Table 691 – AAI-REG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| HO Support | 1 | 0: HO is not supported 1: HO is supported | Shall be present when AAI-REG-REQ is transmitted by an M2M device. |

16.2.3.9 AAI-REG-RSP

Modify Table 692 as indicated

Table 692 – AAI-REG-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| HO Support | 1 | 0: HO is not supported 1: HO is supported | Shall be present when AAI-REG-RSP is transmitted in response to AAI-REG-REQ which includes ‘HO Support’ parameter. |

[Remedy 2: Add the following texts on page 21, line 8 as follows;]

16.2.6 MAC HO procedures

Insert the following texts at the end of subclause 16.2.6

Implementation of HO is optional for M2M device.

----------------- End of the text proposal ---------------------------------------------------------------------------------------