| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Efficient UL Data Transmission after Network Reentry from Idle Mode |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes to omit contention-based BR procedure for UL data transmission after network reentry from idle mode in 16e system. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Efficient UL Data Transmission after Network Reentry from Idle Mode

Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim

ETRI

Shantidev Mohanty

Intel

# Introduction

Generally, an M2M device in idle mode shall perform contention based bandwidth request after network reentry from idle mode in order to transmit UL data. But, if a BS knows the M2M device will perform contention based BR right after network reentry, this contention based BR procedure can be omitted by granting UL BW in unsolicited manner. This method was already adopted for 16m-based system in the last meeting. (C802.16p-10/0078) In this contribution, we propose to use the same method for 16e-based system as well.

Figure 1 shows the proposed method. When an M2M device performs network reentry from idle mode, it may request a BS to grant UL BW for transmission of BR header in unsolicited manner by including Bandwidth request indicator TLV in RNG-REQ message during network reentry procedure. If the BS receives the RNG-REQ message with Bandwidth Request Indicator TLV, the BS may allocate UL bandwidth without a contention-based bandwidth request from the M2M device by including Unsolicited bandwidth grant indicator TLV in the RNG-RSP message. If the Unsolicited bandwidth grant indicator TLV is included in the RNG-RSP message, the BS allocates UL bandwidth within the BR grant time duration for transmission of the BR header after transmitting the RNG-RSP message.

Figure 1 – Proposed UL data transmission procedure of an M2M device in idle mode

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify texts on page 2, line 25 as follows;]

6.3.2.3 MAC Control messages

6.3.2.3.5 RNG-REQ (ranging request) message

Add the following texts at the end of subclause 6.3.2.3.5

The following parameter may be included in the RNG-REQ message when the MS is attempting to perform Network Re-Entry from idle mode.

Bandwidth Request Indicator

Indicates BW grant is required for transmission of BR header after completion of Network Re-Entry

6.3.2.3.6 RNG-RSP (ranging response) message

Add the following texts at the end of subclause 6.3.2.3.6

The following parameter may be included in the RNG-RSP message when the BS sends the RNG-RSP message as a reply to the RNG-REQ message from an MS which is attempting to perform Network Re-Entry from idle mode.

Unsolicited bandwidth grant indicator

Indicates the BS will grant UL BW in unsolicited manner for transmission of BR header after completion of Network Re-Entry from idle mode.

[Remedy 2: Modify texts on page 3, line 52 as follows;]

6.3.23.10.1 Network reentry from idle mode for M2M devices

BS may assign ranging resources, including ranging code and ranging opportunity, dedicated for M2M devices. In this case, M2M devices perform ranging for network (re-)entry using dedicated ranging resources. If BS does not assign dedicated ranging resources, M2M devices perform ranging for network (re-)entry using the ranging resources defined in 6.3.10.3.

During network reentry, the M2M device may request UL BW grant without a contention-based bandwidth request by including Bandwidth Request Indicator TLV in an RNG-REQ message. If a BS receives the RNG-REQ message with Bandwidth Request Indicator TLV, the BS may accept M2M device’s request when network reentry is fully optimized. In this case, the BS may allocate UL bandwidth for transmission of BR header by including Unsolicited bandwidth grant indicator TLV in the RNG-RSP message to the M2M device if RNG-RSP message is the last control message before traffic transmission by the M2M device. If the Unsolicited bandwidth grant indicator TLV is included in the RNG-RSP message, the BS should allocate UL bandwidth within the BR grant time duration for transmission of the BR header after transmitting the RNG-RSP message.

The M2M device should monitor the UL-MAP IE during the BR grant time duration for possible bandwidth allocation without performing any bandwidth request. If the M2M device fails to identify allocated bandwidth within the BR grant time duration, the M2M device shall perform contention based bandwidth request.

The BR grant timer in BS is started when the BS transmits the RNG-RSP message with the unsolicited bandwidth grant indicator TLV to the M2M device.

The BR grant timer in M2M device is started when the M2M device receives the RNG-RSP message with the unsolicited bandwidth grant indicator TLV sent to it.

[Remedy 3: Add the following subclause on page 6, line 42 as follows;]

10. Parameters and constants

Add the following row at the end of Table 554

Table 554 – Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| MS, BS | BR grant timer | UL allocation waiting time for BR header during Network Reentry from idle mode | 8 frames | - | - |

[Remedy 4: Add the following subclauses on page 8, line 38 as follows;]

11.5 RNG-REQ management message encodings

Add the following new row at the end of Table 582

Table 582 – RNG-REQ message encodings

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| Bandwidth request indicator | 24 | 1 | Presence of this parameter indicates BW grant is required for transmission of BR header after completion of Network Re-Entry | - |

11.6 RNG-RSP management message encodings

Add the following new row at the end of Table 585

Table 585 – RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| Unsolicited bandwidth grant indicator | 41 | 1 | Presence of this parameter indicates the BS will grant UL BW in unsolicited manner for transmission of BR header after completion of Network Re-Entry from idle mode | - |

----------------- End of the text proposal ---------------------------------------------------------------------------------------