| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Assignment |  |
| Date Submitted | 2011-07-19 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim ETRI | Email: jscha@etri.re.kr |
| Re: | Call for Comment on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts to define the scope of MGID and assignment method clearly. |  |
| Purpose | For discussion in 802.16p TG and adoption in to the 802.16p AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Assignment

Jaesun Cha, Soojung Jung, Seokki Kim, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

It was agreed to use M2M group ID (MGID) to uniquely identify an M2M group. However, scope of the MGID and MGID assignment procedure are still ambiguous. In this contribution, we propose some texts to clarify MGID scope and assignment procedure.

Figure 1 and Figure 2 show M2M device communication scenarios included in M2M communication requirement document developed by WiMAX Forum [1]. WiMAX forum defines two different scenarios depending on the location of M2M server. In both scenarios, a WiMAX operator shall provide some services to support data transport between M2M devices and M2M server. For the operation of efficient data transport service, we think that MGID shall be mapped to a certain ID which are used for data transport and managed by WiMAX operator, which means that MGID shall be unique within ASN or CSN. In addition, if efficient multicast service across ASNs is considered, then we think MGID shall be unique within CSN.

Figure 1 – Communications with M2M server within CSN       Figure 2 – Communications with M2M server outside CSN

Figure 3 shows an overview of high level M2M system which is included in M2M service requirement document developed by ETSI [2]. Conceptually, Figure 3 is not much different from the previous figures. In this case, M2M core resides in transport network which may correspond to CSN in Figure 1 and Figure 2. Considering this system overview, we also believe that MGID shall be unique within CSN as well for efficient support of data transport between M2M devices and M2M server.

Figure 3 – M2M high level system overview

# References

- DRAFT-T31-127-R020-v01-M, “Requirements for WiMAX Machine to Machine (M2M) Communication”, WiMAX Forum

- TS 102 689 v1.1.1, Machine-to-Machine communications (M2M); M2M service requirements”, ETSI

# Proposed Texts

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify texts on page 2, line 12 as follows;]

M2M group ID (MGID) uniquely identifies an M2M group in the domain of the network entity that assigns MGID, which one or more M2M devices belong to. This ID shall be used to identify a group of devices (e.g., group paging).

An MGID is assigned to a service flow of an M2M device by a network entity afterduring initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update). The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned during normal mode and idle mode. During normalconnected mode, the MGID may be added and changed by DSA and DSC procedure respectively. Re-assignment procedure during idle mode is TBD.

[Remedy 2: Modify texts on page 10, line 24 as follows;]

16.2.1.3 Address for machine to machine application

16.2.1.3.1 M2M Group Identifier (MGID)

A 15-bit value that uniquely identifies an M2M group in the domain of the network entity that assigns MGID, which one or more M2M devices belong to. This ID is used to identify a group of M2M devices.

An MGID is assigned to a service flow of an M2M device by a network entity afterduring initial network entry through DSA procedure and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The MGID assignment procedure and release procedure are TBD. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned during connected state and idle state. During connected statemode, the MGID may be added and changed by DSA and DSC procedure respectively. Re-assignment procedure during idle state is TBD.

----------------- End of the text proposal ---------------------------------------------------------------------------------------