| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for network access entry for a large number of M2M devices |  |
| Date Submitted | 2011-07-08 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. Ming-Hung Tao, Ying-Chuan Hsiao ITRI | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes to network access entry for a large number of M2M devices |  |
| Purpose | To be discussed and adopted by 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for network access entry for a large number of M2M devices

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

Based on IEEE 802.16p SRD [1], this contribution further proposes to clarify the procedure of network reentry from idle more for M2M group. This contribution supports two trigger modes for M2M group including network trigger and M2M group trigger. When a M2M group is expected to report their data or is paged, group delegates of the group send a ranging code based on multicast group ID (MGID) from the above ranging code set to BS. After BS decoded this ranging code, BS sends AAI-RNG-ACK to respond to this ranging code, which includes three ranging status (success, abort and continue) for group.

- References

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)

[2] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report

[3] IEEE 802.16p-rg-11/0008: Proposed Text for Network Reentry for a Large Number of M2M Devices

[4] C80216p-11_0126: Evaluation Guideline for Comparison of Network Entry Solutions

- Simulation Parameter and Results

In order to evaluate the network performance based on M2M groups under different access intensities, one traffic models for M2M group are assumed. Detail information on traffic model is shown in Table 1.

| Characteristics | Traffic model for M2M group |
| --- | --- |
| Number of M2M devices or number of M2M group (number of group members) | 240(50); 240(100) |
| Arrival distribution | Poisson distribution over T |
| Distribution period (T) | 60seconds |

Table 1 Detail information on traffic model of M2M group

Other simulation parameters and conditions refers to output from Adhoc group of simulation on network reentry[4]. Based on simulation results of access success probability on the table 2, network reentry based on group delegate is 100% but normal network reentry of M2M group have some access loss. Based on comparison of Statistics of number of ranging transmissions between M2M group with group delegate and normal M2M group in figure 1 and figure 3, for network reentry based on group delegate, access retransmission times of 90% users is 7 but for normal M2M group, access retransmission times of 90% users is 14. So Performance of power saving for network reentry based on group delegate is better than that of normal M2M group network reentry. Based on comparison of access delay between M2M group with group delegate and normal M2M group in figure 2 and figure 4, performance of access delay for network reentry based on group delegate is better than that of normal M2M group network reentry. So network reentry based on group delegate can resolve the issue of network access congestion and reduce access delay and retransmission time and improve performance of network access.

Based on our simulation results, we suggest 802.16p WG to accept the following proposed text in 802.16p AWD.

| Scenario | Performance measures | Number of MTC devices per cell |  |
| --- | --- | --- | --- |
|  |  | With group delegate | Normal group |
| M2M group 240(50) | Access Success Probability | 100% | 99.9% |
| M2M group 240(100) | Access Success Probability | 100% | 97.05% |

Table 2 Comparison of access success probability

Figure 1 Comparison of Statistics of number of ranging transmissions between M2M group with group delegate and normal M2M group for scenario1 (240(50))

Figure2 Comparison of access delay between M2M group with group delegate and normal M2M group  for scenario1 (240(50))

Figure 3 Comparison of Statistics of number of ranging transmissions between M2M group with group delegate and normal M2M group for scenario2 (240(100))

Figure2 Comparison of access delay between M2M group with group delegate and normal M2M group  for scenario2 (240(100))

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 1 Page 23 in P80216p-10_0018r2 with the followings:]

16.2.18.7.3 Network reentry from idle mode for M2M group

In order to reduce network congestion produced by a large number of M2M devices, network reentry based on group from idle mode procedure for M2M devices should be used. M2M device in a M2M group is called as group member (GM). M2M device which is authorized to act as representative for one M2M group to initiate the first ranging access of the M2M group is called as group delegate (GD). BS should assign a ranging code set and ranging opportunities to M2M groups. When a M2M group is expected to report their data or is paged, group delegates of the group send a ranging code based on multicast group ID (MGID) from the above ranging code set to BS.

After BS decoded this ranging code, BS sends AAI-RNG-ACK to respond to this ranging code, which includes three ranging status (success, abort and continue) for group.

All GMs of this group listen to AAI-RNG-ACK in the ranging procedure based on group. After received AAI-RNG-ACK including that ranging code, GM acts according to ranging status.

  - If ranging status is “success”, all GMs in this group start its network reentry procedure by transmitting a ranging code. The ranging code chosen may be in legacy code sets for ranging or may be a new one and the legacy initial ranging opportunity or a new one may be used for transmitting the ranging code. When A GM receives ”success” of ranging status for ranging code for the group in AAI-RNG-ACK, the GM shall randomly select a backoff value within the backoff window for group specified by S-SFH SP3. This random backoff value within the backoff window for group indicates the number of initial ranging opportunities that the GM shall defer before transmitting a ranging code.

  - If ranging status is “abort”, all GM in this group shall start the ranging abort timer and abort the ranging process until the ranging abort timer expires. Similar as the 802.16m system, after abort timer is expired, this group should restart the ranging procedure based on group.

  - If ranging status is “continue”, the GD in this group shall adjust its parameters accordingly and continue the ranging process as done on the first entry.

16.2.18.7.3.1 Selection of M2M group delegate

Selection of M2M group delegate should support fix and mobility of group. Random selection rule by M2M devices can be used for supporting selection of M2M group delegate.

When member of M2M group need report UL data, it will receive random selection probability θ by broadcast message AAI-SCD and produce random number by itself and then comparing produced random number m with random selection probability θ,

If m≤θ, it can’t become group delegate,

if m>θ, it can become group delegate,

Group delegate will choice certain superframe based on MGID and then send dedicated ranging code to BS in this superframe. If group delegate listen “ continue or abort” information for dedicated ranging code for this group in AAI-RNG-ACK, group delegate won’t change its identification of delegate. If group delegate listen “ success ” information for dedicated ranging code for this group in AAI-RNG-ACK, group delegate will go back to group member.

If group member can’t listen any information on dedicated ranging code for this group, group member will produce random number and compare it with random selection probabilityθ (random selection probability θ maybe will be incrementally changed).

BS will broadcast random selection probability by broadcast message (AAI-SCD). And BS controls variety of random selection probability θ and during the interval between two AAI-SCD messages, random selection probability θ is fixed. MS calculates the random data and probability based on the following (xxx) equation:

Yrand=(Xrand_seed*m+n)mod j

m,n is integer and one of these two parameters has to be prime numbe; j is the max   of  Yrand  and prime number; 2^16-15=65521;

PSelection= Yrand/j

------------------------------- Text Proposal 1 End ----------------------------------------------------

------------------------------- Text Proposal 2 Start ---------------------------------------------------

[Remedy2: Modify the table 680 from line 50 on Page 11 in P80216p-10_0018r2 with the followings:]

Table 680—AAI-RNG-ACK message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| }//end of for |  |  |  |
| For (j=0; j<N_Received_Codes for M2M group; j++) { |  | The number of ranging preamble code indices(i.e. N_Received_Codes)[1..64] detected in this corresponding ranging opportunity. |  |
| Ranging preamble code index for M2M group | 5 | Ranging preamble code index for M2M group received in this ranging opportunity. |  |
| Ranging Status | 2 | Indicate whether ranging preamble code or UL burst is received within acceptable limits by BS. 0b00 = success for M2M Group 0b01 = abort for M2M Group 0b10 = continue for M2M group |  |
| If (Ranging Status == 0b01) { |  |  |  |
| Ranging Abort Timer | 16 | Timer defined by an BS to prohibit the M2M group from attempting network entry at this BS, for a specific time duration. Value: 0 (Do not try ranging again at the BS.) Value: 1-65535, In units of seconds |  |
| }else if (Ranging Status ==0b10) { |  |  |  |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |  |
| If( API Bit#0==0b1 ) { |  |  |  |
| Timing offset adjustment | 15 | Amount of time required to adjust AMS transmission (in units of 1/Fs). MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1..16384] that corresponds to 0x0000 ~ 0x3FFF,respectively The GD shall advance its transmission time if the value is negative (i.e. MSB = 0b1) and delay its transmission time if the value is positive (i.e. MSB = 0b0). |  |
| } |  |  |  |
| If( API Bit#1==0b1 ) { |  |  |  |
| Power level adjustment | 4 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the GD shall apply to its current transmission power for initial ranging. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 3 bits represent power level correction value of [1..8] that corresponds to 0b000 ~ 0b111, respectively |  |
| } |  |  |  |
| If (API Bit#2==0b1) { |  |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency.The correction is 2% of the subcarrier spacing (•ƒf) multiplied by the 9-bit number interpreted as a signed integer. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively. |  |
| } |  |  |  |
| }//end of (Ranging Status == 0b10) |  |  |  |
| }//end of for |  |  |  |
| }//end of for each |  |  |  |
| }//end of for |  |  |  |
| ... | ... | ... | ... |

Table 708—AAI-SCD message Field Description

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Probability threshold of M2M group delegate selection | 10 | Probability threshold Value of quantized in 0.001 dB steps as from 0 to 1. |  |

------------------------------- Text Proposal 2 End ---------------------------------------------------

------------------------------- Text Proposal 3 Start ---------------------------------------------------

[Remedy3: Modify the table 835 from line 14 on Page 23 in P80216p-10_0018r2 with the followings:]

Table 835—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| Reserved |  |  |
| Initial ranging backoff start for M2M Group | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff end For M2M Group | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Valuesof n range 0-15 |
| Reserved |  |  |
| } |  |  |

------------------------------- Text Proposal 3 End ---------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------