| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Ranging Resource Allocation for M2M group |  |
| Date Submitted | 2011-07-08 |  |
| Source(s) | Lei Zhou, Hai Wang, Xufeng Zheng China Samsung Telecom R&D Center Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. Ming-Hung Tao, Ying-Chuan Hsiao ITRI | l.zhou@samsung.com +86-10-59253333 |
| Re: | Call for contributions for 802.16p WG |  |
| Abstract | This contribution proposes to ranging resource allocation for M2M group |  |
| Purpose | To be discussed and adopted by 802.16p for 802.16p amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Ranging Resource Allocation for M2M group

Lei Zhou, Hai Wang, Xufeng Zheng

China Samsung Telecom R&D Center

Hyunjeong Kang, Chiwoo Lim,Kaushik Josiam, Jaeweon Cho, Rakesh Taori

Samsung Electronics Co., Ltd.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

Based on IEEE 802.16p SRD [1], this contribution further proposes to clarify the procedure of network reentry from idle more for M2M group. This contribution propose to allocate dedicate ranging resource for M2M group.

References

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD)

[2] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report

[3] IEEE 802.16p-rg-11/0008: Proposed Text for Network Reentry for a Large Number of M2M Devices

- The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

------------------------------- Text Proposal 1 Start ---------------------------------------------------

[Remedy1: Add proposed text from line# 50 Page 27 in P80216p-10_0018r2 with the followings:]

16.3.8.2.4.1.2 Ranging channel for M2M devices

When a M2M group is expected to report their data or is paged, group delegates ofthe group send a ranging code based on multicast group ID (MGID) from the above ranging code set to BS.

Calculation equation on dedicated ranging code is as follows:

rdedicated ranging code=mod(floor (MGID/M), Nranging opportunity)   (xx)

The M2M individual additional RP codes shall be used for initial network entry and association of M2M individual device. The Zadoff-Chu sequences with cyclic shifts are used for the RP codes. The pth RP code xp(k) is defined and generated in equation (280) (see subclause 16.3.8.2.4.1). Ncont is the total number of initial (0~NIN-1) and handover RP codes (NIN~NIN+NHO-1) per sector for normal contention-based approach. Ndedi is the total number of dedicated handover RP code. When dedicated handover RP code set isn’t allocated, the available additional RP codes set can be used for M2M group. NM2M group is the total number of the available additional RP codes set for M2M group (Ncont + Ndedi ~ Ncont+ Ndedi+NM2M group -1) where maximum possible NM2M add per sector is 32.

For certain M2M group, 4 cases for this:

mod (MGID,M)-mod(C,M)=0                (xxx)

(xxx)

NM2M group refers to table 917-1;αmultiplexing factor of dedicated ranging code can be carriered in S-SHF SP1. N ranging opportunity is related on configuration of ranging opportunity. C is related on superframe number with ranging opportunity. MGIDtotal is the total number of MGID.

- Configuration of ranging opportunity==0, C=4*superframe number+i; i is expressed as frame n umber (0,1,2,3); Nranging opportunity= 4

- Configuration of ranging opportunity==1, C=superframe number; Nranging opportunity= 1

- Configuration of ranging opportunity==2, C=superframe number/2; mod(superframe number,2)==0; Nranging opportunity=1/2;

- Configuration of ranging opportunity==3, C=superframe number/4; mod(superframe number,4)==0; Nranging opportunity=1/4.

------------------------------- Text Proposal 1 End ----------------------------------------------------

------------------------------- Text Proposal 2 Start ---------------------------------------------------

[Remedy3: Add the followings to the table 833 from line 14 on Page 23 in P80216p-10_0018r2 with the followings:]

Table 833.—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ... | ... | ... |
| multiplexing factor of dedicated ranging code | 3 | Indicate multiplexing ratio of dedicated ranging code 0b000:1 0b001:1/2 0b010:1/4 0b011:1/8 0b100:1/16 0b101:1/32 0b110:1/64 0b111:1/128 |
| ... | ... | ... |

------------------------------- Text Proposal 2End ---------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------