| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Priority access for network entry/re-entry of a large number of M2M devices |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Lei Zhou,Kaushik Josiam, Hai Wang, Xufeng Zheng, Hyunjeong Kang, Chiwoo Lim, Jaeweon Cho, Rakesh Taori Samsung Electronics | E-mail: l.zhou @samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contributions from IEEE802.16p WG |  |
| Abstract | This contribution proposes to the scheme on priority access from network entry/reentry of a large number of M2M devices |  |
| Purpose | To be discussed 802.16p WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Priority access for network entry/re-entry of a large number of M2M devices

Lei Zhou,Kaushik Josiam, Hai Wang, Xufeng Zheng, Hyunjeong Kang, Chiwoo Lim, Jaeweon Cho, Rakesh Taori
Samsung Electronics

# Introduction

Cell bar concept is used for M2M devices.

------------------------------------------------------ Start of Proposed Text -----------------------------------------------------

------------------------------------------------------ Start of Proposed Text1 ----------------------------------------------------

[Remedy1: Add proposed text from line 31 on Page 21 in 80216p-10_0018r2 with the followings:]

16.2.15. 7 Access class of M2M devices

The ABS may restrict ranging of M2M devices by setting the access class to 1 in the AAI-SCD message. Access class set to 1 restricts M2M devices from performing ranging for network re-entry. The M2M devices may perform network entry/reentry after the BS sets the M2M access class to 0.

Access class restriction in AAI-SCD message may be used for configuring each of the 4 frames in a superfame individually. The ABS can decide the frame(s) in a superframe that the M2M devices can use for ranging during network re-entry.

| Access class restriction | Notes |
| --- | --- |
| 0 | M2M devices may access the network |
| 1 | M2M devices may not access the network |

Table xx mapping for access class

------------------------------------------------------ End of Proposed Text 1-----------------------------------------------------

------------------------------------------------------ Start of Proposed Text 2----------------------------------------------------

[Remedy 2: Add the followings to the Table 708 from line# 47 Page 18 in80216p-10_0018r2 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Access class restriction of frame (i) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame (i+1) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame (i+2) | 1 | INTEGER (0..1) | Optional |
| Access class restriction of frame (i+3) | 1 | INTEGER (0..1) | Optional |
| ... | ... | ... | ... |

------------------------------------------------------ End of Proposed Text -----------------------------------------------------