| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Authenticated Validity for M2M devices |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Eldad Zeira Interdigital Communications LLC | E-mail: eldad.zeira@interdigital.com |
| Re: | Call for Comments for 802.16p AWD 802.16p-10/0018r2 |  |
| Abstract | The contribution discusses and proposes text to fulfill some of the security related requirements for 802.16p. See also associated slides in S802.16p-11/0134 or its revision. |  |
| Purpose | To be discussed and adopted by TGp M2M |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Authenticated Validity for M2M devices

Eldad Zeira

InterDigital

# Introduction

This contribution proposes text to fulfill some of the security requirements for M2M networks in 802.16p. Specifically, the SRD requires that (6.4):

The 802.16p system shall support integrity and authentication of M2M devices, as well as integrity and privacy of M2M application traffic which requires a secure connection.

6.4.1	The 802.16p system shall support a device validity check between the device and the network.

6.4.2	The 802.16p system shall enable a flexible security suite that can be adjusted per the security requirements of the M2M application

See also previous contributions C802.16p-11/0007r1 and C802.16p-11/0093.

# Text Proposals

<<<<<<<<   Remedy 1	>>>>>>>>>>>>>>>>>

Modify 16.2.15 as follows:

When the AMS powers on and it has no context information of any ABS, the AMS shall perform the initial

network entry procedure with the following steps:

a) Scan for DL channel and establish DL PHY synchronization with the ABS

b) Obtain DL/UL parameters (from P-SFH/S-SFH IEs etc.) and establish DL MAC synchronization

c) Perform ranging and automatic adjustment

d) Negotiate basic capability

e) Perform AMS authorization and key exchange

f) Perform registration, send device validity information, and setup default service flows.

An M2M Device that is capable of self validity check and has failed that test shall not attempt to enter the network.

In network entry, if the AMS cannot attach to the preferred cell, the AMS may choose to perform a network entry without any preference of the BS type, even though the AMS has preference of the BS type in general.

<<<<<<<<<<<<<

Add at the bottom of 16.2.15:

Each AMS contains the following information when shipped from the manufacturer:

- A 48-bit universal MAC address (per IEEE Std 802-2001) assigned during the manufacturing process.

This is used to identify the AMS to the various provisioning servers during initialization.

- Security information (e.g., X.509 certificate) used to authenticate the AMS to the security server and

authenticate the responses from the security and provisioning servers.

- Device validity testing capability information.

<<<<<<<<<<<<<

Modify 16.2.15.6 as follows:

In the AAI-REG-REQ, the AMS informs the ABS of its capability parameters to be negotiated except those

capabilities already negotiated with the ABS through AAI-SBC-REQ/RSP messages. The AMS sends its validity information. In AAI-REG-RSP, the

ABS responds with the accepted capability parameters and with validity acceptance confirmation if validity has been accepted by the network. If the AMS omits some capability parameters in the

AAI-REG-REQ, the ABS considers the AMS follows the default values for those parameters and, if

acceptable, the ABS may omit those parameters applying default value in its AAI-REG-RSP.

If validity hasn’t been accepted the M2M Device shall exit the network without any further signaling.

Add the following entries to AAI-REG-REQ (table 685)

| Field | Size (bits) | Value (description) | Condition |
| --- | --- | --- | --- |
| Validity information | TBD | Implementation specific information indicating device validity testing capabilities and results of such test | - |

Add the following entry to AAI-REG-RSP (table 686)

| Field | Size (bits) | Value (description) | Condition |
| --- | --- | --- | --- |
| Validity acceptance confirmation | 1 | (=1) Indicates that device validity has been accepted by the network | - |

<<<<<<<<   Remedy 2	>>>>>>>>>>>>>>>>>

Modify at the bottom of 6.3.9:

The procedure can be divided into the following phases:

- Scan for DL channel and establish synchronization with the BS

- Obtain Tx parameters (from UCD message)

- Perform ranging

- Negotiate basic capabilities

- Authorize SS and perform key exchange

- Perform registration and send device validity information

- Establish IP connectivity

- Establish time of day

- Transfer operational parameters

- Set up connections

Implementation of phase e) is optional. This phase shall be performed if both SS and BS support

Authorization Policy. Implementation of phases g), h), and i) at the SS is optional. These phases shall only

be performed if the SS has indicated in the REG-REQ message that it is a managed SS.

An M2M Device that is capable of self validity check and has failed that test shall not attempt to enter the network.

Each SS contains the following information when shipped from the manufacturer:

- A 48-bit universal MAC address (per IEEE Std 802) assigned during the manufacturing process.

This is used to identify the SS to the various provisioning servers during initialization.

- Security information as defined in Clause 7 (e.g., X.509 certificate) used to authenticate the SS to the

security server and authenticate the responses from the security and provisioning servers.

- Device validity testing capability information as certificates or as pointers to certificates.

Modify 6.3.9.9:

Registration is the process by which the SS is allowed entry into the network and a managed SS receives its

Secondary Management CID and thus becomes manageable. To register with a BS, the SS shall send a REGREQ message to the BS. The M2M Device includes its validity testing capability information and optionally its validation certificate. The BS shall respond with a REG-RSP message including validity acceptance confirmation if validation certificate has been accepted by the network. For an SS that has indicated being a managed SS in the REG-REQ message, the REG-RSP message shall include the Secondary Management CID.

If validity hasn’t been accepted the M2M Device shall exit the network without any further signaling. Figure 79 shows the procedure that shall be followed by the SS.

Insert in 6.3.2.3.7:

The following TLV may be added if the MS supports H-FDD:

H-FDD sleep capabilities (11.7.8.10)

An M2M Device attempting initial network entry shall also include in RNG-REQ:

Validity information (TBD bits); Implementation specific information indicating device validity testing capabilities and optionally results of such test.

Insert in 6.3.2.3.8:

The following TLV may be added if the MS supports H-FDD:

H-FDD sleep capabilities (11.7.8.10)

When responding to M2M Device initial network entry the BS shall include in RNG-RSP:

Validity acceptance confirmation, 1 bit; logical “1” Indicates that device validity has been accepted by the network.