| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Multicast Operation in idle mode |  |
| Date Submitted | 2011-07-06 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Rakesh Taori Samsung Electronics | E-mail: anilag@samsung.com, hyunjeong.kang@samsung.com |
| Re: | Call for Comments on the 802.16p Amendment Working Document , IEEE 802.16p-10/0018r2 |  |
| Abstract | This contribution proposes modification to multicast operation in idle mode |  |
| Purpose | To be discussed and adopted in IEEE 802.16p-10/0018r3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Multicast Operation in idle mode

Anil Agiwal, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Introduction

The section 16.2.28.4.3 in AWD [1] defines the M2M Multicast operation in idle mode. The following issues needs to be addressed.

- M2M device receives an AAI-PAG-ADV message with multicast traffic indication and MTST. If the M2M device does not receive multicast packet in the frame indicated by MTST, what should M2M device do?

- M2M device receives an AAI-PAG-ADV message with multicast traffic indication and without MTST. How long should M2M device wait for the multicast packet?

# Proposed Solution

- M2M device receives an AAI-PAG-ADV message with multicast traffic indication and MTST. If the M2M device does not receive multicast packet in the frame indicated by MTST, what should M2M device do?

Proposal: If the M2M device does not receive multicast packet in the frame indicated by MTST, it may enter the paging unavailable interval as specified in 16.2.18.2

- M2M device receives an AAI-PAG-ADV message with multicast traffic indication and without MTST. How long should M2M device wait for the multicast packet?

Proposal: If the M2M device does not receive MTST in the AAI-PAG-ADV message with multicast traffic indication it shall wait for the multicast packet until the start time of next paging listening interval.

# Proposed text with change marks

16.2.28.4.3 M2M Multicast operation in idle mode

An M2M BS shall provide the multicast service for M2M devices in idle mode. Before an M2M BS sends DL multicast data, the M2M BS shall transmit the paging message including the multicast traffic indication to M2M devices during the paging listening intervals of the M2M devices. When an M2M device receives the multicast traffic indication through the paging message during its paging listening interval, the M2M device shall start receiving the DL multicast data without the idle mode termination.

The multicast transmission start time may be included in the paging message in order to indicate when the DL multicast data is sent by the BS. The value of multicast transmission start time shall be less than the start time of next paging listening interval of the devices receiving the AAI-PAG-ADV message. The M2M device may power down until the frame indicated by multicast transmission start time in the AAI-PAG-ADV message. If the M2M device does not receive multicast data in the frame indicated by multicast transmission start time, it shall enter the paging unavailable interval as specified in 16.2.18.2.

If the M2M device does not receive multicast transmission start time in the paging message with multicast traffic indication, it shall wait for the multicast packet until the start time of next paging listening interval.

# References

[1] 80216p-10_0018r2: Enhancements to Support Machine-to-Machine Applications