| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications to Multicast Procedure and Uplink Report (Idle Mode) |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | [STDS-802-16] 802.16p (M2M) AWD Call for Comments |  |
| Abstract | This contribution proposes a new assignment A-MAP IE for conveying M2M multicast service. |  |
| Purpose | To be discussed and agreed in IEEE 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications to Multicast Procedure and Uplink Report (Idle Mode)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

# Introduction

In the current AWD, the description for obtaining multicast service based on Table 845 and 860 is missing. It would be better to provide some description in the AWD to link the MGID and the access procedure for multicast service.

In addition, regarding to the M2M report code in idle mode, it is unclear when the M2M device shall wait the AAI-PAG-ADV message to follow the uplink report instruction, we try to clarify it in this contribution.

# Text Proposal

Adopt the following text modifications to IEEE P802.16p AWD:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Insert the following blue text to page 23, line 43]

16.2.28.4 Multicast Operation for machine to machine (M2M) applications

The ABS shall use a combination of MGID and FID to provide the multicast service. The same MGID and FID is assigned to a group of M2M devices that participate in the same multicast service and is assigned by a network during DSA procedure.

<INS> To access the multicast service, the M2M device which is assigned an MGID shall apply the 16-bit CRC mask with masking prefix = 0b0, message type indicator = 0b010, and decimal value = 4094 to decode the assignment A-MAP IE. If the MGID is included in the Broadcast Assignment A-MAP IE, the M2M device shall obtain the multicast burst according to the instruction in the Broadcast Assignment A-MAP IE. <\INS>

[Remedy #2: Insert the following blue text and delete the following red text to page 22, line 12]

16.2.18.7.1 Paging operation

AAI-PAG-ADV with M2M report code set to 0b1 may be used to poll M2M devices for periodic uplink non-realtime data transmission for fixed devices. The interval of periodic uplink data transmission should be longer than or equal to the paging cycle. When a device receives the AAI-DREG-RSP message with the Transmission Type set to 1 and Max number of paging cycle attribute, the device may wait for the AAI-PAG-ADV with M2M report code = 1 at <DEL> Max number of paging cycle x paging cycle <\DEL> <INS> the superframe where SFN mod (Max number of paging cycle * paging cycle) = paging offset <\INS>. If the device does not receive the AAI-PAG-ADV with M2M report code = 1, it should not send the uplink.

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r2: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE 802.16p-10/0018r2: IEEE P802.16p Amendment Working Document (AWD).

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.

[4] IEEE P802.16m/D12, Draft Amendment to IEEE Standard for Local and Metropolitan area Networks.