| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Allocation of ranging channel for M2M |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for allocation of ranging channel for M2M. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Allocation of ranging channel for M2M

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

According to IEEE 802.16p-10/0018r2[1], BS may assign ranging resources dedicated for M2M devices. The ranging resource dedicated for M2M devices can be periodically allocated by UCD and dynamically allocated by UL MAP in 16e. The periodic or dynamic allocation of ranging resource dedicated for M2M devices may be used for efficient ranging operation of M2M devices as the case may be.

However there is no consensus approach to allocate the ranging resource dedicated for M2M devices in 16m. We can consider periodic allocation of the ranging resources dedicated for M2M devices by system information (SFH or SCD) and dynamic allocation of the ranging resources dedicated for M2M devices by Broadcast assignment A-MAP similar to 16e. This contribution proposes the method which allocates the ranging resource dedicated for M2M devices by Broadcast assignment A-MAP.

# References

[1] IEEE 802.16p-10/0018r2, “Amendment Working Document,” May. 2011.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify Table 860 as follows:]

Table 860—Broadcast Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carries handover ranging channel allocation information 0b10: This IE carries multicast assignment information 0b11: This IE carries multicast assignment information for M2M application or ranging channel allocation information for M2M devices |
| if {Function Index == 0b00} { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 13 | Reserved bits |
| }else{ |  |  |
| Reserved | 15 | Reserved bits |
| } else if {Function Index == 0b01} { |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 29/25 | Reserved bits. If Number of Ranging Opportunities is set to 0b0, Size in bits is 29. Otherwise, Size in bits is 25. |
| } else if {Function Index == 0b10} { |  |  |
| Multicast Group ID | 12 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 958 |
| Resource Index | 11 | 512 FFT size: 0 in first 2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 4 | Reserved bits |
| }else{//Function Index == 0b11} { |  |  |
| SubFunction Index | 1 | 0b0: Multicast assignment information for M2M application 0b1: Ranging channel allocation information for M2M devices |
| If {SubFunction Index==0b0}{ |  |  |
| MGID | 15 |  |
| Burst Size | 6 |  |
| Resource Index | 11 |  |
| Long TTI Indicator | 1 |  |
| }else{// SubFunction Index==0b1 |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3. 0b0: 0b01 0b1: 0b10 |
| Dedicated ranging indicator | 1 | 0: Ranging method defined are used for purpose of normal ranging 1: Ranging method defined are used for purpose of ranging using dedicated CDMA code and transmission opportunity assigned in the AAI-PAG-ADV message |
| Reserved | 27/22 | Reserved bits. |
| } |  |  |
| } |  |  |
| } |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------