| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for M2M devices with long paging cycle |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for M2M devices with long paging cycle. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for M2M devices with long paging cycle

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, we agreed on the very long paging cycle for idle mode of M2M devices. The paging cycles (10 minutes, 1 hour and 1day) are defined in the AAI-DREG-RSP message but not defined in the AAI-PAG-ADV message. This contribution proposes some changes to the AAI-PAG-ADV message for individual paging of M2M devices with long paging cycle.

# References

[1] IEEE 802.16p-10/0018r2, “Amendment Working Document,” May. 2011.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify Table 706 as follows:]

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (i=0;i<M;i++){ |  |  |  |
| For (j=0;j<Num_AMSs;j++){ |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218– 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08: 32768 superframes 0x09: 262144 superframes 0x10: 4194304 superframes 0x1108-0x15: reserved | Present if the S-SFH Network Configuration bit == 0b0. Values 0x08-0x10 may be applied to M2M devices only. |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message[0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing netwok re-entry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported. |
| If(Action code == 0b10){ |  |  |  |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame nuimber in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| … |  |  |  |
| } |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------