| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for M2M ranging channel allocation by UCD message |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for M2M ranging channel allocation by UCD message. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for M2M ranging channel allocation by UCD message

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, we agreed on the M2M ranging channel allocation by UCD message. There are two ranging method (i.e. Initial ranging/Handover ranging over two symbols and Initial ranging/Handover ranging over four symbols) for M2M ranging channel allocation, but the ranging regions cannot be allocated simultaneously unlike 16e. This contribution proposes some changes to the UCD message for M2M ranging channel allocation.

# References

[1] IEEE 802.16p-10/0018r2, “Amendment Working Document,” May. 2011.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify Table 568 as follows:]

Table 568—UCD channel encodings

| Name | Type(1byte) | Length | Value |
| --- | --- | --- | --- |
| M2M Ranging Region | 25 | 6/12 | The value of TLV consists of up to two concatenated sections (one section per Ranging method), each having the following structure: Bit 0: dedicated ranging indicator Bits 1-2: ranging method Bits 2-9: num subchannels Bits 10-16: num OFDMA symbols Bits 17-23: subchannel offset Bits 24-31: OFDMA symbol offset Bits 32-34, Parameter d that defines periodicity of 2d frames Bits 35-39, Allocation phase expressed in frames, 0≤ Allocation phase <periodicity(=2d) Bit 40, When this bit is set to 1, the ranging allocation defined by Ranging Region TLV Type = 212 shall not be used by M2M devices Bits 41-47, Reserved |

----------------- End of the text proposal ----------------------------------------------------------------------------------------