| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of MGID reassignment procedure in idle state for IEEE 802.16p system |  |
| Date Submitted | 2011-07-09 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon, Kwangjae Lim ETRI Giwon Park LG Electronics | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| Re: | Comments on 802.16p Amendment Working Document (AWD) Content for Session #74 |  |
| Abstract | This contribution provides a text for MGID reassignment procedure in IEEE 802.16p system. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of MGID reassignment procedure in idle state for IEEE 802.16p system

Namgi Kim

Kyonggi University

Soojung Jung, Jaesun Cha, Seokki Kim, Chulsik Yoon, Kwangjae Lim

ETRI

Giwon Park

LG Electronics

# Proposal

According to the sections 16.2.1.3 in the AWD [1], the IEEE 802.16p system supports M2M Group Identifier (MGID). However, the reassignment procedure of MGID in idle state is TBD. Therefore, we propose a MGID reassignment procedure in idle state for the current AWD.

# References

[1] IEEE 802.16p-10/0018r2, “IEEE 802.16p Machine to Machine (M2M) Amendment Working Document (AWD),” May 2011.

# Proposed Text

We differentiated any texts modified by this contribution by using a different color as follows.

- Texts added by this contribution: blue underlined texts.

- Texts deleted by this contribution: red strikeout texts.

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

### 16.2.1.3.1 M2M Group Identifier (MGID)

A 15-bit value that uniquely identifies an M2M group in the domain of the network entity that assigns MGID, which one or more M2M devices belong to.

An MGID is assigned to an M2M device by a network entity during initial network entry and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The MGID assignment procedure and release procedure are TBD. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned. During connected modestate, the MGID may be added and changed by DSA and DSC procedure respectively.

During the idle state, the MGID may be changed by location update (i.e., M2M device-initiated location update or ABS-initiated location update) or network reentry. When the ABS updates the MGID through the ABS-initiated location update, ABS can perform the group location update as well as individual location update. When the ABS changes the MGID of all M2M devices within the Multicast Group, ABS can trigger the group location update via paging message. When the M2M device performs the timer based update, if the ABS needs to update the MGID of M2M device, the AAI-RNG-RSP message with new MGID is sent by the ABS in response to the AAI-RNG-REQ message.

### 16.2.3.2 AAI-RNG-RSP

Modify Table 679 as indicated

Table 679 – AAI-RNG-RSP Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... |  | ... | ... |
| Unsolicited bandwidth grant indicator | 1 | 1: indicates an unsolicited BW grant will be available for transmission of BR header without request from AMS during network entry or indicates an unsolicited BW grant will be available for transmission of BR header without request from M2M device during network reentry from idle mode | Shall be included when AMS is attempting network entry Shall be included if bandwidth request indicator is included in AAI-RNG-REQ when M2M device is attempting network reentry from idle mode |
| For(i=0;i<Num_MGID;i++){ |  | Number of MGID and FID (Num_MGID) to update in the T-ABS[1..TBD]. Mapping of current MGID and FID and new MGID and FID to be updated. | Presented if it needs to be updated |
| Current MGID | 15 |  |  |
| Current FID | 4 |  |  |
| New MGID | 15 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| ... |  | ... | ... |

----------------- End of the text proposal ---------------------------------------------------------------------------------------