| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Efficient SMS Transmission over l6e based M2M Networks |  |
| Date Submitted | 2011-07-18 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | P802.16p AWD |  |
| Abstract | Propose the enhancement to enable efficient SMS Transmission over M2M Networks |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Efficient SMS Transmission over 16e based M2M Networks

Joey Chou,

Intel

- Introduction

IEEE 802.16p-11/0124 contribution provides the tracebility between M2M requirements in SRD and M2M Functionalities in AWD. The table below shows a requirement that has not been addressed in AWD.

| SRD (80216p-10_0004r2) Requirements |  |  | AWD - WirelessMAN-OFDMA |  | AWD -WirelessMAN-Advanced |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1 |  | efficient transmission of small burst sizes; minimize protocol and signaling overhead to support small burst transmission (e.g. support short burst transmission in OFDMA networks without requiring full connection establishment). |  |  |  |  |

Basically, requirement 6.3.1 is intended to do two things:

- Minimize the singaling protocol overhead associated with small burst transmission.

- Not require full connection establishment for short burst transmission.

SMS (Short Message Service) is becoming an important transmission mechanism for M2M communication, due to mature infrastructure and the ubiquity of cellular networks. Moreover, the small message payload of SMS makes for a perfect mechanism to support small burst transmission. However, the SMS can’t be transmitted efficiently over the 802.16e.

This contribution proposes enhancements to 16e in order to address the requirement 6.3.1 of efficient transmission of small bursts. In addition, these enhancements are able to optimize the M2M operation and radio resource allocation for time-controlled, time tolerant, and infrequent M2M traffic that are defined in the requirements listed in the table below.

| SRD (80216p-10_0004r2) Requirements |  |  | AWD - WirelessMAN-OFDMA |  | AWD -WirelessMAN-Advanced |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2 |  | optimized operation for time controlled, time tolerant and in-frequent traffic to conserve power | 6.3.2.3.60 | M2M_POLL-ADV | 16.2.3.22 | long paging cycle and M2M paging offset in AAI-DREG-RSP |
|  |  |  |  |  | 16.2.3.22 | Transmission Type and Max number of paging cycle in AAI-DREG-RSP |
|  |  |  |  |  | 16.2.3.23 16.2.18.7.1 | Group paging and report solicitation by MGID in AAI-PAG-ADV |
|  |  |  |  |  | 16.2.3.24 16.2.18.7.1 | Group paging indication in PG-Info |
| 6.1.2.1 |  | efficient radio resource management for time-controlled and time-tolerant traffic. | 6.3.2.3.60 | M2M_POLL-ADV | 16.2.3.22 | Transmission Type and Max number of paging cycle in AAI-DREG-RSP |

- Discussion

Please reas section in ….

- References

- IEEE 802.16p-11/0124, “M2M SRD/AWD Functionality Matrix”, Inbar Anson Bratspiess, Dan Gal

- Proposed Text

4.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

6.3.2.3.60 M2M_POLL-ADV (M2M broadcast paging) message

The M2M_POLL-ADV message shall be sent on the Broadcast CID or Idle Mode Multicast CID during the BS paging interval. The M2M_POLL-ADV is to be broadcast on the same downlink frame as MOB_PAG-ADV, so the devices can receive the message after waking up from the idle mode.

The format of the message is shown in Table 166a.

[Change table 166a, as follows:]

Table 166a—M2M_POLL-ADV message format

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| M2M_POLL-ADV_Message_format() { |  |  |
| Management Message Type = 70 | - | - |
| Num_M2M_Devices | 8 | Number of M2M devices |
| For (j = 0; j < Num_M2M_Devices; j++) { |  |  |
| M2M device MAC Address hash | 24 | The hash is obtained by computing a CRC24 on the MS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB. |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report |
| M2M command | 7 | Indicate the command to control the M2M device |
| Reserved | 7 |  |
| } |  |  |

----------------------------------------------  Text End -----------------------------------------------