| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DREG-CMD Action Code to support M2M_POLL-ADV |  |
| Date Submitted | 2011-07-09 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | P802.16p AWD |  |
| Abstract | Propose DREG-CMD Action Code to support M2M_POLL-ADV |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DREG-CMD Action Code to support M2M_POLL-ADV

Joeyn Chou,

Intel

- Introduction

As shown below, 802.16p indicates that a device can receive M2M_POLL-ADV message after waking up from the idle mode. When a device sends DREG-CMD to enter the idle mode, the BS does not know if it should send MOB_PAG-ADV or M2M_POLL-ADV to such device.

6.3.2.3.60 M2M_POLL-ADV (M2M broadcast paging) message

The M2M_POLL-ADV is to be broadcast on the same downlink frame as MOB_PAG-ADV, so the devices can receive the message after waking up from the idle mode.

This contribution adds an option in the action code to indicate the M2M device is to be paged by M2M_POLL-ADV message.

- Proposed Text

2.1 Proposed Text #1

---------------------------------------------- Text Start -----------------------------------------------

6.3.2.3.26 DREG-CMD (de/reregister command) message

Table 96—Action codes and actions

| Action code (hexadecimal) | Action |
| --- | --- |
| 00 | SS shall immediately terminate service with the BS and should attempt network entry at another BS. |
| 01 | SS shall listen to the current BS but shall not transmit until a RES-CMD message or DREG-CMD message with action code 02 or 03 is received. |
| 02 | SS shall listen to the current BS but only transmit on the basic and primary management connections. |
| 03 | SS shall return to normal operation and may transmit on any of its active connections. |
| 04 | This option is valid only in response to a DREG-REQ message with De-Registration Request Code = 0x00. The SS shall terminate current Normal Operations with the BS. |
| 05 | MS shall begin MS idle mode initiation. See 6.3.23.1. |
| 06 | This option is valid only in response to a DREG-REQ message with De-Registration Request Code = 0x01. The behavior of MS to this action code is described in 6.3.23.1. |
| 07 | This option indicates that the M2M device is to be waken up by the M2M_POLL-ADV message |
| 078 – 0xFF Reserved |  |

----------------------------------------------  Text End -----------------------------------------------