| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M group identifier functionalities and design requirements |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | In this contribution we will analyze some basic functionalities and design requirements of MGID |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group ID Functionalities and Design Requirements

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1], M2M group identifier (MGID) for IEEE 802.16p system is defined as an identifier of one or more M2M devices belong to. However, the detail using scenarios of MGID have not yet been investigated in depth. Therefore, in this contribution, we will analyze some basic functionalities of MGID including:

- Group control signaling for M2M groups

- DL multicast service for M2M groups

- Group paging for M2M groups

And regarding to these potential usage requirements of MGID we can propose some corresponding MGID assignment and management procedures in [2][3][4][5].

# Basic MGID functionalities analysis

In this section, we will give some detail descriptions of MGID usages and the practical requirements, e.g. MGID length and life cycle.

## MGID for Control signaling

Since the M2M devices in a same group have similar properties, e.g. MCS level and power control, these control messages could be common for them. That means multiple M2M devices in a group could get the control signaling by one control message potentially. In order to identify this control message for the specific M2M group,  MGID should be masked or embedded in it as well as the individual control signaling, e.g. DL Assignment AMAP IE [7], is masked by STID in 16m.

And regarding to the compatibility with the legacy 16m and 16e devices, it is better that these control signaling could be based on the existing control signaling, e.g. DL/UL AssignmentA- MAP IE in 16m, instead of other new MAP IE.

Figure 1. MGID used for the group based control signaling

## MGID for Multicast

In the figure 2, MGID using for DL multicast is shown. Multicast service provides an efficient way to transmit some common DL data to a group of M2M devices also. MGID in this using scenario should be served as the multicast group ID in 16m and 16e. From the figure below, MGID for multicast is not only valid in Connect mode but also in Idle mode.

Figure 2. MGID for DL Multicast

## MGID for Group paging

In the figure 3, the MGID for the group based paging in M2M [1] is shown also. MGID may be included in a paging message instead of an individual identifier which is defined as Deregistration Identifier in 16m or MAC address in 16e. From the figure 3, MGID for group paging should be valid in the idle mode also.

Figure 3. MGID for Group paging

As explained above, we can conclude variable MGID requirements in the typical usage scenarios above:

|  | Control signaling |  | Multicast |  | Group paging |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 16m | 16e | 16m | 16e | 16m | 16e |
| Involved PHY/MAC Messages | Mask of DL/UL Assignment MAPIE | A content field in DL/UL MAP IE | A content field in MBS-DATA-IE | A content field in MBS-MAP-IE | A content field in AAI-PAG-ADV | A content field in MOB-PAG-ADV |
| Life cycle | Connect |  | Connect, Idle |  | Idle |  |
| Length | 12bits | 16bits | 12bits, consist with MBS-ID | 16bits, consist with MBS-ID | 18bits, 24bits | 24bits |

Meanwhile, in order to minimize the current standard impaction it is better to use the current control messages to support M2M service instead of the new ones. For instance, the current DL/UL Assignment A-MAP IE could be used for the resource allocation for the multiple devices in a M2M group also with MGID masking.

Therefore, regarding to the system compatibility with 16e/16m, we propose to define M2M group identifier to satisfy with the following requirements:

- Valid in both Connect and Idle mode

- Length is 12bits or 16bits in 16m or 16e respectively which is consist with minimum{CID, Multicast group ID, paging ID}

# Conclusion

Based on the discussion in section Error: Reference source not found basically MGID should support the following functionalities:

- Group based control signaling for M2M group

- DL multicast control/data for M2M group

- Group paging for M2M group

And MGID should be satisfied with the following requirements so far.

- Valid in both Connect and Idle mode

- Length is 12bits or 16bits in 16m or 16e respectively which is consist with minimum{CID, Multicast group ID, paging ID}

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Add the following sections in IEEE802.16p AWD

16.2.1.3.1 M2M Group Identifier (MGID)

A 12-bit value that uniquely identifies an M2M group which one or more M2M devices belong to. This ID shall be used to the following functionalities:

- Group based control signaling for M2M group

- DL multicast control/data for M2M group

- Group paging for M2M group

.

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-10/0018r2

- IEEE C80216p-11/0153

- IEEE C80216p-11/0154

- IEEE C80216p-11/0155

- IEEE C80216p-11/0156

- IEEE 802.16e2009

- IEEE 802.16mD12

- IEEE C80216p-11/0104