| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Group ID Assignment Scheme in IEEE802.16e System |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group ID(MGID) assignment scheme based on the 802.16e in order to minimize the modification to the standard. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group ID Assignment Scheme in IEEE802.16e System

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

In [1], M2M group identifier (MGID) for IEEE 802.16p system is defined in [1] and the further detail discussion for its usages and design requirements are present in [2] also. But how to assign MGID in the practical systems,  e.g. 802.16m/16e,  has not been addressed yet.

Therefore in this contribution, we propose a MGID assignment scheme based on reusing existing dynamic sever ice flow establishment mechanism for MBS, which will minimize the impaction to the current 802.16e [3] system. The other similar scheme for 802.16m is presented in [4] also.

# MGID Assignment in 16e

Firstly, we can briefly introduce some MGID design issues which are addressed in [2].

- MGID should be valid in both Connect and Idle mode.

- MGID length is same as the other IDs which will be included in the corresponding control signaling, e.g. DL/UL MAP IE.

At same time, in 802.16e Multicast group ID could be satisfied with the two requirements mentioned above. That means MGID could reuse Multicast group ID establishment and management mechanism.

In Figure 1, the propose MGID establishment procedure in 802.16e is shown. In 802.16e, the service flow connection could be initiated by either MS or BS. Therefore, MGID establishment process should consider both BS initiated and M2M device initiated situations depending on the connection initiation entity.

## M2M Devices initiate the connection

- Step1. After M2M device’s entry into the network, it will notify its traffic characteristics, e.g. the traffic interval, traffic packet size, to the BS by “SDU-Size” and “SDU Inter-arrival Interval ” in DSA-REQ message as the grouping criteria.

- Step2. BS will aggregate the M2M devices with similar traffic pattern together as a M2M group. Then BS will assign a same CID to these M2M devices in the same M2M group in DSA-RSP message. M2M Group Indicator in DSA-RSP message should be enabled also. Actually the conveyed CID in this message served as “MGID” in 16e.

- M2M devices initiated process

- BS initiated process

Figure 1. MGID assignment and typical usages

## BS initiate the connection

- Step1. BS and upper layer will predefine some M2M groups.

- Step2. BS will assign a same CID to these M2M devices in the same M2M group in DSA-REQ . M2M Group Indicator in DSA- REQ message should be enabled also. Actually the conveyed CID in this message served as “MGID” in 16e.

- Step3. M2M devices will feedback DSA- RSP to BS as the confirmation.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Add the following sections in IEEE802.16p AWD

11.13 Service flow management encodings

…

Table 606—Service flow encodings

| Type | Parameter |
| --- | --- |
| 1 | SFID |
| 2 | CID |
| 3 | Service Class Name |
| 4 | MBS |
| 5 | QoS Parameter Set Type |
| 6 | Traffic Priority |
| 7 | Maximum Sustained Traffic Rate |
| 8 | Maximum Traffic Burst |
| 9 | Minimum Reserved Traffic Rate |
| 10 | M2M group ID indicator |
| 11 | Uplink Grant Scheduling Type |
| 12 | Request/Transmission Policy |
| … |  |
| 32 | Paging Preference |
| 33 | MBS zone identifier assignment |
| 34 | Reserved |
| 35 | Global Service Class Name |
| 36 | Reserved |
| 37 | SN Feedback Enabled |

- 11.13.x M2M group ID indicator

The value of this field specifies the CID(11.13.2)  assigned by the BS to a service flow is used for a M2M group connection . The 1-bit value of this field is used in BRs and in MAC PDU headers.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].10 | 1 | Bit 0: non M2M group 1: M2M group CID Bit2-7: Reserved | DSx-REQ DSx-RSP DSx-ACK |

- 11.13.25 SDU Inter-Arrival Interval parameter

This parameter specifies nominal interval between consequent SDU arrivals as measured at MAC SAP. If this parameter used to specify the M2M service SDU interval, its basic unit should be “1s”.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].30 | 2 | SDU inter-arrival interval in the resolution of 0.5 ms If M2M group CID indicator is true, its basic resolution unit should be 1s | DSA-REQ,DSA-RSP DSC-REQ,DSC-RSP REG-REQ, REG-RSP |

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-10/0018r2

- IEEE C80216p-11/0152

- IEEE 802.16e2009

- IEEE C80216p-11/0154

- IEEE 802.16mD12

- IEEE C80216p-11/0104