| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M Group ID Assignment Scheme in IEEE802.16m System |  |
| Date Submitted | 2011-07-08 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | RE: Call for comments on the 802.16p AWD |  |
| Abstract | This contribution proposes the M2M group ID(MGID) assignment scheme based on the 802.16m in order to minimize the modification to the standard. |  |
| Purpose | For review and adoption into 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M Group ID Assignment Scheme in IEEE802.16m System

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

M2M group identifier (MGID) for IEEE 802.16p system is defined in [1] and the further detail discussion for its usages and design requirements are also present in [2]. But how to assign MGID in the practical systems, e.g. 802.16m, has not been addressed yet.

Therefore in this contribution, we propose a MGID assignment scheme based on reusing existing dynamic service flow (DSA) establishment mechanism for MBS, which will minimize the impaction to the current 802.16m [3] system. The other similar scheme for 802.16e is proposed in [4] also.

# MGID Assignment in 16m

Firstly, we can briefly introduce some MGID design issues which are discussed in [2].

- MGID should be valid both in Connect and Idle mode.

- MGID length is same as the other IDs which will be included in the corresponding control signaling, e.g. DL/UL assignment AMAP IE.

At same time, in 802.16m E-MBS ID could be satisfied with the two requirements above. That means MGID could reuse MBS establishment and management mechanism.

In Figure 1, the propose MGID establishment procedure in 802.16m is shown. In 802.16m, the service flow connection could be initiated by either AMS or ABS. Therefore, MGID establishment process should consider both BS initiated and M2M device initiated situations depending on the connection initiation entity.

## M2M Devices Initiated

- Step0. When M2M device registers to the network, it will report to BS its “M2M capability” which will indicate this device could be grouped as M2M group. Actually this additional parameter could be applied to not only the MGID establishment but also other optimized processes for 16p, e.g. dedicated ranging resource allocation for 16p.

- Step1. After M2M device’s entry into the network, it will notify its traffic characteristics, e.g. the traffic interval, traffic packet size, to the BS by “SDU-Size” and “SDU Inter-arrival Interval ” in AAI-DSA-REQ message as the grouping criteria.

The M2M device which intends to join a M2M group will inform ABS by “E-MBS service” message. For instance, when E-MBS service is “111” in DSA-REQ from M2M device to ABS, it denotes that M2M device would like join one of M2M groups which are managed by ABS and M2M server.

- Step2. BS will aggregate the M2M devices with similar traffic pattern together as a M2M group.  Once it receives the M2M group join request in Step1, BS will find the fitful M2M group for them whether it existed or not.

Subsequently BS will assign a same E-MBS ID to these M2M devices in the same M2M group in AAI-DSA-RSP message. EMBS Service field in AAI-DSA-RSP will be set to “111” to indicate this E-MBS ID is used as M2M group ID purpose indeed.

It is obviously that E-MBS ID embedded in AAI-DSA-RSP message served as M2M group ID here. For our understanding easily, we can denote this E-MBS ID as MGID indeed in Figure 1.

And M2M ID needs to be assigned to M2M device in AAI-DSA-RSP to support the resource allocation for the individual M2M devices in a group. And the M2M group control signaling mechanism including the resource allocation for the individual devices will be discussed in [].

- Step3. M2M devices will feedback AAI-DSA- ACK to ABS as the confirmation.

- M2M devices initiated process

- BS initiated process

Figure 1. MGID assignment in Connect Mode

## BS Initiated

- Step0. When M2M device registers to the network, it will report to BS its “M2M capability” which will indicate this device could be grouped as M2M group. Actually this additional parameter could be applied to not only the MGID establishment but also other optimized processes for 16p.

- Step1. BS and upper layer will predefine some M2M groups.

- Step2. BS will assign a same E-MBS ID to these M2M devices in the same M2M group in AAI-DSA-REQ . EMBS Service in AAI-DSA-REQ message will be set to “111” to indicate this E-MBS ID is for M2M group indeed. At same time, the additional M2M ID will be assigned in AAI-DSA-REQ.

- Step3. M2M devices will feedback AAI-DSA- ACK to ABS as the confirmation.

# Conclusion

In this contribution, we proposed a MGID assignment mechanism which is based on the existing DSA MAC control messages. Generally speaking this method could achieve the following benefits.

- Flexible MGID usage application, e.g. Multicast service, M2M group control signaling and paging.

- Good legacy system compatibility with 16m/16e. For example, most of signaling related needs only to be modified with some identifier type.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Revised the following text in Section16.2.1.3.1  in the latest 16p AWD

16.2.1.3.1 M2M Group Identifier (MGID)

A 12-bit value that uniquely identifies an M2M group which one or more M2M devices belong to. This ID shall be used to send control messages to a group of devices (e.g., group paging) or to transmit the multicast data to the M2M devices.

An MGID is assigned by  the service flow establishment process as well as the multicast group does and released during an explicit network exit (e.g., power down location update) or when the device enters DCR mode. The assigned MGID shall be retained by an M2M device even in idle state unless the M2M device exits from the network. The MGID can be re-assigned; the re-assignment procedure is TBD.

16.2.1.3.2 M2M Identifier (M2M ID)

A TBD-bit value that uniquely identifies a M2M device in a M2M group. This ID shall be used to send some UE-specific control messages to a M2M devices belong to a M2M group (e.g., resource allocation based on M2M group scheduling).

Proposal #2 : Add the following text in Section16.2. in the latest 16p AWD.

16.2.3.11 AAI-REG-REQ

Table 685 —AAI-RNG-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| M2M capability | 2 | 0b00: not M2M device 0b01: M2M device w/o grouping Ob10: M2M devices with grouping Ob11: reserved |  |

16.2.12.12 Service Flow/Convergence Sublayer Parameters

Table 782.—Service flow/convergence sublayer parameters

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |

Table 782a E-MBS Service for M2M Configuration

|  | Bit#0 | Bit#1 | Bit#2 | Notes |
| --- | --- | --- | --- | --- |
| Set0 | 0 | 0 | 0 | Not support EMBS and M2M grouping |
| Set1 | 1 | 0 | 0 | Support Single ABS E-MBS only |
| Set2 | 0 | 1 | 0 | Support multiple ABS E-MBS w/o macro diversity |
| Set3 | 0 | 0 | 1 | Support multiple ABS E-MBS with macro diversity |
| Set4 | 0 | 1 | 1 | Support M2M grouping: M2M device request join M2M group. E-MBS ID could be initialized “0000”. |
| Set4 | 1 | 1 | 1 | Support M2M grouping:. E-MBS ID in the legacy spec used as M2M group ID |

16.2.3.47.1 AAI-DSA-REQ

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| …. |  |  |  |
| M2M ID | TBD | Indicates the individual M2M devices ID in a group |  |

16.2.3.47.2 AAI-DSA-RSP

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If(M2M capability = 1){ |  |  |  |
| E-MBS Service | 3 | Indicates whether the E-MBS service and M2M group is being requested or provided for the connection that is being setup. The configuration of these field is given in the table782a. |  |
| } else |  |  |  |
| { E-MBS Service } | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| …. |  |  |  |
| M2M ID | TBD | Indicates the individual M2M devices ID in a group |  |

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0018r2

- IEEE C802.16p-11/0152

- IEEE 802.16mD12

- IEEE C802.16p-11/0153

- IEEE 802.16e2009

- IEEE C80216p-11/0103